3GPP TSG-RAN WG4 Meeting #117	R4-2520471

Dallas, USA, 17th-21st November, 2025

Source:	Nokia

Title:	Discussion on remaining UEIBM requirements

Agenda item:	4.18.2

Document for:	Discussion

Introduction

In RAN4#116bis meeting the agreements were reached and all open items pertaining to core part requirements were closed. In this document, we present our views regarding some agreements that in our view RAN4 needs to discuss in the RAN4#117 meeting.

Discussion on reporting delay requirements with timer

Otherwise, if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- eventInstanceCount-r19 is configured by the network

- TL1-meas_basic is the measurment period above based on the configuration,

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19).

RAN4 has defined requirements for the case where the eventInstanceCount-r19 > 1

First, we would like to point out that starting and ending point is not defined for the timer-based reporting. The definition of starting and ending point in the specification is only defined for the no-timer based reporting:

When eventDetectionTimeWindowLength-r19 is not configured, The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel

Current specification does not define starting and ending point for timer-based reporting.

The starting and ending point clearly needs to be defined for the timer-based reporting case as well. Furthermore, as we pointed out in the last meeting, the starting point for the non-timer based reporting cannot directly be reused for timer-based reporting, because of the scenario where the UE restarts the timer and report is triggered by the restarted timer.

If the timer expires and the UE starts another timer based on event condition becoming met again after timer expiration, it is clear that the situation is the same as if the first timer would trigger the report.

However, if the event condition becomes met as in Figure 1 already during the previous timer which does not trigger a report, the agreed reporting delay does not cover this case if the starting point is defined to be the moment when the event condition becomes met over the air. In the example of Figure 1, eventInstanceCount-r19 = 4 and timer duration is 5 SSB periods. Only 3 event instances meet reporting condition during the first timer, so the timer expires without a report. UE restarts the timer because the condition is still met on the first event occasion after timer expiration. However, the condition became met already during the previous timer two SSB periods ago and hence the delay (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel does not cover the whole delay, which is now (2 + eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel.

Figure 1: Restarting the timer when eventInstanceCount-r19 = 4 and timer length is 5 measurement periods.

If report is triggered by a restarted timer, event condition may have become met already during the previous timer, in which case the agreed timer-based reporting delay may not be sufficient to cover the time from event condition becoming met until the first PUCCH.

Because of Observation 2, either the starting point or the delay needs to be adjusted. In the last meeting we proposed ways to capture this, but the way proposed was not agreeable to companies. Therefore, we would like to propose the following:

When eventDetectionTimeWindowLength-r19 is configured, the event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface.

  - If a timer is already running at the time of the event that will trigger a measurement report, the time until start of the timer during which the report is triggered is added in the event triggered measurement reporting delay

We have proposed how to capture this in our CR [2].

Discussion on timeRestrictionForChannelMeasurements

RAN4 sent LS R4-2511656 to RAN1 asking feedback if there was any issue with RAN4 agreement about timeRestrictionForChannelMeasurement configuration, with the following content:

RAN4 is discussing measurement requirements for event-triggered L1-RSRP reporting. It is RAN4 common understanding that NW can configure “timeRestrictionForChannelMeasurement” for event-triggered L1-RSRP reporting. For Event-2, RAN4 agreed to define separate requirements for measurement period when the UE is configured or not configured with“timeRestrictionForChannelMeasurement, similar to existing measurement period requirements. Specifically, for SSB-based and periodic and semi-persistent CSI-RS based L1-RSRP measurement, if timeRestrictionForChannelMeasurement is configured, the measurement period is based on M = 1, and event evaluation should be based on the most recent result from both new beam and current beam; If timeRestrictionForChannelMeasurement is not configured, the measurement period for current and new beam is based on M = 3. For aperiodic CSI-RS based L1-RSRP, M is always equal to 1.

No answer has been received yet from RAN1, meaning in our view that up to now RAN1 has not identified any issue with the agreement and the information RAN4 provided in the LS. More specifically, and as highlighted in green above, RAN4 stated that:

- if timeRestrictionForChannelMeasurements is not configured, then M=3 for both current and new beam(s),

- if timeRestrictionForChannelMeasurements is configured, then M=1 for both current and new beam(s).

Note that we support both RAN4#115 agreement and LS R4-2511656. On the other hand, we think they have not been properly captured in the most recent version of TS 38.133 (version 19.2.0), and that is why we propose our companion CR [2]. In the following, we try to explain better why our companion CR is needed.

High layer parameter timeRestrictionForChannelMeasurements is part of CSI-ReportConfig. As per legacy (Rel-15) text highlighted just below in green from clause 5.2.1.4.3 in TS 38.214, timeRestrictionForChannelMeasurements applies to the RS/beam measurements, i.e., either SSB or CSI-RS measurements, that are “reported” by the UE. Note that such RS/beam measurements “reported” by the UE are with legacy CSI reporting mechanisms, i.e., with periodic/semi-persistent/aperiodic CSI reporting, included in resourcesForChannelMeasurement, also in CSI-ReportConfig.

timeRestrictionForChannelMeasurements in CSI-ReportConfig applies to the RS/beam measurements, i.e., either SSB or CSI-RS measurements, that are reported by the UE.

##### 5.2.1.4.3 L1-RSRP Reporting For L1-RSRP computation - the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'type C' and 'typeD' when applicable. - the UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128. --- omitted text --- If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

When a UE is configured with UEIBM, i.e., with a CSI-ReportConfig including higher layer parameter eventTypeUE-IBR, assuming the main case of Event-2, when the event conditions are met, the UE reports a certain number of RSs/beams that are part of the set of new beams (see green highlight below from clause 5.2.1.5.4.1 in the most recent version of TS 38.214 (version 19.1.0)). Note that the set of new beams has been initially denoted with a new high layer parameter newBeamResourceSet, still currently used in the most recent version of 38.214: on the other hand, we noted that RAN2 has recently agreed to reuse for the set of new beams exactly the legacy high layer parameter resourcesForChannelMeasurement, and that is why newBeamResourceSet does not appear in the most recent version 19.0.0 of TS 38.331, see also the Appendix for that.

Note that the current beam may not be reported by the UE. On one side, the network is not obliged to include the current beam into resourcesForChannelMeasurement. On the other side, the UE must report the current beam when high layer parameter enabledCurrentBeamReport (also for this parameter, we observe that RAN2 has recently agreed to slighly update its name into currentBeamReport) is enabled in CSI-ReportConfig (see cyan highlight below still from clause 5.2.1.5.4.1 in the most recent version of TS 38.214 (version 19.1.0)).

5.2.1.5.4.1a UE Initiated CSI reporting for event 2 For a UE configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’, an event instance is determined for a reference signal configured by the newBeamResourceSet if the L1-RSRP value determined for a transmission occasion of such reference signal is an eventThreshold greater than the L1-RSRP value determined for a transmission occasion of - the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition, else - the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state (applied to the same CC of CSI-ReportConfig if CCofIndicatedTCI is not configured, or to the CC indicated by CCofIndicatedTCI in the CSI-ReportConfig), if newBeamResourceSet is a CSI-SSB-ResourceSet. After transmitting UEIRI, the UE reports, as defined in Clause 6.3.2.1.2 of [5, TS 38.212], in a single reporting instance nrofReportedRS-UEIBR CRIs or SSBRIs corresponding to reference signals provided by the newBeamResourceSet that comprise at least one reference signal that triggers the UEIRI transmission. For each CSI or SSBRI, the CSI report includes the absolute L1-RSRP or differential L1-RSRP and, when PresenceOfConditionMetIndicator is configured, condition met indicator indicating whether the reference signal indicated by reported CRI or SSBRI triggers the UEIRI transmission, and, when enabledCurrentBeamReport is configured, the differential L1-RSRP corresponding to the reference signal in the indicated TCI state, or to the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state. The UE sends the CSI report - on a PUSCH indicated by the DCI format 0_1/0_2 in a PDCCH reception if reportTransmissionMode is configured as ‘ModeA’ and the CSI trigger state associated with the CSI-ReportConfig is indicated in the CSI request field in the DCI format 0_1/0_2, or - on a type 1 CG-PUSCH configured by configuredPUSCHResourceOfModeB in the same CC as the corresponding CSI-ReportConfig, on the first available transmission occasion numOfSymbols-ModeB symbols after the end of the transmitted PUCCH if reportTransmissionMode is configured as ‘ModeB’, where the periodicity of the PUCCH resource and type 1 CG-PUSCH resource is the same, numOfSymbols-ModeB is based on the numerology of the PUCCH resource with UEIRI transmitted, and the CG-PUSCH does not carry UL-SCH.

Because of that, in our view:

In the current version of the specifications, timeRestrictionForChannelMeasurements in a UEIBM CSI-ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR:

- applies to the new beams, i.e., to the RSs/beams included in high layer parameter resourcesForChannelMeasurement;

- does not always apply to the current beam.

Therefore, to properly capture RAN4 agreement and LS R4-2511656, we think that we should explicitly write in the specifications that the value of M determined for the measurement period of the new beams by timeRestrictionForChannelMeasurement in a UEIBM CSI report configuration is also used for the current beam, to avoid the ambiguity in case either the current beam is not part of resourcesForChannelMeasurement or currentBeamReport is not enabled.

Capture as proposed in our companion CR that the value of M determined for the new beams by timeRestrictionForChannelMeasurements in a UEIBM ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR, is also used for the current beam.

Conclusion

In this paper, the following observations and proposals were made:

Observation 1: Current specification does not define starting and ending point for timer-based reporting.

Observation 2: If report is triggered by a restarted timer, event condition may have become met already during the previous timer, in which case the agreed timer-based reporting delay may not be sufficient to cover the time from event condition becoming met until the first PUCCH.

Proposal 1: When eventDetectionTimeWindowLength-r19 is configured, the event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface.

a.If a timer is already running at the time of the event that will trigger a measurement report, the time until start of the timer during which the report is triggered is added in the event triggered measurement reporting delay

Observation 3: timeRestrictionForChannelMeasurements in CSI-ReportConfig applies to the RS/beam measurements, i.e., either SSB or CSI-RS measurements, that are reported by the UE.

Observation 4: In the current version of the specifications, timeRestrictionForChannelMeasurements in a UEIBM CSI-ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR:

-	applies to the new beams, i.e., to the RSs/beams included in high layer parameter resourcesForChannelMeasurement;

-	does not always apply to the current beam.

Proposal 2: Capture as proposed in our companion CR that the value of M determined for the new beams by timeRestrictionForChannelMeasurements in a UEIBM ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR, is also used for the current beam.

- References

- R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, RAN4#116bis

- R4-2520472, CR for UEIBM reporting delay, Nokia, RAN4#117

- Appendix. CSI-ReportConfig with UEIBM

CSI-ReportConfig information element -- ASN1START -- TAG-CSI-REPORTCONFIG-START CSI-ReportConfig ::= SEQUENCE { reportConfigId CSI-ReportConfigId, carrier ServCellIndex OPTIONAL, -- Need S resourcesForChannelMeasurement CSI-ResourceConfigId, csi-IM-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R nzp-CSI-RS-ResourcesForInterference CSI-ResourceConfigId OPTIONAL, -- Need R reportConfigType CHOICE { periodic SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH SEQUENCE { reportSlotConfig CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH SEQUENCE { reportSlotConfig ENUMERATED {sl5, sl10, sl20, sl40, sl80, sl160, sl320}, reportSlotOffsetList SEQUENCE (SIZE (1.. maxNrofUL-Allocations)) OF INTEGER(0..32), p0alpha P0-PUSCH-AlphaSetId }, aperiodic SEQUENCE { reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF INTEGER(0..32) } }, --- omitted text --- timeRestrictionForChannelMeasurements ENUMERATED {configured, notConfigured}, timeRestrictionForInterferenceMeasurements ENUMERATED {configured, notConfigured}, codebookConfig CodebookConfig OPTIONAL, -- Need R dummy ENUMERATED {n1, n2} OPTIONAL, -- Need R groupBasedBeamReporting CHOICE { enabled NULL, disabled SEQUENCE { nrofReportedRS ENUMERATED {n1, n2, n3, n4} OPTIONAL -- Need S } }, cqi-Table ENUMERATED {table1, table2, table3, table4-r17} OPTIONAL, -- Need R subbandSize ENUMERATED {value1, value2}, non-PMI-PortIndication SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerConfig)) OF PortIndexFor8Ranks OPTIONAL, -- Need R --- omitted text --- CSI-ReportUE-IBR-r19 ::= SEQUENCE { eventTypeUE-IBR-r19 CHOICE { event1-r19 SEQUENCE { eventThreshold-r19 RSRP-Range }, event2-r19 SEQUENCE { eventThreshold-r19 INTEGER (0..31) }, event7-r19 SEQUENCE { eventThreshold-r19 INTEGER (0..31), valueOfQ-r19 INTEGER (1..8) } }, reportTransmissionMode-r19 CHOICE { modeA-r19 NULL, modeB-r19 SEQUENCE { pusch-ResourceOfModeB-r19 SEQUENCE { configuredGrantConfigIndex-r19 ConfiguredGrantConfigIndex-r16, ul-BWP-Id-r19 BWP-Id, servCellIndex-r19 ServCellIndex }, minimumPucch-PuschOffset-r19 ENUMERATED {symb0, symb1, symb2, symb4, symb8, symb16, symb32, symb64, symb128, symb256, symb512} --Editor’s note: minimumPucch-PuschOffset can be updated based on further RAN1 discussion. } }, nrofReportedRS-UE-IBR-r19 ENUMERATED {n1, n2, n3, n4}, tci-ServCellIndex-r19 ServCellIndex OPTIONAL, -- Need R currentBeamReport-r19 ENUMERATED {enabled} OPTIONAL, -- Need R conditionFulfillmentIndicator-r19 ENUMERATED {enabled} OPTIONAL, -- Need R eventCountWindow-r19 SEQUENCE { eventInstanceCount-r19 INTEGER (2..16), eventDetectionTimeWindow-r19 ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280} } OPTIONAL, -- Need R pucch-Resource-r19 SEQUENCE { periodicityAndOffset CHOICE { sym2 NULL, sym6or7 NULL, sl1 NULL, -- Recurs in every slot sl2 INTEGER (0..1), sl4 INTEGER (0..3), sl5 INTEGER (0..4), sl8 INTEGER (0..7), sl10 INTEGER (0..9), sl16 INTEGER (0..15), sl20 INTEGER (0..19), sl40 INTEGER (0..39), sl80 INTEGER (0..79), sl160 INTEGER (0..159), sl320 INTEGER (0..319), sl640 INTEGER (0..639) }, resource PUCCH-ResourceId, ul-BWP-Id-r19 BWP-Id, pucch-Cell-r19 ENUMERATED {spCell, pucch-Scell} } }