3GPP TSG-RAN4 Meeting #117	R4-2520476

Dallas, US, 17 – 21 November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR TC for Intra-cell SSB-based UEIBM reporting in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of test cases for NR_MIMO_Ph5 WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of the following test cases: - Intra-cell SSB-based event triggered reporting for UE initiated beam management without eventDetectionTimeWindowLength-r19 - Intra-cell SSB-based event triggered reporting for UE initiated beam management with eventDetectionTimeWindowLength-r19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature is not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.6.6.1.X, A.6.6.1.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Change #1 >

#### A.6.6.1.X	Intra-cell SSB-based event triggered reporting for UE initiated beam management without eventDetectionTimeWindowLength-r19

##### A.6.6.1. X.1	Test Purpose and Environment

The purpose of this test is to verify event triggered L1-RSRP measurement reporting delay for eventType-r19 set to ‘event2’ defined in clause 9.5.3.4. Supported test configurations are specified in table A.6.6.4.1.1-1.

##### A.6.6.1.X.2 	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). The general and cell specific test parameters as specified in tables A.6.6.4.1.2-1 and A.6.6.4.1.2-2 apply except for the parameters defined in this clause. Test parameters specific to this test are given in table A.6.6.1.X.2-1 and table A.6.6.1.X.2-2 below.

In CSI measurement configuration, the UE is configured with eventType-r19 set to ‘event2’, dl-OrJointTCI-StateList and without eventDetectionTimeWindowLength-r19. The test is conducted for reportTransmissionMode ‘ModeA’ and consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During test period T1, the UE receives from SSB#0, which defines the the current beam. During test period T2, the TE also transmits SSB#1, which defines the new beam.

Table A.6.6.1.X.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| resourcesForChannelMeasurement | 1~3 |  | SSB#1 |
| eventThreshold-r19 | 1~3 | dB | 5 |
| reportTransmissionMode-r19 | 1~3 |  | ModeA |
| nrofReportedRS-UE-IBR | 1~3 |  | 1 |
| currentBeamReport | 1~3 |  | enabled |
| pucch-Resource-r19: periodicityAndOffset | 1,2 | slot | 20 |
|  | 3 |  | 40 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |

Table A.6.6.1.X.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.64 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -79.65 |
|  | 3 |  | -91.64 | -91.64 | -Infinity | -76.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -51.56 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -60.59 | -45.46 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| NOTE 1: The resources for first uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test Requirements

The UE shall send an indication on the first PUCCH within the measurement reporting delay defined in clause 9.5.3.X from the beginning of time period T2. The UE shall not send an indication on the first PUCCH as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

< End of change1  >

< Change #2 >

#### A.6.6.1.Y	Intra-cell SSB-based event triggered reporting for UE initiated beam management with eventDetectionTimeWindowLength-r19

##### A.6.6.1.Y.1	Test Purpose and Environment

The purpose of this test is to verify event triggered L1-RSRP measurement reporting delay for eventType-r19 set to ‘event2’ with eventDetectionTimeWindowLength-r19 as defined in clause 9.5.3.4. Supported test configurations are specified in table A.6.6.4.1.1-1.

##### A.6.6.1.Y.2 	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). The general test parameters as specified in table A.6.6.4.1.2-1 and A.6.6.4.1.2-2 apply except for the parameters defined in this clause. Test parameters specific to this test are given in table A.6.6.1.Y.2-1 and table A.6.6.1.Y.2-2 below.

In CSI measurement configuration, the UE is configured with eventType-r19 set to ‘event2’, dl-OrJointTCI-StateList and eventDetectionTimeWindowLength-r19. The test is conducted for reportTransmissionMode ‘ModeA’ and consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During test period T1, the UE receives from SSB#0, which defines the the current beam. During test period T2, the TE also transmits SSB#1, which defines the new beam.

Table A.6.6.1.Y.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| eventThreshold-r19 | 1~3 | dB | 5 |
| reportTransmissionMode | 1~3 |  | ModeA |
| eventDetectionTimeWindowLength-r19 | 1~3 | ms | 80 |
| eventInstanceCount-r19 | 1~3 |  | 3 |
| nrofReportedRS-UE-IBR | 1~3 |  | 1 |
| currentBeamReport | 1~3 |  | enabled |
| pucch-Resource-r19: periodicityAndOffset | 1,2 | slot | 20 |
|  | 3 |  | 40 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |

Table A.6.6.1.Y.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.64 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -79.65 |
|  | 3 |  | -91.64 | -91.64 | -Infinity | -76.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -51.56 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -60.59 | -45.46 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| NOTE 1: The resources for first uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.6.1.X.X.3	Test Requirements

The UE shall send an indication on the first PUCCH within the measurement reporting delay defined in clause 9.5.3.X from the beginning of time period T2. The UE shall not send an indication on the first PUCCH as long as the reporting criteria with eventInstanceCount-r19 is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

< End of changes >