3GPP TSG-RAN WG4 Meeting 117	R4-2520491

Dallas, USA, Nov. 17-21, 2025

Source: 	Xiaomi

Title: 	Overview for 6GR RRM

Agenda Item:	8.6

Document for:	Discussion

- Overview

Radio Resource Management (RRM) is foundational to cellular performance. As 6G emerges—with heterogeneous use cases, proliferating bands, multi-source/beam operations, and cross-domain measurement demands—the limits of today’s isolated, task-driven RRM become explicit. Task-by-task optimization has hit its scalability ceiling in 5G, producing redundancy, conflicts, O(n²) integration costs, and avoidable power/latency overhead that incremental patching can no longer cure.

The path forward is architecture-first: define a unified, resource-aware measurement framework (kernel + capability model), and let tasks plug in as compliant components. This shifts RRM from siloed procedures to a coherent system that is efficient, predictable, and evolvable for 6G.

- RRM framework

Our Core proposition is this:

- 6G is not about adding more tasks — it is about upgrading the architecture. We do not unify measurements; we unify the Measurement System. Architecture first. Tasks follow.

      - 2.1 Unified Measurement framework

The 3GPP RAN4 working group has systematically evolved measurement requirements to support diverse NR features, culminating in the layered UE measurement framework of TS 38.133. We classify the related measurement tasks into four domains:

- Protocol: L1 and L3 measurement for mobility

- Spatial Domain:

- Inter-cell Measurement for different Transmission Source in L1, e.g. Multiple TRPs for MIMO, Multiple neighbor cells for mobility

- Intra-cell Measurement for the same Transmission Source in L1, e.g. RLM，BFD, CBD, L1-RSRP for serving cell

- Time Domain: Idle mode and connected mode

- Frequency Domain: intra-frequency and inter-frequency

Fig. 2-1 Illustration of NR measurement tasks

- 2.1.1 The True Nature of TS 38.133

As illustrated in last section, more than 70% contents of 38.133 defines the requirement for various measurement task. In 38.133, We mainly defined five aspects for each measurement:

      - - Measurement time

      - - Measurement restriction

      - - Scheduling restriction

      - - Guaranteed performance (Accuracy)

      - - Related UE capability

A systematic analysis of TS 38.133 reveals that its main clauses are direct specification of core Measurement Operation System kernel, establishing its architectural design through normative requirements and UE behavior.

      - Next, we will explain why they are specifications of an OS kernel.

      - 2.1.1.1 Why 38.133 defines the Kernel for a Multi-Task Measurement OS

This fundamental problem—managing multiple tasks on a platform with constrained resources and guarantee the performance—is precisely the problem that an Operating System (OS) kernel is designed to solve. Consequently, the main work of RAN4 is how to design of a "Multi-task Measurement OS Kernel."

This analogy is not merely illustrative; it is functionally exact. Figure 2-2 shows that just as a PC OS kernel manages applications, RAN4’s rules manage measurement tasks. The kernel API layer, defined by configurations like ResourceConfig and ReportConfig, serves as the standardized interface between the “applications” (defined by RAN1/2) and the kernel (managed by RAN4).

Figure 2-2: The ‘Measurement OS’

This direct one-to-one correspondence can be clearly visualized:

Table 2-1: Analogy between PC/Mobile OS and Measurement OS

| Familiar PC/Mobile OS | The Measurement “OS” Designed by RAN4 |
| --- | --- |
| Applications (Apps) | Measurement Tasks (e.g., RLM, BFD, mTRP, LTM) |
| Platform Constraints (CPU, Memory, I/O) | Platform Constraints (FFT Units, Buffers, Searchers, etc.) |
| OS Kernel | The Measurement Kernel |
| - Process Scheduling | - Measurement Scheduling |
| - Conflict Arbitration | - Conflict Resolution (via Measurement/Scheduling Restrictions) |
| - Scheduling Unit (Time Slice) | - Scheduling Unit (Reference Signal Periodicity) |
| - Guaranteed App performance | - Guaranteed Measurement accuracy |

The specification of this Measurement OS Kernel is architected around four interdependent core functions, as defined by TS 38.133:

- Platform constraints Abstraction: UE Capabilities

  - Function: To model the constraints of the underlying hardware/software platform.

  - Kernel Implementation: Defined through assumptions and normative references to UE Capabilities (e.g., SearcherNumber, FFT Units, total number of cells, frequency layers).

  - Analogy: This abstracts the "hardware specs" that the kernel must manage.

- The Scheduler: Sharing Factors

  - Function: To allocate limited measurement opportunities and processing resources across competing tasks.

  - Kernel Implementation: Instantiated through the framework of Sharing Factors (e.g., P_L1, P_L3, P_BFD/P_CBD, P_TRP, CSSF, etc.) and the defined Measurement Time, which together form a multi-domain (time, spatial) scheduling algorithm.

  - Analogy: The core CPU scheduler that decides which "process" (measurement task) gets the "CPU time" (RS occasion) and for how long.

- The Arbiter: Restriction Rules

  - Function: To provide final, hard-line arbitration when scheduling is insufficient to resolve conflicts.

  - Kernel Implementation: Executed through two critical sets of rules: Measurement Restrictions (for inter-task conflict) and Scheduling Restrictions (for conflict between measurement and data planes).

  - Analogy: The kernel's priority and preemption rules that ensure system stability when resources are critically contended.

- Performance Guarantees: Accuracy Requirements

  - Function: To define the Quality of Service (QoS) target for each measurement task, ensuring the system's output is meaningful.

  - Kernel Implementation: Specified as normative Measurement Accuracy requirements (e.g., RSRP accuracy under defined conditions).

  - Analogy: The system's performance contract, guaranteeing that applications (measurement tasks) meet their functional performance goals. It is the ultimate validation criterion for the scheduler's effectiveness.

Therefore, we can answer the question "What does TS 38.133 define?" with a precise and profound definition:

TS 38.133, together with the UE capabilities it references, collectively defines the kernel of a "Multi-Task Measurement Operating System."

It reveals the profound nature of RAN4's work: we are not merely definers for each designed feature, but the architects of a critical embedded measurement system kernel that manages both resources and performance outcomes.

Observation 2-1: TS 38.133, together with referenced UE capabilities, defines the "Multi-Task Measurement OS Kernel." This kernel manages the UE's measurement subsystem through platform abstraction, resource scheduling, and conflict arbitration.

Therefore, the main work of RAN4 is how to define the multi-task Measurement Kernel from system-wide aspect.

- 2.1.2 The Foundational Gap: Task-Driven Design vs. Systemic Execution

Having established the ‘Measurement OS Kernel’ as the correct analytical model, we now identify a fundamental architectural flaw in the current 5G measurement framework: the misalignment between task-independent standard design and UE’s inherent multi-task execution reality.

2.1.2.1 The Core Gap: Per-Task Standards vs. Systemic UE Execution

RAN4 has traditionally operated as a downstream player in 3GPP, defining performance requirements for each measurement task in isolation—a per-task design paradigm.

However, in a real UE, all these tasks must execute concurrently on shared hardware, competing for limited resources. RAN4 thus implicitly functions as the de facto architect of a multi-task measurement OS, yet without the explicit architectural mandate to design it systematically.

This discrepancy defines the critical gap:

Observation 2-2: A fundamental gap exists between the per-task, independent design of 3GPP measurement specifications and the inherently multi-task, resource-constrained execution environment of a real UE.

2.1.2.2 Symptoms of a Fragmented Kernel

This gap manifests as a ‘Fragmented Kernel’ architecture, characterized by six systemic pathologies, as shown in Feature 2-3.

Figure 2-3: Task-Driven fragmented design

- Redundancy: The system fails to recognize common physical measurement objects.

  - Examples: L1 and L3 performing independent RSRP measurements on the same SSB; Duplicate Gap counting in L1 and L3. RLM and BFD, L1-RSRP and CBD conducting similar measurements; mTRP and LTM configuring duplicate measurements for the same physical transmission source(serving cell).

- Conflicts: Decentralized specification creates contradictory rules and scheduling logics.

  - Examples: Serving cell L1-RSRP being assigned different scaling factors(P factor) in LTM vs. mTRP contexts; BFD/CBD and L1-RSRP applying inconsistent scaling factors for multi-CC/multi-TRP scenarios; fundamental conflict between L1 (FFT-based) and L3 (Searcher-based) carrier aggregation measurement models.

- Missing Coordination: Critical multi-task and cross-state coordination mechanisms are undefined.

  - Examples: No standardized arbitration between concurrent LTM (mobility) and mTRP (throughput) measurement tasks; measurement context (e.g., identified beams, neighbor cells) acquired in Idle/Inactive state being discarded upon transition to Connected state.

- Inefficient Static Allocation: Resource allocation strategies are rigid and non-adaptive.

  - Examples: Fixed use of N=8 beams for all FR2 spatial scanning, regardless of criticality; static inter-frequency measurement priorities that cannot adapt to dynamically prioritize high-load or critical carriers.

- Fragmented UE Capability Definition: Capabilities are siloed per-task, preventing dynamic resource pooling.

  - Examples: Separate definitions for RTD>CP capability and simultaneousRxDataSSB-DiffNumerology capability, despite relying on the same underlying FFT processor resource; task-specific buffer sizes (e.g., for LTM max neighbors vs. mTRP) constraining the same physical memory pool.

- Unsustainable Evolution Model: The paradigm creates prohibitive complexity for introducing new features.

  - Examples: O(n²) integration complexity requiring each new feature to bilaterally negotiate with all existing tasks; protocol redundancy from repetitive definition of similar measurement rules, timelines, and sharing factors across multiple specification sections.

These interconnected failure modes collectively degrade performance, increase power consumption, and hinder the evolution towards 5G-Advanced and 6G.

Observation 2-3: The systemic pathologies observed in 5G—redundancy, conflicts, lack of coordination, static allocation, fragmented capabilities, and unsustainable evolution—are the direct and predictable symptoms of the underlying ‘Fragmented Kernel’ architecture.

2.1.2.3 Real-World Impact: From Standard Flaws to System-Wide Inefficiency

These architectural flaws directly impair both UE implementation and network performance:

I. UE-Side Consequences:

- Spiraling Complexity: Vendors must develop proprietary solutions to resolve standard-level conflicts and gaps.

- Inconsistent Performance: Divergent implementations lead to unpredictable UE behavior and performance variance.

- Increased Power Consumption: Redundant operations and inefficient scheduling escalate power usage.

II. Network-Side Consequences:

- Reduced Throughput: Unnecessary UE measurement restrictions constrain data transmission opportunities.

- Extended Measurement Latency: Inefficient task execution delays critical report delivery, slowing network decision-making.

- Suboptimal System Performance: The network cannot achieve global resource optimization based on fragmented UE measurement data.

Observation 2-4: This gap increases UE cost and power consumption, degrades network throughput and responsiveness, and ultimately hinders the scalable deployment of 6G.

2.1.2.4 The Urgency: Crossing the Complexity Threshold

A critical question arises: if the ‘Fragmented Kernel’ is so flawed, why did the system not collapse in 4G? The answer lies in a non-linear shift in complexity that has pushed us past an architectural breaking point.

Figure 2-4 below illustrates the exponential rise in measurement task complexity from 4G → 5G → 6G, and the threshold beyond which per-task rules collapse and an OS-like Kernel becomes necessary. 5G crossed the architectural threshold but stayed with LTE-era per-task thinking. 6G cannot repeat this — it needs to start with a deliberately designed kernel.

Figure 2-4. Crossing the Complexity Threshold: 6G Must Architect for a Kernel

In the LTE Era — Below the Threshold (Low Contention): The measurement framework was defined by a relatively small number of tasks with limited concurrency. Resource contention was minimal and manageable. In this environment, simple, decentralized, and static per-task rules were sufficient to ensure adequate performance.

In the 5G Era— Crossing the Threshold (High Contention): The dramatic increase in concurrent measurement tasks (e.g., L3, RLM, BFD, mTRP, LTM) in 5G creates intense and conflicting demands for finite UE resources like time, processing, and spatial beams. The old model of isolated, static rules is mathematically incapable of resolving this high-dimensional contention, causing the conflicts and inefficiencies we face today.

The 6G — Beyond the Threshold (Extreme Contention): Looking towards 6G, this contention will escalate to an extreme degree. Complexity is projected to explode across multiple dimensions: quantitatively, through denser mTRP and cell deployments; qualitatively, by integrating new domains like Non-Terrestrial Networks (NTN); and dynamically, with the introduction of new tasks, e.g. VUEG, ISAC.

Therefore, the demand for a Measurement OS Kernel is not a critique of past designs. It is an evolutionary necessity—the architectural response required to manage a system that has crossed a critical threshold of complexity.

Observation 2-5: 5G crossed the measurement complexity threshold where per-task rules and a fragmented architecture break down, making the adoption of a unified Measurement Kernel an evolutionary necessity for 6G.

- 2.1.3 Closing the Gap: A Unified Architecture-First Framework

Having diagnosed the root cause, identified the symptoms, and established the urgency, the path forward becomes clear. We must transition from unintentionally maintaining a fragmented system to intentionally designing a unified one.

2.1.3.1 The Guiding Philosophy: ‘Unify’

The core of the "Unify" philosophy is:

- system architecture must precede task implementation. It means we stop defining measurement tasks in isolation and start designing them as parts of an integrated system. The goal is to optimize the overall system performance, rather than merely setting individual performance targets.

This philosophy demands a decisive paradigm shift:

- From Distributed Rules to Centralized Governance: Replace task-specific, independent measurement procedures with a unified scheduling authority—the Measurement OS Kernel.

- From Feature-Led to Architecture-Led: All tasks, both existing and future, must integrate as plug-ins to a central framework

Adopting this Unified architecture-first approach yields critical benefits, filling the gap between standard and Real implementation:

- For Standards: This turns complex negotiations into a streamlined process.

- For UE Implementation: This simplifies design, cuts R&D costs, and eliminates the need for proprietary fixes.

- For System Performance: It enables global optimization.

2.1.3.2 The Three Layer Unified Framework: A Blueprint for Sustainable Evolution

Figure 2-5: The "Unified" Philosophy: Transitioning from a Single-Task to a Multi-Task System Architecture

This architectural design follows the proven layered model of modern OS.

Our framework establishes three distinct layers, each with well-defined responsibilities (see Figure 2-5). This structure materializes the "Unify" philosophy and achieves two key goals:

- To decouple RAN4's system optimization work from the feature-specific work in RAN1/RAN2.

- To clearly define RAN4's mandate as the architect of the system-level measurement kernel and abstracted capability model.

The Three-Layer Architecture

      - Unified Task Interface (RAN1/RAN2 Domain)

This layer acts as the OS's application layer. It comprises existing measurement configurations (e.g., ResourceConfig, ReportConfig) defined by other groups. These form a natural "task interface," specifying what to measure and report. This allows RAN1/RAN2 to focus on feature design, relying on a stable interface to the system below.

Ongoing efforts in other WGs to unify configurations validate our approach. This work rightly belongs in this interface layer. Our framework clarifies this boundary, cleanly separating configuration design (other WGs) from kernel scheduling (RAN4).

Note: Detailed scheduling rules remain under RAN4. While parameters from other WGs (e.g., an L1-RSRP filter coefficient) can influence a single task's behavior, these are considered task-local parameters. They govern internal task processing, not global resource arbitration between tasks, and thus do not conflict with the kernel's role as system-wide manager.

      - Unified Measurement OS Kernel (RAN4 Core)

This layer is the intelligent core deliberately designed to systematically resolve the architectural pathologies identified in Section 2. We clarify that this is a conceptual architectural model formalizing and enhancing functions UEs already perform; it does not mandate new APIs.:

- To eliminate Redundancy: A Fusion Engine Enables "measure once, reuse across tasks," preventing duplicate measurements of the same physical signal by L1/L3 or internal tasks.

- To resolve Conflicts: A Global Scheduler acts as a single source of truth, implementing conflict-free resource allocation and replacing contradictory, task-specific rules with a unified policy.

- To enable Coordination: A Coordination Arbiter provides standardized mechanisms for multi-task (e.g., LTM vs. mTRP) and cross-state coordination, replacing implementation-dependent behavior.

- To overcome Static Allocation: A Resource Manager dynamically pools and allocates FFT units, buffers, and beam resources, enabling adaptive rather than rigid strategies.

      - Unified UE Capability (RAN4 Core)

This foundational layer, also under RAN4's purview, abstracts hardware into system-wide resource pools (compute, memory, spatial, FFT, searcher, Panel number, etc.), enabling the kernel's dynamic optimization.

Ultimately, this architecture is designed to solve the unsustainable evolution model. It provides a stable integration path for future tasks: instead of the current O(n²) complexity of bilateral negotiations, any new measurement feature is integrated by translating its requirements through the stable Unified Task Interface. The kernel then automatically absorbs it into its global optimization regime. This transforms the standardization process from perpetual patching into a predictable, scalable integration, fundamentally solving the problem of protocol bloat and complexity explosion.

Therefore, RAN4 to define the Unified Measurement Framework through a three-layer architecture that establishes complementary unification paths:

- Unified Task Interface (RAN1/RAN2): Provides the stable abstraction for what to measure, enabling other WGs' ongoing work to unify feature configurations (ResourceConfig, ReportConfig) while ensuring system compatibility.

- Unified Measurement Kernel (RAN4 core): Focuses on how to measure efficiently, providing system-level optimization—global scheduling, conflict resolution, and measurement reuse—that elevates the performance of all measurement tasks.

- Unified UE Capabilities (RAN4 core): Abstracts hardware/software into dynamically allocable resource pools (compute, memory, FFT, searcher, etc.), creating the foundation that enables kernel optimization.

2.1.3.3 System Primacy in Resource Arbitration

Within the proposed unified architecture, the Measurement Kernel serves as the system-wide resource coordinator. While RAN4 defines the global scheduling policy, we fully recognize that other working groups may define task-specific parameters that influence measurement behavior.

When a potential conflict arises between such parameters and the system-level scheduling goal, RAN4 shall proactively engage with the relevant working groups to coordinate a solution that preserves global efficiency. This collaborative approach ensures that the kernel can fulfill its role as a coherent system optimizer without undermining the feature-specific intents of other WGs.

The scope of this coordination is internal UE scheduling. It does not relax conformance to standardized task definitions, and explicitly designated mandatory constraints from other WGs will be respected.

Observation 2-6: RAN4 shall act as the system coordinator, resolving conflicts between global scheduling and task-specific parameters to preserve overall efficiency.

- 2.1.4 Research Focus: A Future-Proof Framework Through Four-Dimensional Unification

2.1.4.1 A Four-Dimensional Path to a Three-Dimensional Foundation

Current measurement scheduling is overly complex. The root cause is an architectural gap: we try to manage a four-dimensional space—Protocol (L1/L3), Spatial, Time, and Frequency—using only fragmented, one-dimensional rules.

This four-dimensional problem has historical roots. L3 was originally for slow mobility, L1 for fast data link adaptation. However, 5G's low-latency demands pushed critical mobility functions into L1. This created parallel, often redundant procedures across both layers. The protocol dimension is now an artificial barrier to efficiency.

Figure 2-6: 4-Dimensinal Joint Optimization for Measurement OS kernel

The solution requires a key insight: while the problem is 4D, the underlying physical resources exist in only three irreducible dimensions: Space, Time, and Frequency.

Our proposed framework solves this with a decisive two-step transformation:

- Unifies the Protocol Dimension: The kernel dissolves the L1/L3 boundary. It abstracts all tasks into protocol-agnostic resource requests, eliminating architectural redundancy.

- Manages Pure Physical Resources: The kernel then acts as a central optimizer, scheduling these unified requests across the 3D foundation of Space, Time, and Frequency.

This shift—from managing four dimensions of conflict to optimizing three dimensions of unified resources—is our "Return to a Pure 3D Framework". It is inherently future-proof, as any new measurement task can only be a new combination of demands on this stable physical foundation.

2.1.4.2 Research Focus: The Four Pillars of Unification

To realize this vision, our research will concentrate on four key unification efforts:

Research Pillar 1: Protocol Dimension Unification (Eliminating the 4th Dimension)

  - Objective: To eliminate cross-layer redundancy and conflict by creating a single, unified measurement model that transcends the traditional L1/L3 boundary.

- Exploratory Directions:

  - FR1:

    - Study a unified capability assumption for FR1 that reconciles the L3 "Searcher" and L1 "FFT" paradigms, harmonizing their conflicting approaches to PCC/SCC measurement and timing constraints (e.g., RTD>CP).

  - FR2:

    - Investigate unified beam operation models that reconcile wide and narrow beams into a single beam type

    - Study layer-based scheduling mechanisms instead of cell-based scheduling for L1. e.g. RX beam scanning over spatial not per cell, just like what L3 does.

Research Pillar 2: Spatial Dimension Unification for L1

Spatial domain management refers to how measurement tasks target signal sources in physical space. These sources may be Intra-Source (the same transmitter) or Inter-Source (different transmitters), creating two distinct efficiency challenges:

- Part A: Intra-Source Optimization addresses redundancy when multiple tasks (e.g., RLM, BFD, CBD, L1-RSRP) independently target the same transmitter. Inconsistent scaling factors for multi-CC/mTRP scenarios further introduce measurement restrictions.

- Part B: Inter-Source Coordination resolves contention when different tasks (e.g., LTM, mTRP) targeting different transmitter compete for finite spatial resources. This requires unified conflict resolution and scanning procedures.

This two-part structure ensures we solve both layers of the spatial challenge: first optimizing resource usage within individual sources, then coordinating resource allocation across multiple sources.

  - Exploratory Directions for Part A:

    - Study the feasibility of "common-source task groups" with shared measurement result

    - Investigate unified and consistent scaling rules for these task groups in multi-CC and multi-TRP scenarios

  - Exploratory Directions for Part B:

    - Investigate unified spatial scanning procedures that can serve multiple measurement sources simultaneously

    - Explore hierarchical spatial management schemes that differentiate scanning periodicity based on spatial criticality

    - Investigate source-centric measurement approaches to eliminate duplicates across tasks in dense multi-TRP environments, e.g. avoid duplicate measurement for same PCI

Research Pillar 3: Frequency Dimension Unification

  - Objective: To establish a holistic frequency domain management strategy that implements system-wide carrier prioritization and enables intelligent measurement reuse across frequencies.

  - Exploratory Directions:

    - Define dynamic carrier priority rules that adapt to network conditions and UE state, moving beyond static priority assignments.

    - Investigate cross-carrier measurement reuse opportunities in multi-component carrier scenarios, identifying where measurements on one carrier can satisfy requirements for another.

Research Pillar 4: Time Dimension Unification

  - Objective: To establish temporal continuity by bridging IDLE/INACTIVE and CONNECTED states, enabling context reuse and scheduling to eliminate “cold-start” inefficiencies.

Observation 2-7: RAN4 shall investigate a unified, resource-centric measurement framework for 6G. This framework shall transcend traditional protocol layer boundaries (L1/L3) to enable the joint optimization of spatial, temporal, and spectral resources across all measurement tasks, thereby maximizing system-level performance and power efficiency.

- 2.1.5 Summary

In summary, our proposal for Unified Measurement is as below:

Proposal 2-1: RAN4 shall recognize that TS 38.133, together with referenced UE capabilities, constitutes the specification of the UE's "Multi-Task Measurement OS Kernel" — a system responsible for platform capability abstraction, resource scheduling, and conflict arbitration.

Proposal 2-2: RAN4 shall adopt the system-architect role for 6G, with the explicit mandate to bridge the gap between task-independent specification and systemic UE execution. This establishes RAN4 as the core architect of the UE's measurement infrastructure.

Proposal 2-3: RAN4 shall apply an architecture-first principle. A unified framework must be defined first, into which all new and existing measurement tasks are integrated as compliant components, ensuring consistent and scalable system behavior.

Proposal 2-4: RAN4 shall investigate a unified, resource-centric measurement framework for 6G based on a three-layer architecture, which decouples RAN4’s work from other groups:

- Unified Task Interface (RAN1/RAN2): Provides the stable abstraction for what to measure, enabling other WGs' ongoing work to unify feature configurations e.g. Unified ResourceConfig, Unified ReportConfig, while ensuring system compatibility.

- Unified Measurement Kernel (RAN4 core): Focuses on how to measure efficiently. It acts as a resource-centric optimizer, performing global scheduling, conflict resolution, and measurement reuse across spatial, temporal, and spectral resources to maximize system performance and power efficiency.

- Unified UE Capabilities (RAN4 core): Abstracts hardware/software capability(compute, memory, FFT, searcher, etc.), creating the foundation for kernel optimization.

Proposal 2-5: RAN4 shall proactively coordinate with other WGs. When task-specific parameters conflict with system-level scheduling policy, RAN4 will lead the resolution to preserve both feature intent and overall system efficiency.

      - 2.2 Virtual UE Group: A Collaborative Approach for 6G

The Virtual UE Group (VUEG) is a promising 6G technology whose core idea aligns perfectly with our "Unified Measurement Framework." Both concepts believe we should not just optimize single functions in isolation, but seek the highest efficiency from a whole-system perspective. VUEG takes this a step further, extending the idea from a single phone to a "collaborative team" of multiple devices.

A key use case for VUEG is allowing a phone to utilize helper devices like a watch. Acting as the "main device," the phone can offload certain background measurement tasks (e.g., periodically searching for neighbor cells) to the watch. This approach offers two clear benefits:

- It prevents measurement tasks from interfering with the phone's primary functions (like video streaming or calls), ensuring smoother communication.

- It assigns tasks to more power-efficient devices, thereby extending the total battery life of the entire device set (phone + watch).

Defining What Can Be Offloaded

To make this work, we must first clearly define which measurement tasks are suitable for offloading. This requires a focused study to answer:

- Should offloading focus mainly on mobility-related measurements (like finding and evaluating neighbor cells)?

- Or should it also include data throughput-related measurements (like MIMO measurements for the serving cell to improve data speed)?

Working Together with the Unified Framework

The Virtual UE Group paradigm represents a breakthrough in 6G device collaboration - but only if the primary UE's measurement architecture can think and act as one system.

Today's fragmented approach forces VUEG to negotiate separately with each measurement task, creating O(n²) complexity that defeats the purpose of offloading. The UE cannot efficiently delegate what it cannot coherently manage internally.

Our Unified Measurement Kernel solves this by transforming the phone into a single, intelligent measurement system before extending to external devices. The kernel serves as the central command that:

- Harmonizes all internal measurement demands

- Resolves resource conflicts and eliminates redundancies

- Presents one clean interface to VUEG helper devices

This reduces coordination complexity from O(n²) to O(n), turning measurement offloading from a theoretical possibility into a practical system-level optimization.

The figure below shows how a Unified Measurement Kernel consolidates all UE-internal measurement tasks and exposes a single clean API to helper devices under the Virtual UE Group:

Figure 2-7: Internal unification enables scalable external cooperation

The primary UE first consolidates all measurement tasks through the Unified Measurement Kernel, eliminating internal conflicts and redundancies.
Only after internal coherence is achieved does the UE expose a single VUEG API, enabling efficient measurement offloading to helper devices and achieving lower latency, reduced power, and O(n) coordination.

The essential insight: VUEG and the Unified Framework are not separate initiatives - they are mutually dependent. Unified internal architecture enables efficient external expansion. First unify internally, then extend externally.

Efficient measurement offloading requires the UE to think as one system before collaborating with many devices.

Together, the Unified Kernel + VUEG create a compound advantage: lower latency, reduced power consumption, and truly scalable multi-device cooperation for 6G.

Proposal 2-6: RAN4 to study multi-device collaboration under the 6G Virtual UE Group (VUEG) concept. The initial focus should be to identify which measurements can be offloaded to helper devices to gain system-level benefits in power and latency.

Proposal 2-7: To achieve efficient multi-device cooperation within a VUEG, internal UE measurement unification and external collaboration need to be treated as two inseparable parts to maximize total system efficiency across a larger measurement ecosystem.

      - 2.3 Measurement Capability Design

Recent discussions have proposed studying UE capability related to the "searcher" concept. However, our analysis reveals that the "searcher" is specifically defined for L3 mobility measurement in Carrier Aggregation (CA) scenarios, while L1 mobility measurement (LTM) for CA in FR1 employs a different "FFT" concept instead. This represents a fundamental architectural misalignment between L1 and L3 measurement designs for FR1.

Observation 2-8: A fundamental conflict exists between L1 and L3 CA measurement mechanisms for FR1, where measurements inside and outside the SMTC window follow different models ("FFT" vs. "Searcher") and handle critical constraints like RTD > CP inconsistently.

This discrepancy leads to inconsistent measurement behaviors. Specifically, for L3 measurements, there is no differentiation for the RTD > CP scenario, whereas for L1 measurements, the neighbor cell measurement time is scaled by the number of layers when RTD > CP. This indicates divergent underlying assumptions for the RTD constraint between measurements conducted inside and outside the SMTC window.

Observation 2-9: The RTD constraint is handled inconsistently between L1 and L3 CA measurements for FR1, leading to different measurement time calculations and performance expectations depending on the measurement occasion (inside or outside SMTC).

Introducing a standalone capability parameter like "searcher number" without resolving these foundational discrepancies would perpetuate and potentially exacerbate the existing architectural fragmentation observed in 5G.

Observation 2-10: Defining new isolated, feature-specific capability parameters (e.g., "searcher number") without first addressing the underlying architectural misalignment between L1 and L3 measurements will further solidify the fragmented capability model, contrary to the needed evolution towards a unified architecture for 6G.

Proposal 2-8: It is proposed to prioritize the study on aligning the FR1 CA measurement mechanism inside and outside the SMTC window before standardizing any isolated capability parameters like "searcher number."

Proposal 2-9: It is further proposed to focus the capability discussion on establishing a Unified UE Capability architecture for 6G, rather than introducing new feature-specific parameters.

      - 2.4 Measurement Delay Reduction

Figure 2-8: Optimizing Measurement Delay: From Local Patches to Systemic Optimization

Current efforts to reduce measurement delay predominantly focus on optimizing individual tasks in isolation. This localized approach fails to address the root cause of delay—systemic issues such as resource conflicts and redundant operations across the multi-task measurement environment. Achieving meaningful and sustainable reduction requires a fundamental shift from optimizing individual "processes" to redesigning the entire "operating system".

Core Proposal: From Isolated Process Optimization to a Unified System Kernel

We propose the development of a Unified Measurement Kernel to tackle delay at its systemic source. This kernel acts as the core scheduler, managing all measurement tasks based on a holistic view of system resources, and is structured around three key principles:

- Eliminate Redundancy: The kernel identifies and consolidates repetitive measurement operations across tasks, much like an OS eliminates duplicate background processes, to reduce unnecessary resource consumption.

- Resolve Conflicts: It introduces a centralized scheduling authority to manage competing demands for time, frequency, and spatial resources, preventing deadlocks and priority inversions among concurrent tasks.

- Enable Intelligent Scheduling: The kernel implements multi-dimensional scheduling strategies, dynamically allocating limited measurement resources (CPU, beams, memory) to the most critical operations, ensuring high-priority tasks are served first.

In essence, the goal is not to make each individual "application" run faster, but to build a smarter, more efficient measurement "operating system" that eliminates systemic bottlenecks through unified resource management.

Proposal 2-10: It is proposed to study a unified measurement framework that eliminates measurement delay through system-wide resource optimization, moving beyond isolated per-task optimization.

- Unified Measurement gap

In the last RAN4 meeting, measurement gap was agreed to be one of study and research orientations in RAN4 6GR SI. Therefore, we further provided our views on the measurement gap design were focus on the following perspectives at least:

- Measurement definitions by MG

- Measurement gap pattern

- How to configure the measurement gap

- When the measurement gap shall be applied or not

      - 3.1 Measurement types definitions depending on MG

In NR [TS38.133], RRM measurement requirements are categorized by intra-frequency, inter-frequency and inter-RAT measurement individually. Initially in LTE, the motivation to introduce the intra-frequency and inter-frequency measurement in RAN4 [TS36.133] was to differentiate the measurement reporting delay requirements because UE measurements cycle can be different because of MG applied or not simplify.

However, the such difference was vanished in NR. For an example in the figure below, the measurement SSB can be located within or out the active BWP, the intra-frequency measurement can be performed within the measurement gap. Similarly, for the inter-frequency measurements, it is possible to be performed without the measurement gap. Thus, from the requirements themselves, the difference between intra-frequency and inter-frequency measurement from the requirements perspective [TS38.133] is a little bit vague.

Figure 3-1. NR intra-frequency measurement with gap

Observation 3-: In NR the measurement requirements categorized by the intra/inter-frequency is not a precise way by which UE’s measurement behaviours and requirements can be defined distinctly.

On the other hand, for 6GR RAN4 requirement, we believe both within and out-of gap measurements are still needed indeed. The UE measurement behaviour can be essentially dependent with the gap since the available measurement occasion can be determined by the measurement period and reference signal jointly.

Figure 3-2. Gap based and gap less measurements

Observation 3-: For 6GR UE, both the gap-based and gapless measurements are needed.

Therefore, based on the observations above we can propose that:

Proposal 3-1: In 6GRR, the measurement requirements can be differentiated by gap-based and gapless more clearly.

      - 3.2 Measurement gap pattern

In current TS38.133 the multiple measurement gap patterns were specified. Practically in 5G the implemented measurement gap pattern was only limited a few mandatory gap patterns due to UE implementation. For an example, in the current RAN4 test cases in TS38.133 A.6 there are a few test cases with gp#0, gp#1, gp#3 used.

Observation 3-: More than 26 measurement patterns are defined. But there are only a few of them were implemented in 5G.

On the other hand, the too many unnecessary gap patterns will lead the higher UE implementation complexity. For an example, supporting the shorter MGRP need UE perform the measurements more frequently and the short MGL needs UE to process the measurement faster. Additionally, the testing efforts due to the more gap patterns will be increased significantly. From RAN4 standardization effort ourselves the more gap patterns shall be manageable also.

Particularly we see the main extra burden comes from implementation and especially testing of the additional mandatory gap patterns.

Observation 3-: The burden because of the more gap patterns comes from UE implementing, testing efforts and especially RAN4 standardization work loading itself.

This means that with 5G lessons, it would be undesirable to define too many gap patterns which are not in practice used by any 6GR deployment.

Proposal 3-2: RAN4 shall focus on the mandatory measurement gap pattern(s) in 6GR to minimize the gap pattern number.

For the measurement gap, the other important parameters discussed in NR is the gap pattern applicability as given in the table below. In short, there are both per-UE and per-FR MGs defined as in the table below.

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE [TS38.133]

| Measurement gap pattern configuration | Serving cell | Measurement PurposeNote 5 | Applicable Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | E-UTRA + FR1, or | non-NR RAT Note1,2 | 0,1,2,3 |
| Measurement gap | E-UTRA + FR2, or E-UTRA + FR1 + FR2 | FR1 and/or FR2 Note 6,7 | 0-11, 24, 25 |
|  |  | non-NR RATNote1,2 and FR1 and/or FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10, 24 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 | 0,1,2,3 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 only Note 6,7 | 0-11,24,25 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR2 only Note 6,7 | No gap |
| Per-FR | FR2 if configured |  | 12-25 |
| measurement gap | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR1 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | No gap |
|  | E-UTRA and, FR1 if configured | FR1 and FR2 Note 6,7 | 0-11,24,25 |
|  | FR2 if configured |  | 12-25 |
|  | E-UTRA and, FR1 if configured | non-NR RAT Note1,2 and FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |
|  | E-UTRA and, FR1 if configured | Non-NR RAT Note1,2 and FR1 and FR2 Note 6,7 | 0, 1, 2, 3, 4, 6, 7, 8,10,24 |
|  | FR2 if configured |  | 12-25 |

For the per-UE measurement gap, there is not any data/signals transmitted from the serving gNB to UE and UE is not required to receive any data/signals from the serving cell within the measurement gap duration on all frequency carriers/bands. In 5G, the per-FR measurement gap was introduced to reduce the system impacts on the other FRs. For an example, if the per-FR measurement gap was configured instead of per-UE, UE may transmit/receive the data in the other FR.

In 6GR，since UE needs to be operated in the more multiple bands [1], in order to achieve more efficient UE implementation the RF front-end of UE for the range of bands can be integrated together. That is the “band-group” operation can be supported by UE. For an example, the band group of sub3G can include the “LB+MHB+LMHB+MHB”. This direction can be potentially simplify CA in 6GR as well.

Observation 3-: if the measurement gap can be specified by per band-group, UE’s reception and transmission on the other band-group can be allo1wed

Proposal 3-3: the other applicability beside per-UE and per-FR can be studied upon RAN4 RF study on the CA simplified operation.

      - 3.3 Measurement gap configuration mechanism

In 5G NR Release 17 one of key enhancement for measurement gaps is to introduce the pre-configured MG by which the measurement gap can be inactivated when being configured. Technically speaking, in comparison with this pre-configured MG the legacy MG (Type 1 and 2) has not any advantages.

Observation 3-：The pre-configured MG can be more efficient to reduce the measurement delay in comparison with the legacy Type 1& Type 2 MG without any signaling or throughput degradation.

However, in order to keep the backward compatibility in NR, both the legacy tyep1 and type 2 MG and pre-configured MG are supported. This also increase the specification complexity. For an example, the different UE behavior under the MG configuration needed to be specified in RAN4.

Observation 3-: It is redundant to define the two types of measurement gap configuration in RAN4. the unified MG configuration mechanism is needed.

There are two ways to activate the MG in NR also. One is based on the network signaling the other UE autonomously activation. However, there is little benefits with the multiple methods to activate the pre-configured MG.

Observation 3-8: For the pre-configured MG itself, the multiple activation MG methods are unnecessary.

Therefore, we can propose:

Proposal 3-4: RAN4 can consider to unify the measurement gap configuration and activation mechanism in 6GR.

      - 3.4 Gap efficiency optimization in 6GR

Spectrum efficiency improvement has been highlighted as key considerations of 6GR study in the last RANP 6GR workshop. Similarly, from RAN4 RRM study perspective, the enhanced measurement gap design in 6GR needs to minimize the impacts to system spectrum efficiency because the network scheduling data and reception within the measurement gap is not allowed. In other words, in 6GR UE needs to minimize the interruption on the network scheduling as possible.

Observation 3-9: the measurement gap enhancement to reduce the interruption time to the network data scheduling can help to improve the overall system spectrum efficiency.

- NCSG

In 5G, several useful methods were introduced to improve the measurement gap ultization. For an example, with NCSG, UE can perform the measurement on the other frequency carriers via the other vacant RF chain. Therefore, there is no any interruption on PCell within the measurement window but a small one when at the begin and end of this measurement window.

Figure 3-3. NCSG in NR

Observation 3-10: The existing measurement gap design in NR (e.g. NCSG) can avoid the too large impacts on the system spectrum efficiency.

Therefore, we can propose that:

Proposal 3-5: RAN4 can leverage NCSG design in NR as one candidate solutions to improve the measurement gap efficiency in 6GR.

- Gap-less measurement

Beyond NR, both NW and UE behavior within the measurement gap are restricted. For an example, NW can’t schedule the data to UE continuously. That is the transmission to UE was interrupted every measurement gap periodicity. Consequently, the overall system throughout will be artificially degraded because the measurement within gap. For an example, for gap pattern #0, there are about as high as 6/40 system throughput lose in case of there are always measurement with gap needed. Even for NSCG #0, there are also about 2/20.

Observation 3-11: The gapless measurements can bring gains for network system throughput significantly.

Additionally, UE’s measurement delay was also too long because UE can be only allowed to perform in the specific measurement occasion. For an example, according to current TS38.133 UE could not measure the reference signal which is not contained within the measurement gap occasion. Consequentially, UE could only perform the measurement when the reference signal are fully overlapped with measurement gap as illustrated in Figure 3-4 below. In comparison with the measurement without gap, the overall measurement latency was obviously increased.

Figure 3-4. Measurement latency

Observation 3-11: The gapless measurements can also bring gains for mobility latency performance.

Based on the observations above, we suggest that:

Proposal 3-6: For 6GR, in order to reduce UE power consumption and improve the network throughput, the measurement without the gap can be further studied.

In NR RAN4 spec, whether UE needs to follow the requirements of the measurement without gap can be up to multiple specific conditions. For an example, for the intra-frequency measurement, the following conditions are defined.

Observation 3-12: UE can support the measurement without gap when:

- the UE's active bandwidth part (BWP) contains the measurement occasion bandwidth of neighbor cells .

- the UE supports simultaneous reception across the required frequencies .

From UE itself when performing the gapless measurement

Proposal 3-7: RAN4 can firstly study the capability and the conditions of measurement without gap to avoid the ambiguous UE behavior.

      - 3.5 Assumption on the UE implementation architecture

In principle, when RAN4 considering on when the measurement gap can be applied during UE’s measurement procedure, the fundamental factors shall be considered is how many available RF chain and BB processing units are available. For an example, in NR if there is vacant RF chain for CA operation, UE can measure the other carriers without measurement gap but NCSG.

And the assumption on the UE implementation baseline for the measurement is critical to design on the gap. For an example, when considering the minimum gap length the following factors needs to be evaluated.

- RF Switching delay : For sub 6GRHz bands, we expect that 0.5ms from LTE may be evaluated as a starting point

- Vacant RF chain for the parallel measurements and data reception

Moreover, for other important RRM requirements (e.g. the interruption or scheduling restriction) the different requirement can be applied for the different UE implementation architecture. For an instance, in NR the measurement without gap/interruption are highly rely on whether there are other RF chains beside the one for serving cell data reception.

Observation 3-13: the UE architecture including RF and baseband receivers can significantly impact RAN4 RRM requirements especially for measurement gap applicability and interruption.

Therefore, we can suggest that:

Proposal 3-8: Before the more concreted discussions on measurement gap and interruption requirements, RAN4 shall align on the baseline UE architecture.

For an example, in LTE the UE’s architecture was illustrated as below, which was taken as the baseline assumption for the corresponding analysis on whether the gap is needed for CA measurements.

Figure 3-5. Possible UE Architectures in three aggregation scenarios in LTE [4,TR36.815]

On the other hand, considering the different types of devices in 6GRR (e.g. TN, NTN, MBB, FWA), in order to avoid too complicated HW implementation, the unified assumption on the UE baseline architecture is expected.

Observation 3-14: The UE architecture agonistic with use cases (e.g. NTN, MBB, FWA) is beneficial to converge RAN4’s baseline assumption.

- Others

      - 4.1 Mobility related RRM

In NR, several mobility solutions have been introduced across successive releases, including Layer 3 HO, DAPS, CHO, LTM and C-LTM. Correspondingly, RAN4 has defined the performance requirements consists of delay and interruption. Different mobility solutions together with different requirements defined in RAN4 increase the work load of specification as well as the complexity of UE/NW side.

Considering different characteristic of the several mobility solutions, the delay and corresponding interruption requirements are diverse. From requirement perspective, the following components are counted in the total delay requirements

- RRC message decoding delay

- DL sync acquisition, including coarse and fine sync

- UE processing delay of the target cell configuration

- Waiting for the first PRACH occasion/UL transmission resource

During the 6G discussion in RAN2#131bis meeting, the following agreements are achieved:

Agreements in RAN2#131bis

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

We understand the concern from both UE and NW on the delay/interruption. RAN2 has conducted study on the 6G mobility design, targeting seamless, robustness and efficiency. From RAN4 perspective, we think RRM need to wait for RAN2 progress on the mobility framework design, and then study the potential delay/interruption reduction solution.

Proposal 4-1: RAN4 to study the potential delay/interruption reduction solution based on RAN2 progress on mobility framework design.

      - 4.2 UE power saving

Achieving transformative energy efficiency in 6G requires a foundational shift in measurement architecture. A Unified Measurement Framework delivers power savings through two distinct yet complementary mechanisms: intrinsic efficiency within RAN4, and extrinsic enablement for other working groups.

1. Intrinsic Efficiency: Power Saving from Unification within RAN4

The unified framework itself is a primary source of power reduction, directly optimizing RAN4's core domain by:

- Eliminating Redundancy: Consolidating duplicate measurements across tasks and protocol layers directly reduces UE processing cycles and RF component usage.

- Resolving Conflicts: A global scheduler prevents measurement collisions and retries, eliminating the associated energy waste.

- Optimizing Resources: Pooled management of capabilities (e.g., receivers, buffers) ensures hardware is used more efficiently, minimizing active measurement time.

- Extrinsic Enablement: Unlocking Cross-WG Power Saving Innovations

Beyond internal gains, the unified framework acts as the essential enforcer and multiplier for power-saving features defined by other groups, such as RAN1:

- Realizing Complex Features: It provides the system-level coordination required to implement advanced techniques (e.g., on-demand signals, wake-up receivers) without incurring overhead that would negate their theoretical benefits.

- Ensuring Holistic Operation: It enables efficient management of multi-carrier scenarios and extended measurement periods, which are impractical to coordinate across isolated, task-specific managers.

Proposal 4-2: We propose the Unified Measurement Framework as the path to 6G power efficiency, achieving it through internal RAN4 optimization and external enablement of cross-WG innovations.

      - 4.3 Spectrum aggregation related RRM

There are some potential directions for 6G spectrum aggregation that RAN4 need to be deeply involved, such as DL/UL spectrum decoupling, fragmented carrier, and CA via switching and uplink Tx switching. DL/UL spectrum decoupling shows potential benefit on both coverage enhancement and meeting high-throughput demands. Fragmented carrier and CA via switching and uplink Tx switching has been studied by RAN4 during Rel-19 timeline, with identified potential advantages for full spectrum utilization—making these directions worthy of further exploration. These three directions need RF’s involvement in the first stage on the basic architecture. RRM-related studies for spectrum aggregation may be contingent on progress in RF-related work. We understanding these potential directions would involve RRM impact, such as scheduling restriction and interruptions.

Additionally, RAN1 and RAN2 have initiated discussions on the spectrum aggregation. The below agreements are achieved in last RAN2 meeting:

Agreements on Spectrum aggregation

1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

RAN4 can prioritize foundational studies on DL/UL spectrum decoupling, fragmented carrier, and CA via switching and uplink Tx switching. For potential enhancement schemes under consideration by RAN1/2, RAN4 can await their progress.

Proposal 4-3: RAN4 to conduct RRM impact studies for spectrum aggregation, contingent on RF progress and/or the development of potential enhanced schemes by RAN1/2.

      - 4.4 NTN related RRM

In 5G, RRM requirements have been defined for TN and NTN in different clauses. However, from our perspective, most RRM requirements defined for NTN followed the same principle from TN. Taken the measurement principle from TN as baseline, the RRM requirements further consider the NTN specific design, including:

- SMTC adjustment in connected mode and idle mode

- Time/Location-based mobility

- Satellite switch with resync

- RACH-less HO

In last meeting, RAN2 achieved the following agreement for NTN:

For next meeting companies should identify essential requirements for NTN and identify which functionality should be considered in the common design with TN and what are NTN specific and whether they are essential.

Before RAN1 and RAN2’s further progress, we think RRM can agree on the principle in higher level that RRM requirement for NTN to follow the common framework achieved in TN RRM discussion to the maximum extent, except the NTN specific design.

Proposal 4-4: RAN4 to study the unified RRM requirements to support the harmonized TN and NTN.

- Conclusion

In this contribution, we provide the following proposals:

Unified Measurement framework:

Proposal 2-1: RAN4 shall recognize that TS 38.133, together with referenced UE capabilities, constitutes the specification of the UE's "Multi-Task Measurement OS Kernel" — a system responsible for platform capability abstraction, resource scheduling, and conflict arbitration.

Proposal 2-2: RAN4 shall adopt the system-architect role for 6G, with the explicit mandate to bridge the gap between task-independent specification and systemic UE execution. This establishes RAN4 as the core architect of the UE's measurement infrastructure.

Proposal 2-3: RAN4 shall apply an architecture-first principle. A unified framework must be defined first, into which all new and existing measurement tasks are integrated as compliant components, ensuring consistent and scalable system behavior.

Proposal 2-4: RAN4 shall investigate a unified, resource-centric measurement framework for 6G based on a three-layer architecture, which decouples RAN4’s work from other groups:

- Unified Task Interface (RAN1/RAN2): Provides the stable abstraction for what to measure, enabling other WGs' ongoing work to unify feature configurations e.g. Unified ResourceConfig, Unified ReportConfig, while ensuring system compatibility.

- Unified Measurement Kernel (RAN4 core): Focuses on how to measure efficiently. It acts as a resource-centric optimizer, performing global scheduling, conflict resolution, and measurement reuse across spatial, temporal, and spectral resources to maximize system performance and power efficiency.

- Unified UE Capabilities (RAN4 core): Abstracts hardware/software capability(compute, memory, FFT, searcher, etc.), creating the foundation for kernel optimization.

Proposal 2-5: RAN4 shall proactively coordinate with other WGs. When task-specific parameters conflict with system-level scheduling policy, RAN4 will lead the resolution to preserve both feature intent and overall system efficiency.

Virtual UE group

Proposal 2-6: RAN4 to study multi-device collaboration under the 6G Virtual UE Group (VUEG) concept. The initial focus should be to identify which measurements can be offloaded to helper devices to gain system-level benefits in power and latency.

Proposal 2-7: To achieve efficient multi-device cooperation within a VUEG, internal UE measurement unification and external collaboration need to be treated as two inseparable parts to maximize total system efficiency across a larger measurement ecosystem.

Measurement Capability

Proposal 2-8: It is proposed to prioritize the study on aligning the FR1 CA measurement mechanism inside and outside the SMTC window before standardizing any isolated capability parameters like "searcher number."

Proposal 2-9: It is further proposed to focus the capability discussion on establishing a Unified UE Capability architecture for 6G, rather than introducing new feature-specific parameters.

Measurement Delay

Proposal 2-10: It is proposed to study a unified measurement framework that eliminates measurement delay through system-wide resource optimization, moving beyond isolated per-task optimization.

Measurement GAP

Proposal 3-1: In 6GRR, the measurement requirements can be differentiated by gap-based and gapless more clearly.

Proposal 3-2: RAN4 shall focus on the mandatory measurement gap pattern(s) in 6GR to minimize the gap pattern number.

Proposal 3-3: the other applicability beside per-UE and per-FR can be studied upon RAN4 RF study on the CA simplified operation.

Proposal 3-4: RAN4 can consider to unify the measurement gap configuration and activation mechanism in 6GR.

Proposal 3-5: RAN4 can leverage NCSG design in NR as one candidate solutions to improve the measurement gap efficiency in 6GR.

Proposal 3-6: For 6GR, in order to reduce UE power consumption and improve the network throughput, the measurement without the gap can be further studied.

Proposal 3-7: RAN4 can firstly study the capability and the conditions of measurement without gap to avoid the ambiguous UE behavior.

Proposal 3-8: Before the more concreted discussions on measurement gap and interruption requirements, RAN4 shall align on the baseline UE architecture.

Mobility

Proposal 4-1: RAN4 to study the potential delay/interruption reduction solution based on RAN2 progress on mobility framework design.

Power saving

Proposal 4-2: We propose the Unified Measurement Framework as the path to 6G power efficiency, achieving it through internal RAN4 optimization and external enablement of cross-WG innovations.

Spectrum aggregation related RRM

Proposal 4-3: RAN4 to conduct RRM impact studies for spectrum aggregation, contingent on RF progress and/or the development of potential enhanced schemes by RAN1/2.

NTN related RRM

Proposal 4-4: RAN4 to study the unified RRM requirements to support the harmonized TN and NTN.

# 6 Reference

[1] R4-2513274, Overview for 6G spectrum, Xiaomi