# 3GPP TSG-RAN WG4 Meeting #117	R4-2520503

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.2.5.1

Source: 	Xiaomi

Title: 	View on 6GR channel bandwidth

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In this contribution, our preliminary view are provided on CHBW related issues.

- Discussion

  - Overview

Following design principle was considered in NR on system parameter design:

- 4K FFT size with maximum 3300 sub-carriers as baseline

- Configurable numerologies and multiple CHBW sets as per numerology basis introduced in NR

- SSB design and sync raster is pending on Minimum CHBW assumption

- Maximum CHBW is pending on SCS, FFT size, UE processing capacity, and available contiguous spectrum

- NR majorly focus on eMBB UE in day 1 design

Pain points from 5G:

- Flexible numerologies and multiple CHBW sets as per band basis increase gNB, UE design and conformance test complexity

- 3MHz CHBW was introduced in later stage with PBCH design refinement (NBC issue)

- Diverse device types were introduced in late stage with 1/2 Rx exception (XR, Redcap) and RAN4 requirements device dependency (FWA, CPE, Handheld, non-handheld)

Observation 1:  following pain points observed from 5G:

- Flexible numerologies and multiple CHBW sets as per band basis increase gNB, UE design and conformance test complexity

- 3MHz CHBW was introduced in later stage with PBCH design refinement (NBC issue)

Proposal 1: RAN4 initial study on system parameters focus on following direction:

- Unify candidate numerologies (data and SSB) as sub-frequency range basis

- Decide minimum CHBW and maximum CHBW compatible with diverse device types

  - CHBW/FFT size/SCS

2.2.1		Numerology

According to 6G SID, minimum channel bandwidth and maximum channel bandwidth and numerologies required joint effort from both RAN1 and RAN4. Also, from 6G SID, 6G aims to harmonize numerology per sub-frequency range.

6G SID - Physical Layer structure for 6GR, - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

Based on initial discussion in RAN1#122 and further confirmed in RAN#109 with following agreements: at least 15kHz is supported for FDD and 30kHz SCS is supported for TDD for below 6GHz bands and 30kHz/60KHz as candidate for around 7GHz. And in RAN1#122bis, RAN1 further confirmed around 7GHz at least 30kHz supported and same SCS for sync channel and other channels (except PRACH) as per band basis (FFS for FR2-1).

Observation 2: According to 6G SID, 6GR targets to avoid multiple numerologies for the same band/sub-range.

Observation 3: According to RAN1 agreements, 6GR supports at least following SCS for data/control channel except PRACH

- FR1 FDD: 15kHz SCS

- FR1 TDD: 30kHz SCS

- ~7GHz: 30kHz SCS

Observation 4: According to RAN1 agreements, same SCS applied for both sync channel and other data/control channels in the same band.

One of major pain point in NR, NR introduced flexible numerologies configuration and almost each bands support all the numerologies i.e., 15kHz, 30kHz and 60kHz with corresponding channel bandwidth sets in FR1.  However, from real deployment, for each band only single numerology was deployed. These flexible numerologies with multiple options in each bands bring unnecessary complexity to both gNB and UE without any benefits/usage on commercialization.

Another issue, in NR mixed numerologies between SSB and data channel were supported in the same carrier and this bring UE implementation complexity and pending on UE capability, UE may not support mixed numerologies which will bring interruption in data channel reception when performing intra-cell SSB measurement/detection.

In existing NR deployed spectrum/bands, it’s better to align the numerology between 5G and 6GR to simplify the scheduling complexity considering MRSS.

The choice of numerologies selection needs to consider several factors including phase noise robust, maximum CHBW, Cell size/CP length.  On around 7GHz, based on previous release evaluation, we didn’t see any issue on phase noise impact with 30kHz SCS.  Another point, we aim to achieve same coverage as C bands (3.5 GHz) for around 7GHz as 6GR design target.

6G SID The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

With above consideration, it’s better to align numerology between around 7GHz range and C-bands. With 8K FFT operation, single carrier can achieve 200MHz CHBW for 30kHz which also meet the demand on wider channel bandwidth.

Proposal 2: Limit to single numerology as per sub-frequency range/per band basis at least for DL/UL control channel and data channel except PRACH

Proposal 3: Same numerology is applied for SSB (initial cell access) and DL channel in same band

Proposal 4: On 5G migration spectrum/bands, harmonize the numerology between 5G and 6G by taking existing NR commercialization deployment choice into account.

Proposal 5: Following numerology choice is proposed on different sub-frequency ranges as default SCS

| Frequency range | SCS for data/control channel except PRACH | SCS for PBCH (initial cell access) |
| --- | --- | --- |
| Below 3GHz (FDD bands) | 15kHz | 15kHz |
| Below 3GHz (TDD bands) | 30kHz | 30kHz |
| 3GHz ~ 7.125GHz | 30kHz | 30kHz |
| 7.125GHz ~8.4GHz | 30kHz | 30kHz |
| 24.25 GHz -52GHz | 120kHz | 120kHz |

2.2.2		Minimum CHBW

Minimum CHBW

The detailed CHBW sets per numerology, corresponding spectrum utilization per combination of {CHBW, SCS} and supported CHBW sets per band belong to RAN4 scope.

Minimum CHBW is pending on following factors:

- “Minimum available spectrum from 6GR deployment” (RAN4 scope)

- Overall system performance impact to ensure comparable performance as NR (RAN1 scope on common channel design)

- Lowest device capability that 6GR design can be applied with smallest maximum CHBW (RAN1/RAN4 joint effort)

Proposal 6: Regarding minimum CHBW, two aspects need to be considered in RAN4:

- “Minimum available spectrum from 6GR deployment” (RAN4 scope)

- Lowest device capability that 6GR design can be applied with smallest maximum CHBW (RAN1/RAN4 joint effort)

From available spectrum perspective, 5MHz CHBW was specified in NR as minimum CHBW across all the bands. And in later stage, RAN4 also identified 3MHz for special RMR system operation, and in some specific bands operators may hold 3MHz spectrum under below 1GHz bands for example band n28, n85, n26. On some NTN deployment scenario, operators may also hold relatively small block contiguous spectrum.

On smallest maximum CHBW for low tier device this is pending on use case (target peak data rate) and UE complexity/cost for low device that 6GR common system design can support. Referred to NR Redcap and eRedcap discussion, 5MHz ~20MHz can be further discussed. The channel bandwidth at least shall be larger than minimum CHBW from system perspective. Also pending on the supported bands on low tier device, if UE needs to support TDD bands in FR1 we assume at least 10MHz required.  Another point, with small channel bandwidth, the SU is relatively low with high numerology due to RB size granularity limitation.

Observation 5: The spectrum utilization under minimum CHBW is also pending on numerology, SU is relatively low with high numerology due to RB size granularity limitation e.g., 5MHz with 30kHz SCS.

Observation 6: The BW size of common channel i.e., sync channel/coreset channel is also pending on numerology.

Proposal 7: Minimum CHBW is pending on numerology, supported minimum CHBW as following as per numerology basis:

- 15kHz: 5MHz in general, 3MHz applicable for particular bands below 1GHz

- 30kHz: 10MHz

- 120kHz: 50MHz

Proposal 8: On smallest maximum CHBW for lower device type in 6GR, further discuss 5MHz ~20MHz as maximum CHBW.

2.2.2		Maximum CHBW/FFT size

Maximum CHBW/FFT size

Regarding maximum CHBW, RAN4 needs to jointly consider FFT size, numerology and avalibility of spectrum . In terms, maximum CHBW from system perspective, we can assume 8K FFT as baseline for 6G. Also, the potential spectrum that operators on 6GR deployment needs to be considered. Meanwhile we also need to clarify the maximum BW assumption from RAN1 perspective can be different with RAN4 Channel bandwidth in gNB and UE side.

Observation 7: The consideration of minimum CHBW and maximum CHBW in RAN1/RAN4 is different

- From RAN1 perspective, the minimum BW/RB and maximum BW/RB was majorly for whole 6GR system design which needs to ensure forward capability.

- The CHBW sets including maximum CHBW in RAN4 majorly considered the spectrum availability, RF/BB feasibility and implementation restriction which shall be within the range of 6GR RAN1 system design on minimum system BW ~ Maximum system BW

Regarding the spectrum availability in refarming spectrum from 4G and 5G, maximum available contiguous spectrum is not exceeding 200MHz BW e.g., 2.6GHz band n41 (160MHz), 3.5GHz band n77/78 (200MHz maximum with spectrum sharing between operators), and 4.5GHz (100MHz as maximum).

Observation 8: The maximum contiguous BW on refarming spectrum is not exceeding 200MHz.

Observation 9: The regulation on ~7GHz is still under discussion in WRC-27, the situation of spectrum allocation is still unclear.

Another point, in NR all the channel bandwidths in gNB side are optional with declaration basis and the channel bandwidth sets in UE side is mandatory which bring inconsistent and unnecessary complexity in UE side. For NR in FR2, 200MHz CHBW was agreed as mandatory for UE in day1, however typical network deployment only schedule CHBW as 100MHz per carrier with CA to support over 200MHz BW in real field.

Observation 10: The handling of CHBW in gNB side and UE side is different:

- All Channel bandwidth sets in gNB side are optional with declaration basis

- RAN4 agreed mandatory channel bandwidth sets for UE in day 1

- In NR real field deployment, commonly used maximum BW of single carrier in FR1 and FR2 is 100MHz even 200MHz CHBW was mandatory for FR2 UE in day 1

- NR already support asymmetric CHBW in gNB and UE side

Supporting 400MHz with 30kHz SCS single carrier will enforce UE to support 16K FFT which bring lots of effort from both RF front-end, RF-IC and BB side.

Regarding single carrier 400MHz and 2x200 CA, we see the advantage of using CA can ease the difficulty from implementation perspective e.g., CA allow UE to implement 400MHz BW with separate Tx/Rx chain, reduced sampling rate, PA linearity, dynamic range of each carrier. CA approach also allow schedule flexibility to schedule asymmetric BW in DL and UL side which also save power with carrier activated or deactivated.

Observation 11: Overall observations on RF/BB implementation impact and restriction on supporting 200MHz VS 400MHz, and 400MHz (1cc) vs 400MHz (2 cc) summarized in below table:

|  | 200MHz vs 400MHz | 400MHz 1CC VS 200MHz X2 CCs |
| --- | --- | --- |
| RF Front-end (PA/LNA, RF filter, Antenna) | - 400MHz will increase PAPR and bring linearity issue especially for PA. - Typically, effective BW ratio for PA linearity is 3% (BW/Centre frequency) as upper limit due to memory effect under wide BW. On ~7GHz, the upper limit is 200MHz. - Larger BW required more MPR/PAPR, which bring challenge to support high power class which is critical for 7GHz to achieve same coverage as 3.5GHz - In order to support 400MHz, operating point of PA needs to be adjusted which reduce power efficiency on other BW. | - Spectrum aggregation approach allow flexibility of UE to support 400MHz with separate Tx/Rx chains on each CC. - Ease implementation complexity and constraints on PA |
| RF-IC (Mixer, AD/DA, Analogue filter) | 400MHz required large dynamic range and sampling rate which bring challenge on mixer, AD/DA. |  |
| BB (FFT, processing complexity) | 16K FFT required for 400MHz with 30kHz SCS which bring processing complexity and power consumption. | - CA approach allow UE to implement separate FFT on each CC; reduce processing complicity - Separate digital filtering can be applied on each |
| Tx RF performance (Emission, ACLR) | ACLR performance degraded with 400MHz BW compared to 200MHz | No much difference, CA approach allow UE to implement separate filtering on each CC to improve Tx and Rx performance |
| Rx RF performance | Receiver blocking performance degraded with 400MHz |  |
| Other aspects |  | - CA approach provide scheduling flexibility and power saving benefits with carrier activated/deactivated - CA approach provide possibility to support asymmetric BW in DL and UL side from UE perspective. |

Proposal 9: Take 8K FFT as baseline assumption

Proposal 10: Considering 200MHz CHBW as maximum CHBW for 30kHz SCS in initial stage for RAN4 CHBW set introduction

Proposal 11: 400MHz BW supporting can be further discussed once the spectrum availability more clear

- CA or other spectrum aggregation schemes can be considered in later stage to support 400MHz BW

Proposal 12: Specify the minimum CHBW and maximum CHBW based on numerologies and operating frequency

| SCS | Min CHBW | Max CHBW |
| --- | --- | --- |
| 15kHz | 5MHz 3MHz (below 1GHz bands) | 100MHz |
| 30kHz | 10MHz | 200MHz |
| 120kHz | 50MHz | 800MHz |

Asymetric channel bandwidth

In NR, asymetric channel bandwidth in DL and UL was supported in specific bands with detailed bandwidth combinations. Supporting asymetric  channel bandwidth have impact on UE RF requirements e.g. REFSENS and reference architecture e.g. Duplex design for FDD, and LO position for TDD.

RAN4 shall focus on the minimum and maximum CHBW and CHBW set first and then to further study possible enhancement on asymentric channel bandwidth supporting which also pending on operators’ request.

Proposal 13: Further study asymetric channel bandwidth supporting and potentail  enhancement compared to NR after minimum/maximum CHBW and detailed channel bandwidth set fixed.

  - Spectrum utilization

Regarding spectrum utilization, NR specified transmission bandwidth configuration and guard-band for each BW to decide SU as per {CHBW, SCS} basis.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

The spectrum utilization ratio on existing NR {CHBW, SCS} are summarized in below tables:

Table X: NR FR1 SU per {CHBW, SCS}

Table X: NR FR2-1 SU per {CHBW, SCS}

We can observe that the SU specified in NR already high than 90%, especially for large channel bandwidth SU is close to 95%~98% in FR1 and ~95% for FR2-1. There is no much room to further optimize SU under large CHBW. Also, pending on numerologies, the SU is relatively low for small channel bandwidth especially under high numerology e.g., 30kHz SCS. This is due to RB size granularity limitation. One possible approach, we shall limit minimum CHBW as per numerology basis to avoid low spectrum efficiency.

Proposal 14: For CHBW between 25MHz ~ 100MHz (15kHz/30kHz SCS) in FR, taking existing SU from NR as baseline unless strong motivation well justified for the evaluation and improvement. And prioritize following case on evaluation work

- 15kHz: 5MHz CHBW, 30kHz: 10MHz, 15MHz, 20MHz

The spectrum utilization is pending on several factors including waveform assumption, modulation orders and corresponding RF core requirements including emission requirements, Tx EVM and PA model.  There is trade-off between SU and RF performance (MPR, Tx EVM).

Proposal 15: RAN4 further evaluate spectrum utilization with trade-off between improved SU, RF performance and UE/gNB complexity with detailed parameter assumption

- RF non-linearity assumption: PA model, I/Q imbalance, PN

- TX RF core performance assumption: OBE (emission and ACLR), Tx EVM

- Waveform and modulation orders

- Spectrum shaping technologies

Given the dependency of waveform, RAN4 RF core requirements for SU evaluation, RAN4 shall focus on more critical/fundamental issues and start the evaluation work on SU in later stage.

Proposal 16: Postpone SU evaluation work to later stage of Rel-20 till waveform, CHBW sets, relevant RF core requirements concluded.

In last RAN4 meeting, some companies also proposed to consider evaluate SU gain with relaxed requirements e.g. Tx EVM relaxation, or emission requirements relaxation. In Rel-19 NR, RAN4 developed feature “reduced MPR with relaxed emission requirements”, and Rel-20 RAN4 will further study MPR reduction with EVM relaxation. However, above relaxation on emission/Tx EVM are under network control, which is not always valid. Developing multiple SU pending on applicable requirements on each CHBW, this bring both NW and UE complexity for both scheduling and implementation.  Reduced MPR vs Multiple SU on requirements relaxation, we believe reduced MPR more feasible way.

Proposal 17: RAN4 shall specify single SU as per {channel bandwidth, SCS} combination basis w/o dependency on spectrum shaping technilogies and RF requirement relaxtion conditions.

  - CHBW sets/Irregular channel bandwidth

In NR, under Rel-17/18 RAN4 established study item to study potential solutions on irregular channel bandwidths. The initial demand from operators were collected as show in table below.

Irregular BW from TR 38.844

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 6, 7, 11, 12 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

During the study, RAN4 identified several solutions with pros and cons summarized in the TR. Overlapping CA/Overlapping CHBW was identified as candidate solution to resolve this issue. During the study, RAN4 also identified major limitation on SSB and channel placement under irregular BW was channel raster granularity. 900kHz minimum distance between carriers is required with 100kHz channel raster and 180kHz RB size. This brings spectrum utilisation issue.  Later, to allow flexibility of channel placement, RAN4 agreed to introduce enhance channel raster to resolve this issue.

Another limitation was SSB placement for less than 10MHz, the common part over overlapped carrier is not wide enough to cover 3.6MHz SSB and 4.32MHz corset#0. Then two separate SSB per carrier with time multiplexing for 6MHz/7MHz is required which bring resource waste and scheduling complexity. To address this, 6MHz/7Mhz dedicated channel bandwidth was introduced in later stage.

For 6GR, CHBW is still the key parameters and RAN4 shall care exam channel bandwidth sets to control overall UE design/test complexity.

Observation 12: With enhanced channel raster adopted in Rel-18 i.e., 20kHz channel raster, NR already resolve the demand on supporting 10MHz above irregular BW by network scheduling and implementation e.g. Overlapping CA /Overlapping CHBW from network perspective.

Observation 13: The restriction of 6MHz, 7MHz BW in NR was SSB/Coreset BW exceeding overlapping region between 2 normal carriers.

Observation 14: For 6GR, it’s FFS whether dedicated 6MHz, 7MHz CHBW needed or not which is pending RAN1 design on common channel BW if 3MHz Sync/Control channel supporting then no needs to support 6MHz/7MHz CHBW.

Proposal 18: RAN4 shall careful exam channel bandwidth sets to balance UE design/test complexity and flexibility to fully usage operators’ spectrum

- RAN4 requirements and system parameter design are developed based on Channel bandwidths

- Granularity of CHBW sets need to be carefully considered e.g.  separate step-size in different BW range

Table x: Exemplary step size for regular CHBW sets

| CHBW range | Step size |
| --- | --- |
| 5MHz-50MHz | 5MHz |
| 50MHz -100MHz | 10MHz |
| 100MHz-200MHz | 50MHz/20MHz |

Proposal 19: RAN4 needs to collect operators’ request on irregular BW request first.

- Taking existing collected irregular BW request from TR 38.844 as starting point including 6MHz, 7MHz, 11MHz, 12MHz and 13MHz

Proposal 20: Study potential solution to support irregular spectrum with following direction:

- Overlapping CA /Overlapping CHBW from network perspective

- Channel raster/sync raster and channel mapping rule design to be compatible with flexible BW

- Conclusion

Overview:

Observation 1:  following pain points observed from 5G:

- Flexible numerologies and multiple CHBW sets as per band basis increase gNB, UE design and conformance test complexity

- 3MHz CHBW was introduced in later stage with PBCH design refinement (NBC issue)

Proposal 1: RAN4 initial study on system parameters focus on following direction:

- Unify candidate numerologies (data and SSB) as sub-frequency range basis

- Decide minimum CHBW and maximum CHBW compatible with diverse device types

Numerology

Observation 2: According to 6G SID, 6GR targets to avoid multiple numerologies for the same band/sub-range.

Observation 3: According to RAN1 agreements, 6GR supports at least following SCS for data/control channel except PRACH

- FR1 FDD: 15kHz SCS

- FR1 TDD: 30kHz SCS

- ~7GHz: 30kHz SCS

Observation 4: According to RAN1 agreements, same SCS applied for both sync channel and other data/control channels in the same band.

Minimum CHBW

Proposal 6: Regarding minimum CHBW, two aspects need to be considered in RAN4:

- “Minimum available spectrum from 6GR deployment” (RAN4 scope)

- Lowest device capability that 6GR design can be applied with smallest maximum CHBW (RAN1/RAN4 joint effort)

Observation 5: The spectrum utilization under minimum CHBW is also pending on numerology, SU is relatively low with high numerology due to RB size granularity limitation e.g., 5MHz with 30kHz SCS.

Observation 6: The BW size of common channel i.e., sync channel/coreset channel is also pending on numerology.

Proposal 7: Minimum CHBW is pending on numerology, supported minimum CHBW as following as per numerology basis:

- 15kHz: 5MHz in general, 3MHz applicable for particular bands below 1GHz

- 30kHz: 10MHz

- 120kHz: 50MHz

Proposal 8: On smallest maximum CHBW for lower device type in 6GR, further discuss 5MHz ~20MHz as maximum CHBW.

Maximum CHBW

Observation 7: The consideration of minimum CHBW and maximum CHBW in RAN1/RAN4 is different

- From RAN1 perspective, the minimum BW/RB and maximum BW/RB was majorly for whole 6GR system design which needs to ensure forward capability.

- The CHBW sets including maximum CHBW in RAN4 majorly considered the spectrum availability, RF/BB feasibility and implementation restriction which shall be within the range of 6GR RAN1 system design on minimum system BW ~ Maximum system BW

Observation 8: The maximum contiguous BW on refarming spectrum is not exceeding 200MHz.

Observation 9: The regulation on ~7GHz is still under discussion in WRC-27, the situation of spectrum allocation is still unclear.

Observation 10: The handling of CHBW in gNB side and UE side is different:

- All Channel bandwidth sets in gNB side are optional with declaration basis

- RAN4 agreed mandatory channel bandwidth sets for UE in day 1

- In NR real field deployment, commonly used maximum BW of single carrier in FR1 and FR2 is 100MHz even 200MHz CHBW was mandatory for FR2 UE in day 1

- NR already support asymmetric CHBW in gNB and UE side

Regarding single carrier 400MHz and 2x200 CA, we see the advantage of using CA can ease the difficulty from implementation perspective e.g., CA allow UE to implement 400MHz BW with separate Tx/Rx chain, reduced sampling rate, PA linearity, dynamic range of each carrier. CA approach also allow schedule flexibility to schedule asymmetric BW in DL and UL side which also save power with carrier activated or deactivated.

Observation 11: Overall observations on RF/BB implementation impact and restriction on supporting 200MHz VS 400MHz, and 400MHz (1cc) vs 400MHz (2 cc) summarized in below table:

|  | 200MHz vs 400MHz | 400MHz 1CC VS 200MHz X2 CCs |
| --- | --- | --- |
| RF Front-end (PA/LNA, RF filter, Antenna) | - 400MHz will increase PAPR and bring linearity issue especially for PA. - Typically, effective BW ratio for PA linearity is 3% (BW/Centre frequency) as upper limit due to memory effect under wide BW. On ~7GHz, the upper limit is 200MHz. - Larger BW required more MPR/PAPR, which bring challenge to support high power class which is critical for 7GHz to achieve same coverage as 3.5GHz - In order to support 400MHz, operating point of PA needs to be adjusted which reduce power efficiency on other BW. | - Spectrum aggregation approach allow flexibility of UE to support 400MHz with separate Tx/Rx chains on each CC. - Ease implementation complexity and constraints on PA |
| RF-IC (Mixer, AD/DA, Analogue filter) | 400MHz required large dynamic range and sampling rate which bring challenge on mixer, AD/DA. |  |
| BB (FFT, processing complexity) | 16K FFT required for 400MHz with 30kHz SCS which bring processing complexity and power consumption. | - CA approach allow UE to implement separate FFT on each CC; reduce processing complicity - Separate digital filtering can be applied on each |
| Tx RF performance (Emission, ACLR) | ACLR performance degraded with 400MHz BW compared to 200MHz | No much difference, CA approach allow UE to implement separate filtering on each CC to improve Tx and Rx performance |
| Rx RF performance | Receiver blocking performance degraded with 400MHz |  |
| Other aspects |  | - CA approach provide scheduling flexibility and power saving benefits with carrier activated/deactivated - CA approach provide possibility to support asymmetric BW in DL and UL side from UE perspective. |

Proposal 9: Take 8K FFT as baseline assumption

Proposal 10: Considering 200MHz CHBW as maximum CHBW for 30kHz SCS in initial stage for RAN4 CHBW set introduction

Proposal 11: 400MHz BW supporting can be further discussed once the spectrum availability more clear

- CA or other spectrum aggregation schemes can be considered in later stage to support 400MHz BW

Proposal 12: Specify the minimum CHBW and maximum CHBW based on numerologies and operating frequency

| SCS | Min CHBW | Max CHBW |
| --- | --- | --- |
| 15kHz | 5MHz 3MHz (below 1GHz bands) | 100MHz |
| 30kHz | 10MHz | 200MHz |
| 120kHz | 50MHz | 800MHz |

Asymetric channel bandwidth

Proposal 13: Further study asymetric channel bandwidth supporting and potentail  enhancement compared to NR after minimum/maximum CHBW and detailed channel bandwidth set fixed.

SU

Proposal 14: For CHBW between 25MHz ~ 100MHz (15kHz/30kHz SCS) in FR, taking existing SU from NR as baseline unless strong motivation well justified for the evaluation and improvement. And prioritize following case on evaluation work

- 15kHz: 5MHz CHBW, 30kHz: 10MHz, 15MHz, 20MHz

Proposal 15: RAN4 further evaluate spectrum utilization with trade-off between improved SU, RF performance and UE/gNB complexity with detailed parameter assumption

- RF non-linearity assumption: PA model, I/Q imbalance, PN

- TX RF core performance assumption: OBE (emission and ACLR), Tx EVM

- Waveform and modulation orders

- Spectrum shaping technologies

Proposal 16: Postpone SU evaluation work to later stage of Rel-20 till waveform, CHBW sets, relevant RF core requirements concluded.

Proposal 17: RAN4 shall specify single SU as per {channel bandwidth, SCS} combination basis w/o dependency on spectrum shaping technologies and RF requirement relaxation conditions.

CHBW sets/Irregular BW

Observation 12: With enhanced channel raster adopted in Rel-18 i.e., 20kHz channel raster, NR already resolve the demand on supporting 10MHz above irregular BW by network scheduling and implementation e.g. Overlapping CA /Overlapping CHBW from network perspective.

Observation 13: The restriction of 6MHz, 7MHz BW in NR was SSB/Coreset BW exceeding overlapping region between 2 normal carriers.

Observation 14: For 6GR, it’s FFS whether dedicated 6MHz, 7MHz CHBW needed or not which is pending RAN1 design on common channel BW if 3MHz Sync/Control channel supporting then no needs to support 6MHz/7MHz CHBW.

Proposal 18: RAN4 shall careful exam channel bandwidth sets to balance UE design/test complexity and flexibility to fully usage operators’ spectrum

- RAN4 requirements and system parameter design are developed based on Channel bandwidths

- Granularity of CHBW sets need to be carefully considered e.g.  mutilple step-size in different BW range

Proposal 19: RAN4 needs to collect operators’ request on irregular BW request first.

- Taking existing collected irregular BW request from TR 38.844 as starting point including 6MHz, 7MHz, 11MHz, 12MHz and 13MHz

Proposal 20: Study potential solution to support irregular spectrum with following direction:

- Overlapping CA /Overlapping CHBW from network perspective

- Channel raster/sync raster and channel mapping rule design to be compatible with flexible BW

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514650,“WF on 6GR system parameter”, Feature Lead (Huawei)

Annex A: RAN4/RAN1/RAN-P related agreements

RAN1#122 and RAN#109 agreements Agreement - Regarding the RAN1 agreement below, the following is agreed: - Keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study. Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. RAN#109 - In RP-252873, proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion) Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. FFS: whether RF and BB UE BW are same or different RAN1#122bis Agreement 6GR study assumes same SCS between 6GR Sync signals and other channels/signals (except PRACH) for a given band. - FFS: same/different SCS between 6GR sync signal and other channels/signals (except PRACH) for FR2-1. - Note: ISAC is separately discussed in ISAC session. Agreement - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

RAN4#116bis - Sub-topic 3-1: Max Channel Bandwidth - Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations. - Sub-topic 3-2: Min Channel Bandwidth - Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective. - Sub-topic 3-3: FFT size - Agreement - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions. - Sub-topic 3-4: Numerology - Agreement - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions. - Sub-topic 3-5: Spectrum utilization - Agreement - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations. - Sub-topic 3-6: Asymmetric channel bandwidths - Agreement - Study the need and feasibility to enable asymmetric CBW # Topic #5: Irregular channel bandwidth - Agreement - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths - Other options not presented in this meeting are not precluded