3GPP TSG-RAN WG4 Meeting #117	                                       R4-2520509

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.10

Source:	Xiaomi

Title:	Overview for 6GR ISAC scope

Document for:		Discussion

- Introduction

In RAN#108, the “New SID: Study on 6G Radio” was approved [1]. The detailed objectives leading by RAN4 was shown below:

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In last RAN4 meeting, following high-level agreements reached in [2]:

General scope and timeline Before April 2026, the agenda for sensing will be kept. RAN4 discussion will primarily focus on the following aspect - Sensing related regulatory status and requirement survey - Potential architecture consideration based on the RAN plenary use case study - views sharing on the potential RAN4 scope based on the identified use cases out of RAN plenary study - Identify less RAN1 design dependent RAN4 aspects if any

In this contribution, we provide our views on the high-level principle of RAN4 study on measurement procedure and requirements, RF, coexistence and testability respectively for 6G sensing.

- Discussion

2.1 Overall scenario

According to SA1 TR 22.837, 32 use cases are identified which are categorized into three groups, i.e., object detection and tracking, environment monitoring and motion monitoring. Additionally, 20 advanced use cases are further agreed in the current SA1 TR 22.870, e.g., environment reconstruction, etc. It’s expected down-selection/prioritization on some use cases over SA/RAN and WGs meetings otherwise the workload will be too broad.

In previous RAN-P discussion, following agreements reached in RAN-P 6G study:

Agreement: (RP-252960, pCR to TR 38.914) Editor note: More sensing use cases can be included depending on further discussion. The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

Observation 1-1: According to 6GR RAN-P SI TR 38.914, at least support  use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs. And other sensing use cases pending on further discussion.

ISAC in 5GA only considers TRP monostatic sensing for UAV use case. 6G Rel-20 is the first study item for an extensive study on ISAC in 3GPP system. And according to current SID, there is not any restriction or preference on the sensing mode. That is the following typical sensing modes can be considered in the 6G studies

- TRP monostatic

- TRP-TRP bistatic

- UE monostatic

- TRP-UE bistatic

- UE-TRP bistatic

- UE-UE bistatic (Uu based)

Observation 1-2: In 6GR, target use cases, sensing modes (e.g., mono-static, bi-static) and deployment scenarios are subject to RAN/SA and RAN1’s discussion and decision.

Since the sensing mode and other using scenarios still pending other WGs and RAN/SA decision, and the other fundamental physical layer design (e.g. the reference signal for sensing) are still subject to RAN1’s discussion. The earliest time for RAN1 discussion is Q2’ 26.

Thus, we shall wait other WGs’ progress to have discussion.

Observation 1-3: With sufficient progress from other WGs, it’s hard to have efficient discussion in RAN4 for ISAC

However, alternatively considering the limited time and lack of guideline from RAN plenary in the beginning of the SI, it is preferred to prioritize the basic scenario of ISAC for RAN4 study. For an example, RAN4 can start the discussion for TRP mono-static and TRP-UE bi-static sensing which are more general and without too high implementation complexity.

Proposal 1-1: RAN4 can start ISAC discussion from the typical sensing modes (e.g. TRP-TRP mono static sensing and TRP-UE bi-static sensing).

| TRP- TRP Mono-static sensing | TRP- UE bi-static sensing |
| --- | --- |

2.2 Co-existence

TDM ISAC splits one frame into alternating “sensing slots” and “communication slots”; base-stations and terminals work in half-duplex on the same frequency—radar first, data next. The upside is hardware simplicity and zero self-interference; the downside is that the extra “silent” period forces a trade-off between data throughput and sensing refresh rate. Meanwhile, ISAC normally share the same base stations with eMBB service which means the assumptions and parameters should be the same during the co-existence study.

Therefore, the TDM ISAC should only consider the interfence between base stations from two networks. Which means that the interfence situation would be similar with the coexistence study of two networks. And the ACLR and ACS requriments can also reuse the values from that studies.

FDM ISAC divides the frame’s spectrum into two continuous chunks: one for always-on radar and one for simultaneous data, relying on full-duplex self-interference cancellation to let both flows coexist. The benefit is ever-present sensing without eating into time slots; the drawback is a permanent bandwidth tax and the need for additional cancellation circuitry, raising both cost and power. For this way, it should be noted that the base station could generate adjacent channel interference to the UEs in the same system.

Figure 1.

However, in the current discussion in RAN1, there is no conclusion for TDM and FDM since the discussion is in the early state.

Observation 2-1: For FDM ISAC, adjacent channel interference to the FDD receiver should be considered.

Observation 2-2: For TDM ISAC, the ACLR and ACS can be reused.

Proposal 2-1: RAN4 should waits the agreements from RAN1 to decide whether the coexistence study is necessary.

UAVs are one of the main tagrets of ISAC, more and more UAVs would be used in the Urban area in the future, in order to control the UAVs and make sure the fly path of UAV is legally and controlable. For the UAVs which normally flying in the higher postion, which means the ISAC beam should towards to the higher postion where normal UEs cannot acchieve. Considering this situation, the interference would getting higher compare to the normal base stations which only serving UEs on the ground, since the beam would be easier cover other base stations. And consider the numbers growing, the interference may not able to ignored, especially in the urban area.

Figure 2.

Observation 2-3: The aggressor base stations work in ISAC-UAVs scenario will generate higher interference to other base stations.

Proposal 2-2: RAN4 should discuss the necessity of the coexistence study for the scenario of ISAC-UAVs to other base stations.

2.3 RF aspects

The detailed RF aspects are strongly depending on physical layer design on waveform, multiplexing mode between communication and sensing (FDM, TDM and SDM), target sensing mode/scenarios and interference coordination/cancellation.

Observation 3-1: The detailed RF impact is pending other WGs’ design.

One of key issue is the interference handling and RF feasibility for variety of scenarios:

- between sensing transmitters/receivers

  - Same or different sensing mode (monostatic/bistatic)

- between sensing and communication

- Same-link or cross-link interference

RAN4 can first focus on Reference architecture and RF feasibility of interference handling on receiver side. Similar evaluation methodology as SBFD self-interference evaluation can be used to evaluate receiver RF performance impact and cancellation technologies including reference architecture.

Proposal 3-2: In initial stage, RAN4 shall focus on the study on reference architecture and RF feasibility of interference handling on receiver side from gNB side (TRP-TRP mono static sensing)

2.4 RRM

2.4.1 Overall impact

There are essentially multiple different sensing modes proposed in SID which are divided into mono-static and bi-static, as well as regarding which entities are involved, i.e. gNBs and UEs. From RAN4 requirements perspective, the impact on the requirements due to the different sensing mode can be summarized in the table below.

Table 4-1. Potential RAN4 spec impacts

|  | gNB requirements impacts | UE RRM requirements impacts |
| --- | --- | --- |
| TRP Mono-static | gNB measurement accuracy requirements | No |
| TRP-TRP bistatic | gNB measurement accuracy requirements | No |
| UE monostatic | No | No |
| TRP-UE bistatic | No | UE measurement reporting and accuracy requirements |
| UE-TRP bistatic | gNB measurement reporting and accuracy requirements | UE transmit timing requirements |
| UE-UE bistatic (Uu based) | No | UE transmit timing requirements UE measurement reporting and accuracy requirements |
| Cooperative sensing and/or multi-static sensing | gNB measurement reporting and accuracy requirements | UE transmit timing requirements UE measurement reporting and accuracy requirements |

Since the sensing mode and other using scenarios can be also under RAN1 discussion for time being. And the other fundamental physical layer design (e.g. the reference signal for sensing) are still under RAN1’s discussion. Thus, we can also wait other WGs’ progress.

Proposal 4-1: Which types of RRM requirements shall be studied in 6G SI can wait for the other WGs’ progress.

2.4.2 Measurement results and requirements

Generally, similar to the positioning measurements in 5G NR, to obtain the sensing results the sensing receiver nodes (either TRP or UE) needs perform measurements on a configured sensing signal (e.g., a legacy NR PRS or other new 6GR RS). The types of measurements can be the reference signal strength, timing, or velocity, and horizontal/vertical angle (position and velocity).

Also in TS 22.137, SA1 also defines several key metrics for 5G sensing. The definitions of the metrics are self-contained below for references.

5G wireless Sensing service is required to fulfil different performance requirement (e.g., accuracy, resolution, latency, etc.) based on the characteristics of one or multiple target object(s) and/or the environment to be sensed in a target sensing service area. The following set of key performance requirements is used and shown for each service scenario in Table 6.2-1. - Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude. - Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity. - Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy. - Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude. - Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results. NOTE 1: An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive. - False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results. NOTE 2: An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative. - Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface. - Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results. 5G wireless sensing performance requirements are applied to 3GPP sensing data and sensing results.

Thus, we believe the following measurement requirements could to be studied in RAN4.

- Delay, and/or Doppler, and/or angle in sensing RX

- Delay, Doppler, angle, and power per path

- Accuracy for the measurements which used for the final positioning/velocity estimation

Proposal 4-2: RAN4 can consider the several common measurements requirements below for sensing purpose:

- Power strength, timing delay, Doppler, angle in sensing RX

- Power strength, timing delay, Doppler, angle per path in the sensing RX

- Accuracy for the measurements which used for the final positioning/velocity estimation

Moreover, in TS 22.137 several KPI for the different sensing use case in 5G was provided.

In principle, when RAN4 define the necessary requirements for sensing can take these KPI as a start point. e.g.

- the delay of a successful measurement report

- the confidence level for a successful detection

- the minimum accuracy report granularity

Proposal 4-3: RAN4 can take the KPIs of 5G sensing as a start point to discuss the necessary measurement requirement. E.g.

- the delay of a successful measurement report

- the confidence level for a successful detection

- the minimum accuracy report granularity

2.4.4 Specific gap patterns for sensing measurement

In 6G ISAC, the sensing reference signal to be measured can be located in the different carriers as the communication signal. In order to avoid any data missed because UE needs to retune to the sensing signal in the different carrier as the serving cell, the similar mechanism as NR SSB measurement gap can be introduced for sensing measurement over the multiple carrier.

Observation 4-1: From RAN4 perspective, if UE needs to receive the more than one sensing signals from the different cells/carriers as the serving cell, the gap window like the measurement gap in NR will be specified in RAN4 spec.

That is NW needs not to scheduling any data during the time window of gap. At same time, UE is required not to transmit any UL data to NW within this time window (gap) also.

And according to some preliminary evaluation results, in order to achieve the reliable Doppler shift estimation, the measured reference signal shall be long enough (e.g. >40ms) as illustrated in Figure 2 below.

Figure 3.

Observation 4-2: In order to support the more precise and contiguous Doppler shift estimation over a measurement sample, the measurement duration of one occasion shall be not less than 40ms.

Therefore, we can propose that:

Proposal 4-4: From RAN4 perspective, the measurement gap pattern for sensing especially for the Doppler shift estimation can be reconsidered.

2.5 Test

For ISAC, RAN4 needs to study suitable test method considering movement of sensing target and new sensing metric, and new ISAC channel model.

The target metric on sensing is different compared to traditional communication which is pending on use cases. According to 5G-A use case on UAV detection, following candidate metrics were considered in RAN1 evaluation:

| Metric | Value |
| --- | --- |
| Missed detection Probability | [5]% |
| False Alarm Probability Type 1 | [5]% |
| False Alarm Probability Type 2 | [5]% |
| Horizontal Positioning Accuracy | [10] m with confidence level 90% |
| Vertical Positioning Accuracy | [10] m with confidence level 90% |
| Velocity Accuracy | [5] m/s with confidence level 90% |

Another point, the channel model on sensing is also different compared to traditional communication pending sensing mode as captured in TR 38.901 under 5G-A study. Channel emulator can be used to model the movement of detect objective

Proposal 5-1: RAN4 need to study test method for ISAC considering movement of sensing targets, new sensing requirement metric and ISAC channel model.

- Conclusion

In this contribution, the preliminary analysis on 6G sensing works in RAN4 was provided. The observations and proposals can be summarized as

Overall:

Observation 1-2: In 6GR, target use cases, sensing modes (e.g., mono-static, bi-static) and deployment scenarios are subject to RAN/SA and RAN1’s discussion and decision.

Observation 1-3: With sufficient progress from other WGs, it’s hard to have efficient discussion in RAN4 for ISAC

Proposal 1-1: RAN4 can start ISAC discussion from the typical sensing modes (e.g. TRP-TRP mono static sensing and TRP-UE bi-static sensing).

Coexistence study:

Observation 2-1: For FDM ISAC, adjacent channel interference to the FDD receiver should be considered.

Observation 2-2: For TDM ISAC, the ACLR and ACS can be reused.

Proposal 2-1: RAN4 should waits the agreements from RAN1 to decide whether the coexistence study is necessary.

Observation 2-3: The aggressor base stations work in ISAC-UAVs scenario will generate higher interference to other base stations.

Proposal 2-2: RAN4 should discuss the necessity of the coexistence study for the scenario of ISAC-UAVs to other base stations.

RF impact

Observation 3-1: The detailed RF impact is pending other WGs’ design.

Proposal 3-2: In initial stage, RAN4 shall focus on the study on reference architecture and RF feasibility of interference handling on receiver side from gNB side (TRP-TRP mono static sensing)

RRM study:

Proposal 4-1: Which types of RRM requirements shall be studied in 6G SI can wait for the other WGs’ progress.

Proposal 4-2: RAN4 can consider the several common measurements requirements below for sensing purpose:

- Power strength, timing delay, Doppler, angle in sensing RX

- Power strength, timing delay, Doppler, angle per path in the sensing RX

- Accuracy for the measurements which used for the final positioning/velocity estimation

Proposal 4-3: RAN4 can take the KPIs of 5G sensing as a start point to discuss the necessary measurement requirement. E.g.

- the delay of a successful measurement report

- the confidence level for a successful detection

- the minimum accuracy report granularity

Observation 4-1: From RAN4 perspective, if UE needs to receive the more than one sensing signals from the different cells/carriers as the serving cell, the gap window like the measurement gap in NR will be specified in RAN4 spec.

Observation 4-2: In order to support the more precise and contiguous Doppler shift estimation over a measurement sample, the measurement duration of one occasion shall be not less than 40ms.

Proposal 4-4: From RAN4 perspective, the measurement gap pattern for sensing especially for the Doppler shift estimation can be reconsidered.

Test

Proposal 5-1: RAN4 need to study test method for ISAC considering movement of sensing targets, new sensing requirement metric and ISAC channel model.

- References

- RP-251881, “Study on 6G Radio”, NTT Docomo

- R4-2514592, “WF on 6GR sensing”, Feature Lead (ZTE)

- TS38.133 v19.1.0

- TS38.331 v19.1.0