# 3GPP TSG-RAN WG4 Meeting #117	R4-2520511

# Dallas, US, 17-21 Nov, 2025

Agenda Item:	8.10

Source: 	Xiaomi

Title: 	Overview for 6GR spectrum sharing

Document for:	Discussion

- Introduction

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. Spectrum sharing is one of the area with following candidate scope:

According to the 6G SID, 6GR target frequency includes FR1, FR2-1 and frequency range between 7GHz ~ 24GHz:

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

Compared to 5G system, the brand new clean spectrum for 6GR day 1 deployment is rare. Following spectrum sharing/co-existence scenarios are observed:

- Spectrum sharing/co-existence with legacy system (4G and 5G) on existing reframing frequency in FR1 and FR2

- 6G system co-existence with incumbent services in FR3 e.g. radar, military, and so on

In this contribution, our preliminary view on spectrm sharing is provided on following areas:

- Multi-RAT Spectrum sharig (interference handling and sync raster)

- 5G/6G Mobility (RRM mobility and measurement)

- TN-NTN spectrum sharing

- Discussion

  - Multi-RAT spectrum sharing

Overall scope

According to the 6G SID, following objective is dedicated for 5G NR and 6GR for spectrum migration:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Observation 1: 6G standalone, Intra-RAT CA, MRSS (5G-6G), inter-RAT mobility between 5G and 6GR are considered as Day 1 feature for 6GR deployment.

Obseravtion 2: The inclusion of LTE/6G interworking and co-existence is FFS and subject to RAN-P decision.

In NR, RAN4 developed several features considering spectrum sharing and migration from 4G to 5G, e,g, 100kHz channel raster, 30kHz SSB pattern B, CRS-IC. And supporting of these configurations and features are subject to operators’ demand.  For 6GR, we also can collect operators’ feedback on the demand of MRSS between 5G and 6G.

Proposal 1: On spectrum sharing, focus on MRSS between 5G and 6G case in FR1 (400MHz ~ 7.125GHz) and FFS on FR2

- Collect operators’ feedback on potential deployment on MRSS between 5G and 6G in existing NR refarming bands in FR1 and FR2-1

System parameter impact

Numerology

On numerology, 6GR target to have unified numerologies as per band per sub-frequency basis. And existing NR refarming bands, we observed only single numerology was deployed in existing commercial network even multiple numerologies specified in NR.

Another issue, in NR on some bands, multiple SSB numerologies (both 15kHz SSB and 30kHz SSB) and were specified considering dynamic spectrum sharing between NR and 4G due to SSB placement with MBSFN configuration in LTE.

For spectrum sharing between 5G and 6G, we have no such issue foreseen which means it’s feasible to align SSB and data numerologies with single SCS as per band per sub-frequency range basis.

Proposal 2: Avoid mixed numerologies between NR and 6GR for MRSS scenario.

Proposal 3:  6GR target to have aligned single numerology between NR and 6GR for both data/control channel and SSB as per band/per sub-frequency range basis.

Proposal 4: Following numerologies proposed on NR refarming bands

| Frequency range | SCS for data/control channel except PRACH | SCS for PBCH (initial cell access) |
| --- | --- | --- |
| Below 3GHz (FDD bands) | 15kHz | 15kHz |
| Below 3GHz (TDD bands) | 30kHz | 30kHz |
| 3GHz ~ 7.125GHz | 30kHz | 30kHz |
| 24.25 GHz -52GHz | 120kHz | 120kHz |

Channel raster

For channel raster, on LTE refarming bands to NR, 100kHz channel raster was considered. However, 100kHz channel raster provide some restriction on channel arrangement especially on irregular BW. Also, SS raster shift was introduced due to 100kHz channel raster.

Observation 3: 100kHz channel raster bring some restriction on channel mapping and sync raster design.

It’s better to further evaluate any possibility to drop 100kHz channel raster for 6GR and any impact on MRSS.

Proposal 5: RAN4 further evaluate the needs on 100kHz channel raster for 6GR on refarming bands.

Sync raster

Another issue on PBCH design for initial access, how to differentiate 5G PBCH and 6G PBCH for MRSS if SSB block is similar between 5GR and 6GR and then SS raster location maybe overlapped between 5G SSB and 6G SSB. It will require additional effort for UE to discriminate 5G PBCH and 6G PBCH. Those details are pending on RAN1 design on SSB block.

Proposal 6: RAN4 needs to further evaluate sync raster design for 6GR on 5G migration bands pending on RAN1 progress

- E,g. how to discriminate 6G sync raster  and 5G sync raster  if separate SSB introduced for NR and 6GR.

On other system parameters i.e., channel bandwidth, modulation orders and waveform we didn’t see the impact. On potential issue related to irregular BW is 100kHz channel raster limitation as mentioned above which can be further discussed.

Observation 4: No other system parameters i.e., channel bandwidth, modulation orders and waveform impact foreseen on MRSS except channel raster and sync raster.

Interference handling

On spectrum sharing, one of issue is how to handle interference between 6G and 5G/4G for always on signals:

- 6G/5G system: DSS (dynamic spectrum sharing) – PBCH and control channel, CSI-RS for L1 measurement/L3 mobility

- 6G/4G system: Sem-static spectrum sharing- PBCH, CRS (LTE) and control channel

Taking 5G DSS experience into account:

- RE-level rate-matching solution was introduced for CRS interference, and RB level rate-matching was introduced for other always on-signal e.g., PBCH.

- MBSFN and scheduled control channel under different symbols for NR and LTE to avoid interference.

The drawback of rate-matching is increased overhead for spectrum sharing with throughput loss and the UE receiver performance also degraded due to always on signal interference.

RAN4 introduced CRS-IM receiver to handle CRS interference from LTE cells (serving and neighbouring cells) with following scenario:

•	Scenario 1: Serving and interference cells are both operating in DSS (NR+LTE) mode

•	Scenario 2: Serving cell is operating in NR mode and interference cell is operating in LTE mode

| Scenario 1: | Scenario 2: |
| --- | --- |

Figure 5.1.1-1. Scenarios for CRS interference in overlapping spectrum for LTE and NR

Several schemes were evaluated on the performance impact for CRS interference handing:

| Scenario 1 | Reference scheme | Rel-15 serving cell CRS-RM without interference cell CRS handling |
| --- | --- | --- |
|  | Scheme #1 | Rel-16 CRS-RM for 1 interference cell (The rate matched CRS is always the first dominant interference) |
|  | Scheme #2 (Optional) | Rel-16 CRS-RM for 1 interference cell (The rate matched CRS is NOT always the first dominant interference. i.e. 50% probability for rate matching (RM) for the first dominant interference and 50% probability for RM for the second dominant interference) |
|  | Scheme #3 | Rel-15 RB symbol level CRS-RM for 2 interference cells |
|  | Scheme #4 | CRS-IC with network assistance |
|  | Scheme #5 | CRS-IC without network assistance |
|  | Scheme #6 | LLR weighting with network assistance |
|  | Scheme #7 | LLR weighting without network assistance |

The performance difference was summarized in below figure from the evaluation results:

For 6G co-existence with 5G and potential 4G, RAN4 can further evaluate potential solution e.g., advanced receiver and supporting in day 1.

Proposal 7: RAN4 study potential RAN4 centric solutions on handling interference between 4G/5G and 6G for always on signal e.g., control channel, PBCH and CSI-RS.

- Scenario 1: spectrum sharing between 5G/6G

- Scenario 2: 6G and 5G co-existence with neighbour cell interference

- Postpone the discuss until there is sufficient progress from RAN1 for MRSS and initial cell access i.e., no early than Q2’ 2026

  - 5G/6G Mobility

In NR, one of scenarios of inter-RAT measurement without gap is LTE CRS are fully contained within UE’s activated BWP.  For an example, in case of LTE DSS scenario the measurement on LTE CRS can be performed in gap-less way when NR UE is located in the same frequency carriers as the neighbour LTE cells. However, in 6G MRSS, when 6G UE performing the inter-RAT measurement on MRSS 5G neighbour cell, it is possible that the 5G SSB was not contained by the UE’s activaed BWP. It’s worth to further study the potentail inter-RAT measurement under MRSS sceanario.

Proposal 8: RAN4 shall study potentail inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario.

  - TN-NTN spectrum sharing

Direct service from satellite to UE become popular use mode for NTN communication. “Direct-to-handset” services can provide NTN communication over TN spectrum as supplementary service e.g. TN coverage is not available.

Within ITU-R WP4C, Agenda Item 1.13 focuses on studying potential primary allocations for the Mobile-Satellite Service (MSS) within the 694 MHz – 2.7 GHz range. The objective is to enable MSS systems—particularly Direct-Connectivity MSS (DC-MSS) that link standard user equipment (UE) directly to NGSO or GSO satellites—to operate in frequency bands already harmonized for terrestrial IMT. Following the WRC-27 meeting, this scenario may also become a key focus area for 3GPP. Specifically, 3GPP would likely need to address terrestrial network (TN) and non-terrestrial network (NTN) spectrum sharing within TN frequency bands, aiming to provide additional spectrum resources for NTN systems. Within 3GPP, the relevant spectrum sharing scenario involves TN and NTN systems operating within the same or neighbouring geographic areas within a single country.

Considering the issue of spectrum scarcity in 3GPP, the 5G and 6G NTN spectrum sharing/coexistence issue in same/adjacent channel could be one of the potential issues of 6GR NTN system. This issue also caused by the limited NTN spectrum resources; satellite operators could consider the possible of using the 5G NTN spectrum in the 6G NTN system to address spectrum scarcity at least in the earlier stage of 6GR.

3GPP may also need to consider spectrum sharing for NTN with following candidate scenarios:

- NTN operation use TN bands/spectrum

- 5G NTN and 6G NTN share NTN spectrum

How to enable spectrum sharing above two scenarios including static and, dynamic and/or geographic isolation are subject to regulation update and detailed design from RAN1/RAN2 for 6G NTN design.

| illustration of TN and NTN spectrum sharing and multi-RAT NTN spectrum sharing |  |
| --- | --- |

Proposal 9: RAN4 shall study potential solutions and impact on co-channel interference handling and mobility which also related to regulation update including following scenarios:

- NTN operation use TN bands/spectrum

- 5G NTN and 6G NTN share dedicated NTN spectrum

- Conclusion

In this contribution, our preminary view on spectrum sharing are provided on potentail RAN4 impact/scope.

Overall scope:

Observation 1: 6G standalone, Intra-RAT CA, MRSS (5G-6G), inter-RAT mobility between 5G and 6GR are considered as Day 1 feature for 6GR deployment.

Obseravtion 2: The inclusion of LTE/6G interworking and co-existence is FFS and subject to RAN-P decision.

Proposal 1: On spectrum sharing, focus on MRSS between 5G and 6G case in FR1 (400MHz ~ 7.125GHz) and FFS on FR2

System parameter

Proposal 2: Avoid mixed numerologies between NR and 6GR for MRSS scenario.

Proposal 3:  6GR target to have aligned single numerology between NR and 6GR for both data/control channel and SSB as per band/per sub-frequency range basis.

Proposal 4: Following numerologies proposed on NR refarming bands

| Frequency range | SCS for data/control channel except PRACH | SCS for PBCH (initial cell access) |
| --- | --- | --- |
| Below 3GHz (FDD bands) | 15kHz | 15kHz |
| Below 3GHz (TDD bands) | 30kHz | 30kHz |
| 3GHz ~ 7.125GHz | 30kHz | 30kHz |
| 24.25 GHz -52GHz | 120kHz | 120kHz |

Observation 3: 100kHz channel raster bring some restriction on channel mapping and sync raster design.

Proposal 5: RAN4 further evaluate the needs on 100kHz channel raster for 6GR on refarming bands.

Proposal 6: RAN4 needs to further evaluate sync raster design for 6GR on 5G migration bands pending on RAN1 progress

- E,g. how to discriminate 6G sync raster  and 5G sync raster  if separate SSB introduced for NR and 6GR.

On other system parameters i.e., channel bandwidth, modulation orders and waveform we didn’t see the impact. On potential issue related to irregular BW is 100kHz channel raster limitation as mentioned above which can be further discussed.

Observation 4: No other system parameters i.e., channel bandwidth, modulation orders and waveform impact foreseen on MRSS except Numerology, channel raster and sync raster.

Interference handling

Proposal 7: RAN4 study potential RAN4 centric solutions on handling interference between 4G/5G and 6G for always on signal e.g., control channel, PBCH and CSI-RS.

- Scenario 1: spectrum sharing between 5G/6G

- Scenario 2: 6G and 5G co-existence with neighbour cell interference

- Postpone the discuss until there is sufficient progress from RAN1 for MRSS and initial cell access i.e., no early than Q2’ 2026

Inter-RAT mobility

Proposal 8: RAN4 shall study potentail inter-RAT RRM measurement impact including w/o and with gap under MRSS scenario.

TN-NTN spectrum sharing

Proposal 9: RAN4 maybe study potential solutions and impact on co-channel interference handling and mobility which is  also pending on regulation update including following scenarios:

- Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] R4-2514646, WF for [116bis][108] 6G spectrum sharing, Feature lead (CMCC)