3GPP TSG RAN4 Meeting #117							R4-2520549

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	6GR System parameters - Channel Arrangement

Agenda item:	8.2.4

Document for:	Approval

Introduction

Under 6G system parameters topic following items are to be discussed during the Q4 2025 [1].

Waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types.

At this meeting a sperate agenda have been set for the channel arrangements including channel raster, sync raster, channel spacing etc which is discussed in this paper.

WF from last meeting

### Sub-topic 4-1: Channel raster

- Agreement

  - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands

    - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster

  - Investigate the interaction between the channel raster and the synchronization raster

  - Investigate the necessity of channel raster or alternative ways for the channel configuration

    - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster

    - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported

  - Study the listed main proposals especially for the migration and co-existence approaches

    - Other options not presented in this meeting are not precluded

  - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based).

### Sub-topic 4-2: Sync raster

- Agreement

  - Evaluation on sync raster from RAN4 perspective:

    - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4

    - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving

  - Collaboration and planning:

    - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset.

    - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1.

### Sub-topic 4-3: Channel spacing

- Agreement

  - Further study in RAN4 regarding channel spacing

Discussion

Initial access and synchronization maintenance

As we discussed in our RAN1 contribution [R1-2505125], 6GR design for initial access should consider drivers from network energy saving perspective and enabling network to support different types of cells while considering UE complexity and different UEs like eMBB and LPWA devices. Periodical always-on signaling like periodical SS/PBCH transmissions in 5G contribute to network energy consumption. 5G was able to enhance network energy efficiency from 4G by decreasing PSS/SSS periodicity four times and replacing CRS transmitted in every subframe with periodical TRS transmitted more sparsely in time. For 6G it is expected that time domain footprint of the always-on signals and channels related to initial access can be further reduced.

6GR to further reduce the time-domain footprint of always-on signals and channel related to initial access compared to 5G NR, aiming at increased energy savings.

It is important to consider from first release what is the minimum channel bandwidth envisioned to be supported in 6GR to avoid needing in future to adapt the initial-access related signals to reduced bandwidths as also mentioned in our accompanying paper submitted under agenda 8.2.3 [2].

Channel Raster

For NR bands up to 2.4 GHz refarmed from 4G, 100 kHz channel raster is introduced so that the compatibility with the legacy RATs is maintained. As 100 kHz raster cannot always support UE specific channel bandwidth configured within the SIB1 carrier bandwidth unless their number of RBs are both odd or both even. Thus, the enhanced channel raster 10 kHz is introduced to resolve this even/odd issue in Rel-18. UE is mandated to support the enhanced channel raster from Rel-18.

For NR bands above 2.6 GHz for both FR1 and FR2, SCS based raster is introduced to support flexible wideband operation.

The compatibility with NR channel raster would be required for 6G to support efficient multi-radio spectrum sharing (MRSS), therefore the compatibility of 6G channel raster with NR channel raster shall be remained for NR refarming bands.

The 6G channel raster shall be compatible with NR channel raster for the NR refarming bands. Specifically, the 10 kHz enhanced channel raster shall be the baseline for the bands below 2.4 GHz and SCS based raster shall be the baseline for the bands above them.

Sync Raster

When UE is powered on or is in out-of-coverage, UE makes a cell search to find a sync signal transmitted from a network which UE can camp on. If no preferred network or previously registered network is found, UE may go into network scan to exhaustively search for any cell among all the possible frequencies supported by UE. This may cost a lot of search time and consumes a lot of power until UE can get into low power idle mode.

Therefore, the NR sync raster is defined as such that there is a minimum number of entries per band to minimize UE’s initial access efforts. This design concept of NR sync raster is described in TR 38.817-01 clause 4.3.

As RAN1 is still discussing the design of 6G SS/PBCH, it may be early for RAN4 to discuss the 6G sync raster design in detail. However, the following design principles in the NR sync raster can be considered.

- The number of sync raster entries is as small as possible.

- SSB is not in a fixed position within the SIB1 carrier bandwidth and thus one SSB position can support multiple SIB1 carrier bandwidth configurations.

A range of channel raster that one SSB position can support depends on the minimum channel bandwidth of the band and the bandwidth of SS/PBCH as discussed in TR 38.817-01 clause 4.3.1.1. The range gets wider (i.e., the minimum separation of sync raster points gets larger), if the SS/PBCH bandwidth is smaller and the minimum channel bandwidth is larger. In NR Rel-15, 5 MHz is the minimum channel bandwidth and 20 PRB (3.6 MHz) is the size of SS/PBCH for 15 kHz SCS. This has made 1.2 MHz sync raster cluster (of 3 SSB) separation for 100 kHz channel raster and 1.44 MHz sync raster separation for SCS based raster for FR1 bands.

When 3 MHz channel bandwidth is introduced later in Rel-18 for some of FR1 bands, a specific SS/PBCH design is introduced and more dense sync raster (i.e., 600 kHz sync cluster separation) than legacy channel bandwidths is introduced. Requiring 3 MHz channel bandwidth to all frequency bands would cause lots of burden in initial access, specific sync raster design may still be needed.

The sync raster design is further discussed according to the progress of SS/PBCH design and the minimum channel bandwidth requirement. For bands requiring ~3 MHz CBW, specific raster design may still be needed for 6GR as in NR.

Channel Spacing

The nominal channel spacing defined for NR has caused some misunderstanding among those who plan their network deployment due to its complex definition. The benefit of the current definition is that the nominal channel spacing is the same among multiple numerologies so that aggregated carriers are all contiguous or non-contiguous for all the deployed numerologies. The mixed numerology is deployed in none of 5G commercial networks; thus, the nominal channel spacing formula has turned out unnecessarily complex. It is proposed to simplify the nominal channel spacing without considering simultaneous mixed numerologies.

The nominal channel spacing for 6G shall be simplified without considering simultaneous mixed numerologies.

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations have been made:

Proposal 1: 6GR to further reduce the time-domain footprint of always-on signals and channel related to initial access compared to 5G NR, aiming at increased energy savings.

Proposal 2: The 6G channel raster shall be compatible with NR channel raster for the NR refarming bands. Specifically, the 10 kHz enhanced channel raster shall be the baseline for the bands below 2.4 GHz and SCS based raster shall be the baseline for the bands above them.

Proposal 3: The sync raster design is further discussed according to the progress of SS/PBCH design and the minimum channel bandwidth requirement. For bands requiring ~3 MHz CBW, specific raster design may still be needed for 6GR as in NR.

Proposal 4: The nominal channel spacing for 6G shall be simplified without considering simultaneous mixed numerologies.

- 4	References

R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4#116, RAN4 Chair

R4-2520550, 6GR System parameters - Channel Bandwidth, RAN4#117, Nokia.