3GPP TSG-RAN4 Meeting  #117	R4-2520685

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.5

Source:	Apple

Title:	On 6G system parameters – Device types

Document for:	Discussion

# 1	Introduction

At the last meeting, RAN4 started 6G discussion. A WF on 6G system parameters was agreed [1], including the following aspects on device types:

# Topic #7: Device types - Main proposals - Category of Device Types: There is common view to define a limited set of device types from 6G "Day One" to avoid market fragmentation. The most frequently proposed categories are (may not cover all diversified proposals): - Massive IoT/Type C: For sensors, low-end wearables; characterized by low complexity, 1T1R, 3-20 MHz CBW, and basic modulation. - Reduced-Capability/Wearable/Scalable UE/ Type B: For smartwatches, XR; with moderate capabilities (e.g., 1T2R, 100 MHz CBW). - Smartphone/Normal UE (including XR)/Type A: The mainstream handheld device; offering improved performance over 5G (e.g., 1T/4R, 2T4R, 200 MHz CBW, 1024QAM). - FWA/Advanced UE/Type A+: For Fixed Wireless Access, humanoid robot, etc.; with relaxed form-factor constraints and the highest capabilities (e.g., 4T8R/8T8R, 400 MHz CBW, 4096QAM). - Scalable and forward-compatible framework: - Mandatory baseline functionality set: A proposal suggests a mandatory baseline functionality set, in which each functionality shall be well justified to be truly mandatory for all device types. And on top of the mandatory baseline functionality set, different device types could be further defined. Several proposals suggest to have mandatory set as per device type basis. Capability-based parameters: Proposals emphasize that device types should be defined by concrete RF/Baseband parameters, including capabilities such as: - Number of Tx/Rx antennas and MIMO layers - Min and max Channel Bandwidth - Supported modulation orders (DL/UL) - Power Class - Duplex Mode (FDD, TDD, HD-FDD) - RRM mobility - Enhanced signaling and network optimization: - UE capability reporting: Introduce a modular and MAC-layer-assisted UE capability reporting framework to enable efficient, category-based network optimization without excessive per-UE signaling overhead. - Dynamic capability update: Study the possibility for UEs to update their capabilities (e.g., for foldable devices or different power modes), requiring cooperation between the network and the UE. - RAN4's specific role: - Focus on RF-critical attributes: RAN4 should focus on study the RF/RRM/Demod characteristics that differentiate device types, ensuring requirements are achievable, testable and avoid unnecessary specification fragmentation. - Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

In this contribution, we continue the discussion on 6G device types based on the WF above.

# 2	Discussion

6G is expected to support several difference device types including smartphones, wearables, FWA/CPEs, and XR. Clearly, those device types have different form factors and different RF and BB capabilities.

Furthermore, it is worth noting that 5G NR did not specify different device types on day one and had to add RedCap device support in R17. This approach is not preferred as anything after day 1 may have less opportunity to be supported.

Therefore, it is necessary to specify and support diverse device types in 6G from day one, at least with the following types:

- FWA

- Smartphone

- Wearable device

- IoT

Proposal 1: It is proposed to specify and support diverse device types in 6G from day one, at least with the following types:

- FWA

- Smartphone

- Wearable device

- IoT

We would also like to note that there is a need to further differentiate within each device type given the different form factors, implementation complexity and costs, etc. This is also reflected in the fact that even for the smartphone type, there can be difference sizes. For example, candybar smartphones and foldable smartphones would have different physical dimensions and battery capacities, thereby having different capacities to support different number of TX and RX.

Proposal 2: For some device types, there may be need to further differentiate, given the different sizes/form factors.

As for the parameters used to define device types, we propose to consider the following three and provide the detailed reason.

Number of TX/RX

From system performance perspective, the number of TX/RX determines the following performance metrics:

- The max. number of MIMO layers a UE can support in UL and DL, which would directly determine the UL and DL data rate. Note that the max. number of MIMO layers may be smaller than the number of TX/RX, depending on UE capability.

- The max. coverage that can be achieved. In the UL, multiple TX can be used to boost received power at the BS through TX diversity. In the DL, multiple RX can be used to increase UE receiver sensitivity through RX diversity.

From RF perspective, the UE form factor restricts the available physical space to place a certain number of antennas in the UE while maintaining good isolation and radiation efficiency. Also, the TX/RX chains including PA, LNA, mixer, filters, switches, behind each antenna are limited by the silicon area and power consumption constraint. Obviously, power consumption is bound by the form factor too because the form factor would determine the battery size and how heat dissipation can be done.

From BB perspective, how many TX/RX can be supported depends on BB processing capability, which again is limited by power consumption, memory sizes, heat dissipation, etc.

Observation 1: Number of TX/RX determines system performance such as UL/DL data rate (via MIMO layers supported) and UL/DL coverage. It depends on UE form factor and represents UE RF and BB capability.

Max. channel bandwidth (CBW)

From system performance perspective, max. UE CBW determines the max. amount of bandwidth the network can schedule to a UE in a single CC when the amount of traffic requires it and the channel conditions allows it. Clearly, max. UE CBW affects the data rate in both UL and DL. Of course, CA can also be used to increase the bandwidth, although UE support of CA is optional.

From RF perspective, the wider the CBW is, the higher the requirements on RF design. It is more challenging for a PA to maintain linearity and gain flatness over the entire channel bandwidth. To maintain linearity over a wider bandwidth, the PA may become less power-efficient, which generates more heat and consumes more power. It is also harder to address the PA memory effects with adaptive DPD algorithms. On the filter side, the larger the passband is, the harder to design a filter with high selectivity or narrow transition band. It also complicates the antenna design, as antennas bandwidth is limited. More specifically, wider channel bandwidth demands more broadband antenna tuning networks, which adds complexity and loss.

From BB perspective, the wider the channel bandwidth is, the larger FFT size is required given the same SCS. As analyzed in our contribution on CBW [2], larger FFT would significantly increase implementation complexity compared to smaller FFT in terms of computational complexity, memory requirement, power consumption, and silicon area and cost.  Furthermore, supporting wider CBW requires ADCs/DACs with higher sampling rate, which becomes more complex and consumes more power.

Observation 2: Max. CBW affects system performance in terms of peak data rate in both UL and DL. It significantly impacts UE implementation and represents UE RF and BB capability.

Power classes

UE power classes determine the UE max. TX power and consequently UL coverage and data rate. As the most power-consuming RF component, PA power efficiency and linearity greatly impacts implementation. At the same time, there is ongoing discussion in the 6G UE RF track on how to specify power classes in 6G including the potential reduction of the number of power classes to be specified. There is also argument that power classes is less dependent on UE form factor. It is therefore better to further discuss if it should be a defining parameter for device types.

Given the above analysis, we propose

Proposal 3: It is proposed to consider the following parameters in defining device types:

- Number of TX/RX

- Max. channel bandwidth

- FFS: Power classes

# 3	Conclusions

In this contribution, we discuss 6G system parameters and make the following proposals.

Proposal 1: It is proposed to specify and support diverse device types in 6G from day one, at least with the following types:

- FWA

- Smartphone

- Wearable device

- IoT

Proposal 2: For some device types, there may be need to further differentiate, given the different sizes/form factors.

Proposal 3: It is proposed to consider the following parameters in defining device types:

- Number of TX/RX

- Max. channel bandwidth

- FFS: Power classes

# 4	References

R4-2514641, "WF for 6GR system parameter," Feature lead (Huawei, HiSilicon)

R4-2520683, "On 6G system parameters – channel bandwidth," Apple