3GPP TSG-RAN WG4 Meeting #117	R4-2520708

Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	On General aspects in AIML Mobility

Agenda item:	7.12.2

Document for:	Discussion

Introduction

The discussion on AI/ML Mobility for the normative work was started in RAN4#116bis meeting in the scope of WI for Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR [1].

In this paper, we provide our views on scenarios and sub-use cases for AI/ML Mobility use cases.

Discussion

Scenarios and sub-use cases

The following agreement was reached in RAN4#116bis meeting:

Agreement: RAN4 to discuss the potential RRM requirements for cell-level based prediction for the following scenarios: Evaluation scenario; Methodology. FR1 to FR1 intra-frequency temporal domain case A; intra-cell. FR1 to FR1 intra-frequency temporal domain case B; Intra-cell. FR1 to FR1 inter-frequency (frequency domain) (co-located case); Inter-cell. FR2 to FR2 intra-frequency temporal domain case A; Intra-cell. FR2 to FR2 intra-frequency temporal domain case B; Intra-cell. FFS whether to discuss the requirements for the following scenario: Evaluation scenario; Methodology. FR2 to FR2 inter-frequency (frequency domain) (co-located case); Inter-cell.

Also from RAN2 perspective , following  prioritization among evaluation scenarios has been captured in TR 38.744.

Table 1: RAN2 Prioritization of evaluation scenarios [TR 38.744]

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | intra-cell |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Based on Table 1, it is understood that RAN2 has prioritized scenarios 2, 3, and 4 as the main focus for defining the required signalling. Other scenarios, such as FR1 to FR1 intra-frequency temporal domain case A and FR2 to FR2 intra-frequency temporal domain case B, appear to have been deprioritized, potentially due to time constraints, lack of consensus on suitable use cases, or limited performance gains.

From RAN4's perspective, it may seem reasonable to discuss potential requirements for scenario 1 (FR1 to FR1 intra-frequency temporal domain case A) and scenario 2 (FR2 to FR2 intra-frequency temporal domain case B). However, it is essential that RAN2 first defines the necessary signalling (e.g., signalling for applicability reporting) to support these scenarios. Without such signalling, testing these scenarios would not be feasible. Therefore, RAN4 should only consider defining requirements for these scenarios if RAN2 provides the necessary signalling.

Additionally, for scenarios such as FR2 to FR2 inter-frequency (frequency domain) prediction, RAN4 would need to allocate significant time for performance evaluation, potentially conduct new simulations, and define requirements if the performance gain is deemed justifiable. This would require considerable effort given the current RAN4 Time-Unit allocation.

Considering that RAN2 leads this Work Item (WI), it is more appropriate for RAN4 to align with RAN2's defined priorities and focus on performance evaluation and requirement definition for the highest-priority scenarios. Lower-priority scenarios can be addressed subsequently, in coordination with RAN2, to ensure signaling support is available.

Scenarios 1 (FR1 to FR1 intra-frequency temporal domain case A) and 5 (FR2 to FR2 intra-frequency temporal domain case B) have been identified as low priority by RAN2. Furthermore, there has not been extensive study on FR2 to FR2 inter-frequency prediction.

RAN4 to align with RAN2's priority definition of use cases and initially focus on defining the performance and requirements for the high-priority scenarios, specifically FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, and FR2 to FR2 intra-frequency temporal domain case A.

RAN4 to only consider defining potential requirements for scenarios 1  (i.e. FR1 to FR1 intra-frequency temporal domain case A) and scenario 2 (i.e. FR2 to FR2 intra-frequency temporal domain case B) if RAN2 agrees to define necessary signalling to support these scenarios.

Conclusion

In this paper we provide our view on the general aspects of AI/ML Mobility use case, we particularly addressed the issue of scenarios and sub-use cases selection for AI/ML Mobility use cases:

The following Observation and Proposals were made:

Observation 1: Scenarios 1 (FR1 to FR1 intra-frequency temporal domain case A) and 5 (FR2 to FR2 intra-frequency temporal domain case B) have been identified as low priority by RAN2. Furthermore, there has not been extensive study on FR2 to FR2 inter-frequency prediction.

Proposal 1: RAN4 to align with RAN2's priority definition of use cases and initially focus on defining the performance and requirements for the high-priority scenarios, specifically FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, and FR2 to FR2 intra-frequency temporal domain case A.

Proposal 2: RAN4 to only consider defining potential requirements for scenarios 1  (i.e. FR1 to FR1 intra-frequency temporal domain case A) and scenario 2 (i.e. FR2 to FR2 intra-frequency temporal domain case B) if RAN2 agrees to define necessary signalling to support these scenarios.

- References

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, RAN#109