# 3GPP TSG-RAN WG4 Meeting #117	R4-2520735

# Dallas, US, 17th – 21st November, 2025

Source: 	vivo

Title: 	Discussion on 6G channel bandwidth

Agenda Item:	8.2.3

Document for:	Approval

Introduction

A study item on 6G radio [1][2] was approved and RAN4 6G plan and scopes for 6G topics were established [3]. In the last meeting, RAN4 commenced the discussion on system parameters topic and some high-level principle on these topics were established, including waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. The agreements for system parameters in RAN4#116bis were captured in [4]. In this contribution, we further discuss 6G channel bandwidth, including maximum and minimum channel bandwidth, numerology, FFT size and spectrum utilization.

Discussion

For maximum channel bandwidth,

For supported UE channel bandwidths, they are closely dependent with frequency ranges. In 5G NR, various channel bandwidths are supported from 3/5MHz to 100MHz in FR1, 50MHz to 400MHz in FR2-1. Different channel bandwidths can support enough flexibility for deployment and high date peak rate for 5G. In 6GR, we need to reconsider the maximum channel bandwidth and minimum channel bandwidth. For 6GR refarming bands, reusing the channel bandwidth is our preference, that is maximum 100MHz for Sub-6GHz and 3/5MHz for minimum channel bandwidth. For 6G new spectrum, U6G and FR3 have abundant spectrum, which can support more than 100MHz. We can extend 100MHz to 200MHz for these new 6G spectrum, achieving higher peak date rate for 6G and competing with 5G. For 6GR FR2-1, we think reusing channel bandwidth from 5G is enough.

RAN1 and RAN4 had parallel discussions on maximum channel bandwidth, and the following agreements were captured as follows:

Agreement from RAN1#122bis - RAN1 assumes 400MHz maximum channel bandwidth at network side and 30kHz SCS around 7GHz - Study whether and how to enable UE to support 400MHz bandwidth

Agreement from RAN4#116bis In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations.

RAN1 already assumed 400MHz maximum channel bandwidth at network side, considering 30kHz SCS around 7GHz. However, it is not clear for UE how to support such wide channel bandwidth. In RAN4, company proposed to support 400MHz using approaches e.g. 16k FFT, multi-FFT or CA for UE.

From our perspective, we think 200MHz maximum channel bandwidth is enough for UE in 6GR. In 5G NR discussion, there were assumptions that one RF chain can support 200MHz. However, we are not so sure about supporting 400MHz maximum channel bandwidth. Supporting 400MHz single carrier would put high stress on RF components like PA or BB components like ADC or DAC.

From PA perspective, there is no commercial PA can support 400MHz single carrier, although Wi-Fi 7 can support up to maximum 320MHz channel bandwidth. 400MHz in around 7GHz means 5.7% relative channel bandwidths, which may be feasible. However, we need to consider the potential performance loss achieving such wide channel bandwidth. UE may face large power off using this 400MHz Channel bandwidth. It is also hard to achieve linearity techniques e.g. DPD, ET.

From ADC/DAC perspective,400MHz may need sampling rate larger than 1GSPS according to Nyquist Theorem, however maintain higher bit ENOB, the power assumption and cost may roar high. Currently, the sampling rate higher than 1GSPS ADC/DAC was used in the BS side, however, not in UE side. We need to consider the consequences to shift this baseband capability to UE.

Generally, we prefer to use CA to achieve 2*200MHz channel bandwidth at UE side with 400MHz at network side. Carrier Aggregation (CA) is the standard technique used to achieve significantly higher bandwidths. Crucially, by aggregating multiple carriers, each remains within the established 200 MHz maximum bandwidth limit for a single user equipment (UE) operating on a single carrier. This approach is essential because it avoids the extreme complexity and cost involved if a single UE were forced to support much wider bandwidths (e.g., 400 MHz) directly on a single carrier. Specifically, it eliminates the need for a UE to implement an extremely large FFT size exceeding 8k or to develop a single, prohibitively complex and expensive Power Amplifier (PA) capable of handling 400 MHz bandwidths. CA thus delivers higher aggregate data rates while keeping individual UE implementation practical and cost-effective.

Proposal 1: Assuming 400MHz maximum channel bandwidth at network side, it is preferred to use CA to achieve channel bandwidth at UE side, for example 2*200MHz for both UL and DL.

For minimum channel bandwidth,

From our perspective, 6G minimum channel bandwidth needs to cover SSB bandwidth, which needs RAN1’s confirmation. Also, it depends on the spectrum holdings from operators. For spectrum sub-6GHz, 6G still need to utilize the 5G refarming bands which are fragmented. Minimum channel bandwidth 5MHz is still the baseline choice for normal UE, however, 3MHz may still be needed considering some low-tier device and spectrum holdings from some operators. For U6G and newly introduced FR3, we propose 20MHz as minimum channel bandwidth for the efficient use of these abundant spectrums.

In the last meeting, the following agreement was reached:

- Agreement from RAN4#116bis - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective.

Supporting 3MHz bandwidth is primarily relevant for specific non-mainstream operator scenarios. However, using it as the baseline for physical layer initial access is generally not recommended. This approach introduces performance degradation and is considered suboptimal. Therefore, we maintain a strong preference for using 5MHz as the standard access bandwidth, balancing efficiency and broader compatibility.

Proposal 2: For 6GR FR1, it is proposed to adopt minimum channel bandwidths 5MHz for initial access.

Proposal 3: For operators those has only 3MHz spectrum resources, RAN1 may need to specify a mechanism to support the operation of 3MHz spectrum usage.

Numerology and FFT size:

Numerology:

As usual, 6GR numerology is within RAN1’s scope. The agreement on 6GR numerology from RAN1#112 was captured as follows. Many RAN4 discussion is tightly relying on the decisions on 6GR numerology. We can take below agreements as working assumptions in RAN4.

Agreements from RAN1#112:

In the last meeting, RAN4 also had the discussion on numerology

Agreement from RAN4#116bis - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

In current NR bands, multiple numerologies were defined, 15/30/60 for FR1 and 60/120/240 for FR2-1. However, in real deployment, only one numerology is adopted for a specific band. Avoiding this redundancy and complexity in 6GR, it is proposed to support only one numerology for a specific frequency range/ band.

Lessons learned from 5G is that, although different numerologies can be supported in the specification, only one numerology is used for a specific band. The specification can be simplified for numerology. In 5G sub-6G, 15kHz SCS was used for FDD bands and 30kHz SCS was used for TDD. We suggest to adopt these numerologies for 6G. For around 7GHz, RAN1 adopted 30kHz SCS as the numerology.

Proposal 4: In 6G FR1, it is suggested to adopt 15kHz for FDD bands and 30kHz for TDD bands.

FFT size is tightly dependent on the number of RE for the specific channel bandwidth. For example, if we consider 100MHz with 30kHz, the total RE number would be 3276 (273*12), then the best suitable FFT size is 4096. For sub-6GHz, we think FFT size 4096 is OK if we consider maximum 100MHz channel bandwidth. However, it may not be enough if we consider 200MHz for around 7GHz and FR3. The RE number would be doubled for 200MHz, which is approximately 2*3276. In this case, FFT size 8192 can be used.

Agreement - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

Proposal 5:

For Sub-6GHz, maximum FFT size 4096 is suggested for maximum 100MHz with 30kHz SCS;

For FR1 U6G, maximum FFT size 8192 is suggested for the expected maximum 200MHz with 30kHz.

Spectrum Utilization

The following agreement for spectrum utilization was reached in the last meeting:

- Agreement from RAN4#116bis - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations.

In RAN4 discussion, spectrum utilization means the maximum transmission bandwidths NRB or the percentage of the maximum transmission for a given channel bandwidth. For 5G refarming bands to 6G, reusing current SU for existing channel bandwidths can be a baseline. For newly introduced channel bandwidths for 6G, it needs further discussion.

In 5G NR, FR1 can achieve 95%~98% spectrum utilization for CBW larger than 50MHz and 95% spectrum utilization for FR2-1, as shown in below figures. In order to keep the same high level with 5G NR, current spectrum utilization can be used as a starting point for existing channel bandwidths in 6GR refarming bands. For newly introduced channel bandwidth e.g. 200MHz for U6G and FR3, we think spectrum utilization evaluation is needed based on RF requirements (SEM/EVM/ACLR) defined for 6GR, which will be discussed in 6GR RAN4 RF part.

Figure 1. Spectrum Utilization for FR1

Figure 2. Spectrum Utilization for FR2-1

For 6G SU evaluation, we summarized all these assumptions used for SU evaluation in FR1 from UE perspective.

Table 1. SU simulation assumptions from UE perspective in FR1

| CHBW and SCS set | For 15kHz SCS, 3MHz, 5MHz, 10MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 45MHz, 50MHz can be evaluated; For 30kHz SCS, 5MHz, 10MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 45MHz, 50MHz, 60MHz, 70MHz, 80MHz, 90MHz, 100MHz, 200MHz. For 60kHz, it is suggested not to consider it. FFS：the channel bandwidths between 100MHz~200MHz |  |
| --- | --- | --- |
| PA aspects | Power class | PC3 23dBm PC2 26dBm |
|  | PA models | - reusing 5G PA - wait for 6G PA model |
| RF impairments | carrier leakage | -28dBc |
|  | I/Q imbalance | -28dBc |
|  | phase noise | FFS |
| Baseline RF requirements | SEM | 3-100MHz SEM: TS 38.101-1 Table 6.5.2.2-1; For channel bandwidths larger than 100MHz: FFS |
|  | ACLR | PC2：31 dB PC3: 30 dB |
|  | EVM | e.g. QPSK 17.5% |
| Note: Above simulation assumptions may need further update according to 6G study. |  |  |

To evaluate spectrum utilization, we propose a step-by-step approach to simplify the process. Currently, many interrelated factors—such as new waveforms, PA models, and advanced spectral confinement techniques—complicate the analysis when considered simultaneously. Therefore, as the first phase, we recommend conducting simulations for fundamental spectrum utilization using baseline PA models and waveforms. As the next phase, we can then incorporate advanced spectral confinement techniques to evaluate their impact on spectrum utilization improvement.

Proposal 6: As the first phase, we recommend conducting simulations for fundamental spectrum utilization using baseline PA models and waveforms. As the next phase, we can then incorporate advanced spectral confinement techniques to evaluate their impact on spectrum utilization improvement.

Proposal 7: It is suggested to take Table 1 as baseline for SU simulation assumptions from UE perspective and further clarify each item.

We suggestion that RAN4 initiates its evaluation of FDSS by first studying schemes transparent to RAN1, positioning FDSS as a baseline technology to quantify potential gains in Spectral Utilization (SU). Importantly, a fundamental shift in objectives must be acknowledged: while FDSS studies in 5G NR primarily targeted MPR reduction, current 6G research positions FDSS as a key enabler for SU improvement. This divergence creates a critical need for RAN4 to conduct a comprehensive trade-off analysis, weighing these distinct priorities (e.g. MPR reduction vs. SU gain), coordinating unified evaluation methodologies, and defining harmonized success criteria to guide the technology's path forward in 6G standardization.

Proposal 8: It is suggested RAN4 to conduct a comprehensive trade-off analysis for SU improvement using advanced spectral confinement techniques.

Conclusion

This contribution discusses channel bandwidth, FFT, numerology, and spectrum utilization for 6GR. The following proposals are made:

Proposal 1: Assuming 400MHz maximum channel bandwidth at network side, it is preferred to use CA to achieve channel bandwidth at UE side, for example 2*200MHz for both UL and DL.

Proposal 2: For 6GR FR1, it is proposed to adopt minimum channel bandwidths 5MHz for initial access.

Proposal 3: For operators those has only 3MHz spectrum resources, RAN1 may need to specify a mechanism to support the operation of 3MHz spectrum usage.

Proposal 4: In 6G FR1, it is suggested to adopt 15kHz for FDD bands and 30kHz for TDD bands.

Proposal 5:

For Sub-6GHz, maximum FFT size 4096 is suggested for maximum 100MHz with 30kHz SCS;

For FR1 U6G, maximum FFT size 8192 is suggested for the expected maximum 200MHz with 30kHz.

Proposal 6: As the first phase, we recommend conducting simulations for fundamental spectrum utilization using baseline PA models and waveforms. As the next phase, we can then incorporate advanced spectral confinement techniques to evaluate their impact on spectrum utilization improvement.

Proposal 7: It is suggested to take Table 1 as baseline for SU simulation assumptions from UE perspective and further clarify each item.

Table 1. SU simulation assumptions from UE perspective in FR1

| CHBW and SCS set | For 15kHz SCS, 3MHz, 5MHz, 10MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 45MHz, 50MHz can be evaluated; For 30kHz SCS, 5MHz, 10MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 45MHz, 50MHz, 60MHz, 70MHz, 80MHz, 90MHz, 100MHz, 200MHz. For 60kHz, it is suggested not to consider it. FFS：the channel bandwidths between 100MHz~200MHz |  |
| --- | --- | --- |
| PA aspects | Power class | PC3 23dBm PC2 26dBm |
|  | PA models | - reusing 5G PA - wait for 6G PA model |
| RF impairments | carrier leakage | -28dBc |
|  | I/Q imbalance | -28dBc |
|  | phase noise | FFS |
| Baseline RF requirements | SEM | 3-100MHz SEM: TS 38.101-1 Table 6.5.2.2-1; For channel bandwidths larger than 100MHz: FFS |
|  | ACLR | PC2：31 dB PC3: 30 dB |
|  | EVM | e.g. QPSK 17.5% |
| Note: Above simulation assumptions may need further update according to 6G study. |  |  |

Proposal 8: It is suggested RAN4 to conduct a comprehensive trade-off analysis for SU improvement using advanced spectral confinement techniques.

- References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN#108.

[2] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone.

[3] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, RAN4#116.

[4] R4-2514641, WF for 6GR system parameter, Feature lead (Huawei, HiSilicon), RAN4#116bis.