3GPP TSG-RAN WG4 Meeting #117			                        R4-2520756

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.2.4

Source: 	CMCC

Title: 	Discussion on R20 SBFD adjacent channel coexistence

Document for:	Discussion

# 1. Introduction

The discussion on SBFD at the last meeting mainly focused on the SBFD sub-band configuration, and no specific conclusion was reached. The discussions on SBFD conducted in R18 and R19 can serve as a good reference for the discussions in R20.

This contribution will mainly focus on issues such as bandwidth and configuration in the SBFD-SBFD scenario and the viewpoints of the operators.

- Discussion

### 2.1 SBFD configuration

During last we have achieved the following agreement.

Agreements: - For next meeting further consider following options, with the intention to down-select: Option 1: UD (Operator 1) and DU (Operator 2) Option 2: DUD (Operator 1 and 2) Option 3: UD and DUD Provide more description on each specific case. - As baseline use simulation parameters defined in TR 38.858. For 6 GHz, new parameters for BS antenna should be established for SBFD.

Based on the conclusion of the last meeting, three candidate scenarios were listed. From our perspective, all three scenarios have the possibility of actual deployment. However, coexistence RF requirements often consider the worst case. Take band n104 as an example, it has a bandwidth of 700 MHz, if there are multiple operators in a certain area and each operator could be allocated 100 MHz, there will be a situation where three adjacent operators jointly deploy SBFD.

Observation 1: On band n104, there is a situation where multiple adjacent operators jointly deploy SBFD.

In this context, taking Option 1 as an example, as shown in Figure 1, suppose operator deploys SBFD (UD), adjacent operator 2 deploys SBFD (DU), and another operator 3 adjacent to operator 2 also deploys SBFD. At this time, the situation where operator 2 and operator 3 are adjacent is not reflected in the above three options. In this case, if operator 3 deploys DU or DUD, the UL sub-band of operator 2 and the DL sub-band of operator 3 are very close to each other. This scenario should be considered.

Figure 1. Operator 1 and Operator 2 deployed SBFD as UD-DU

Therefore, we suggest considering UD-UD cases. In this scenario, even if there are multiple operators in this frequency band, all the operators could adopt the UD sub-band configuration to achieve the joint deployment of SBFD on this frequency band.

Figure 2. Operator 1, Operator 2 and Operator 3 deployed SBFD as UD subband configuration

Proposal 1: It is suggested to study the scenarios of UD (Operator 1) and UD (Operator 2).

Both the co-located and non-co-located scenarios have been taken into consideration. In the Rel-19 study, the main focus was on the non-collocated scenario, but the co-located scenario is particularly important for operators, so the priority of the co-located scenario in this WID should be higher.

Proposal 2: The priority of the co-located scenario could be higher.

### 2.2 SBFD bandwidth

In China, the frequency band n41 is only operated by CMCC. Therefore, from our perspective, the priority for conducting coexistence analysis in the SBFD-SBFD scenario for n41 is relatively low. As for band n79, the Radio regulatory have authorized a total of 160 MHz frequency band. As shown in Figure 1, frequency range from 4800 MHz to 4900 MHz were allocated to China Mobile, while the frequency range from 4900 MHz to 4960 MHz were allocated to China Broadcasting Network. Therefore, an additional analysis of the coexistence of 100 MHz SBFD and 60 MHz SBFD is required. Therefore, it is suggested that for band n79, the scenario of 100MHz SBFD being adjacent to 60MHz TDD should be studied.

Figure 3. Spectrum at band n79 in China

Proposal 3: It is suggested to analysis the coexistence of 100 MHz SBFD and 60 MHz SBFD for band n79.

Band n104 has a large bandwidth of 700 MHz. Although the WRC-23 has completed the allocation of this frequency band, there is no operator has been licensed to use the spectrum on this band yet. Nevertheless, it can be predicted that this frequency band is likely to be allocated to multiple operators. At the same time, due to the large bandwidth of this band, even in the case of multi-operators, each operator may still be allocated a 100 MHz bandwidth band. Considering all these factors, when SBFD is widely deployed, a SBFD-SBFD coexistence scenario is very likely to occur on this frequency band. Therefore, in our opinion, the research on band n104 has a relatively high priority.

Observation 2: Band n104 is likely to be allocated to multiple operators.

Proposal 4: Band n104 has a high priority and the scenarios that 100 MHz SBFD and 100 MHz SBFD are adjacent could be considered.

# 3. Conclusions

Observation 1: On band n104, there is a situation where multiple adjacent operators jointly deploy SBFD.

Proposal 1: It is suggested to study the scenarios of UD (Operator 1) and UD (Operator 2).

Proposal 2: The priority of the co-located scenario could be higher.

Proposal 3: It is suggested to analysis the coexistence of 100 MHz SBFD and 60 MHz SBFD for band n79.

Observation 2: Band n104 is likely to be allocated to multiple operators.

Proposal 4: Band n104 has a high priority and the scenarios that 100 MHz SBFD and 100 MHz SBFD are adjacent could be considered.

# 4. Reference

- R4-2515091, Way-Forward for NR_BS_RF_Ph2, Ericsson