3GPP TSG-RAN WG4 Meeting#117		R4-2520762

Dallas, USA, Nov 17th - 21th, 2025

Agenda item:	8.2.2

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G modulation

Document for:	Discussion

Introduction

In RAN4#116bis meeting, a WF [1] about RAN4 6G system parameters was approved. In this contribution, we want to continue to provide some views on 6G modulation.

Discussion

Modulation

According to the agreements in RAN1#122bis meeting, uniform 4096 QAM for DL and uniform 1024QAM for UL need to study.

Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation.

Based on the agreements in RAN4#116bis meeting, RAN4 need to focus on the feasibility study and RF requirements.

- Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

Higher order modulation

For higher modulation, from RAN4 perspective, for 1024QAM in uplink, EVM requirement need to study. EVM is related with SNR, besides, EVM is related with RF impairments, which include IQ mismatch, phase noise, and RF nonlinearity. Based on WIFI 7 spec requirements in Table 1, when UE supports 1024QAM UL, relative constellation error is -35dB, which means that SNR needs to be higher than 35 dB. The EVM requirement needs to be less than 1.7%. In addition, we need to consider the interference and multipath fading in real scenarios. Compared to 256QAM for UL, the maximum throughput can be increased by 25%, however, we need additional 6dB SNR to achieve 1024QAM for UL. PAPR may to be higher than 10dB, it will decrease the coverage range. Correspondingly, power consumption will be increased significantly. It is a challenge for UE implementation. Based on the analysis, to evaluate 1024 QAM for UL, EVM requirement, RF impairments (IQ mismatch, phase noise, RF nonlinearity), PAPR requirements, MPR, and throughput for coverage are all need to be considered.

Table1: relative constellation error in WIFI 7 spec

Observation1: Based on the WIFI 7 spec, the required RCE (~EVM) is -35dB (1.7%) for 1024QAM.

In this paper, it is observed that with higher order modulation, which is more sensitive to Tx impairments. Fgiure1 and Figure2 show that the results about the performance of phase noise and frequency offsets to 1024QAM.

Figure1: BER of 1024-QAM with different frequency offsets.                         Figure2: BER of 1024-QAM with different frequency offsets.

Observation2: 1024QAM is more sensitive to frequency offset and phase noise compared to 256QAM.

Proposal 1: To evaluate 1024 QAM for UL, from RAN4 perspective, EVM, RF impairments (IQ mismatch, frequency offset, and phase noise), PAPR and MPR needs to be considered.

5G supported modulation

For existing supported modulation in 5G, new spectrum and new PA model can be taken into consideration to evaluate RF requirements needs to be updated or not in 6G. However, new PA model is under discussion. We can wait for the conclusion about new PA model.

Proposal 2: We can wait for the conclusion about the new PA model to evaluate existing supported modulation for 6GR.

Conclusions

In this paper, we provide our views on 6G modulation, we have made the following proposals and observations.

Observation1: Based on the WIFI 7 spec, the required RCE (~EVM) is -35dB (1.7%) for 1024QAM.

Observation2: 1024QAM is more sensitive to frequency offset and phase noise compared to 256QAM.

Proposal 1: To evaluate 1024 QAM for UL, from RAN4 perspective, EVM, RF impairments (IQ mismatch, frequency offset, and phase noise), PAPR and MPR needs to be considered.

Proposal 2: We can wait for the conclusion about the new PA model to evaluate existing supported modulation for 6GR.

- Reference

- R4-2514650, WF on 6GR system parameter, Huawei, HiSilicon. TSG-RAN4#116bis meeting

- Bojun Sun and Ismail Guven, “Performance of Higher Order Modulations under Different Impairments”. 978-1-4799-7521-1/15/$3l.00 ©20 15 IEEE