# 3GPP TSG-RAN WG4 Meeting #117                               R4-2520773

# Dallas, US, 17th-21st Nov., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on UE RF requirements for Rel-20 UAV

# Agenda Item:   7.7.4

# Document for:  Approval

- Introduction

In last meeting, RAN4 didn’t have enough time to discuss UE RF related issues for Rel-20 UAV, but some agreements on high level issues had been approved in Way Forward [1] as listed following.

Issue 3-1-1: General aspects - Agreements: - The high-power UAV UE applies for the listed example bands in Rel-20 WID. - RF requirements need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors. - The same UE can support either omni-directional antenna and phase array antenna. Issue 3-2-4: Transmit signal quality - Agreements: - As a starting point, the legacy frequency error and transmit modulation quality requirements can be reused for high power UAV. Issue 3-2-5: Output RF spectrum emissions - Agreements: - Take the following conclusions for output RF spectrum emissions as the starting point. Requirements; Conclusions. Occupied bandwidth; Legacy requirements can apply to high power UAV UE.. SEM requirements; The discussion on SEM can be held until ACLR is concluded.. ACLR requirements; RAN4 needs to evaluate ACLR for high power UAV based on co-existence analysis.. Spurious emissions; Legacy requirements can apply to high power UAV UE.. Spurious emissions for UE co-existence; Legacy requirements can apply to high power UAV UE.. Transmit intermodulation; FFS on whether this requirement is needed or not for high power UAV UE..

In this contribution, we provided our views on UR RF requirements for high power UAV.

- Discussion

## 2.1 Maximum output power

For maximum output power (MOP), as mentioned in WID[2], the MOP is up to 33dBm, which is not matched with current power classes. Some companies proposed to introduce a new power class for UAV UE for up to 33dBm MOP for the example bands. Actually, in HPUE topic, companies are discussing whether to introduce a new power class with 32dBm MOP. Frankly speaking, if every time when there is new MOP level and we introduce a new power class, this is a huge workload in RAN4 because a series of requirements such as ACLR, MPR/A-MPR, REFSENS etc. Also, companies are considering to optimize current power class framework in 6GR. We don’t think it is an efficient way to introduce a new power class for UAV.

Similar issue happened in ATG UE in Rel-18, where a capability for ATG UE was introduced to report the rated maximum output power at maximum modulation order and full PRB configurations. The capability report granularity is 1dB and the power range is 23dBm to 40dBm. We suggest high power UAV to follow the same method as ATG. The lower limit of conductive MOP of high power UAV could be 23dBm, and the upper limit of conductive MOP of high power UAV could be 33dBm.

Proposal 1: Introduce the new capability for high power UAV to indicate the rated maximum output power.

With this capability, MPR/AMPR requirements are not needed as UAV can declare a lower power whenever power back-off is needed. Then the UAV can set its configured maximum output power based on NW indication and UAV capability.

Proposal 2: If the rated maximum output power is reported by UAV, no MPR or A-MPR requirements are needed.

## 2.2 Output dynamic range

For output dynamic range, current transmit OFF power, transmit ON/OFF time mask and power control can also apply to high power UAV, while minimum output power needs to be evaluated.

Proposal 3: Transmit OFF power, transmit ON/OFF time mask and power control can apply to high power UAV.

In last meeting, RAN4 has agreed that RF requirements of the UAV need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors. Then RAN4 needs to evaluate the minimum output power for high power UAV equipped with omni-directional antenna and directional antenna array, considering different center frequency, pass loss and antenna gain. Compared to ATG UE, the main difference is the distance between UAV and UAV BS is much closer. A preliminary link calculation on minimum output power is listed in Table 1.

Table 1  The link calculation for minimum output power requirements

| Parameters | Omni-directional antenna | phased array antenna |
| --- | --- | --- |
| center frequency GHz | 2 | 4 |
| Distance meter | [200] | [200] |
| Pathloss dB | 84.49 | 90.51 |
| UAV BS antenna gain | 21 | 21 |
| UAV antenna gain | 0 | [6] |
| Thermal noise power density dBm/MHz | -114 | -114 |
| UAV BS Noise figure dB | 5 | 5 |
| Minimum output power dBm/MHz | -45.51 | -45.49 |
| Minimum output power dBm/5MHz | [-38.52] | [-38.50] |

Proposal 4: The minimum output requirements could be specified as listed in Table 2.

Table 2  Minimum output power requirements for UAV

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | X | X+10log10 (BWChannel /20) | X+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE 1: The minimum output power value is rounded to the nearest number down to one decimal point. NOTE 2: X = [-38] for the UAV with both omni-directional antenna and with phased array antenna. |  |  |  |  |

## 2.3 Output RF spectrum emissions

In last meeting, the agreements on output RF spectrum emissions are that legacy occupied bandwidth, SEM, ACLR, SE,SE for UE co-existence requirements can apply to high power UAV and FFS on whether transmit intermodulation requirement is needed or not for high power UAV. It is observed that no transmitted intermodulation requirements were specified for either Rel-18 UAV or ATG UE. Terrestrial interference scenario is different from scenarios over the air. This requirement is not applicable to high power UAV either.

Observation 1: Transmit intermodulation requirement was not specified for either ATG UE or Rel-18 UAV.

Proposal 5: Transmit intermodulation requirement is not applicable to high power UAV.

- Conclusion

Proposal 1: Introduce the new capability for high power UAV to indicate the rated maximum output power.

Proposal 2: If the rated maximum output power is reported by UAV, no MPR or A-MPR requirements are needed.

Proposal 3: Transmit OFF power, transmit ON/OFF time mask and power control can apply to high power UAV.

Proposal 4: The minimum output requirements could be specified as listed in Table 2.

Table 2  Minimum output power requirements for UAV

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | X | X+10log10 (BWChannel /20) | X+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE 1: The minimum output power value is rounded to the nearest number down to one decimal point. NOTE 2: X = [-38] for the UAV with both omni-directional antenna and with phased array antenna. |  |  |  |  |

Observation 1: Transmit intermodulation requirement was not specified for either ATG UE or Rel-18 UAV.

Proposal 5: Transmit intermodulation requirement is not applicable to high power UAV.

- Reference

- R4-2515159 Way Forward for [116bis][319] NR_UAV, Qualcomm, CMCC, RAN4#116bis, Prague, CZ, Oct. 2025.

- RP-252954 New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), RAN#109 Beijing, China, Sept. 2025.