# 3GPP TSG-RAN WG4 Meeting #117	R4-2520807

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.12.2.2

Source:	China Telecom

Title:	Discussion on RRM spec for RAN4 6G specification enhancement

Document for:	Discussion

- Introduction

In RAN4 #116bis meeting, 6G RAN4 operation efficiency were discussed and the WF [1] was approved.

In this paper, we present our views on RRM spec for RAN4 6G specification enhancement captured in the WF [1].

- Discussion

General potential improvement directions

Status in the WF [1]:

Issue 1-1-3: Issues on RRM Specifications - Observed issues - Issue 1: Redundancy - The duplicated requirement in TS 38.133 leads to confusion and unnecessary extra effort for spec maintenance. - Many instances of redundancy in TS 38.133 for NR, with repeated definitions and tables appearing throughout the specification. - Duplicated clauses exist in TS 38.133. - Duplicated requirements scattered throughout different sections. - Many configuration parameters in test cases are repeated in RRM specs - Issue 2: Cases and scenarios - When defining RRM requirements there are multiple cases and scenarios involved even when they are rarely deployed. - Issue 3: Readability - Measurement requirements categorized by intra/inter-frequency are not efficient in TS 38.133. - Readability and friendliness issues in TS 38.133. - Unclear or non-straightforward relation between requirements and test cases in RRM specs. - Issue 4: Test cases - Test cases exist in the top-level heading in RRM specs

The RRM spec TS 38.133 is defined with requirements for support of radio resource management and all the RRM related core and performance requirements are included. Consequently, the RRM spec TS 38.133 has become a massive specification, packed with different features, cases, scenarios, etc.

For different features, cases, scenarios, etc., the requirements may be similar with each other, however, duplicated requirements are defined for different features, cases, scenarios, etc. Therefore, the issues on redundancy needs to be considered.

In RRM spec, one feature related requirements are defined in different clauses, core requirements, performance requirements and test cases have no relation instruction, which causes the issue on readability, e.g., many new RRM delegates cannot have a good understanding for the requirements on a specific feature and the relation of requirements.

There are many requirements for different features, cases, scenarios, etc. defined in RRM spec, however, not all cases and scenarios are the typical use cases, and not all cases and scenarios are deployed in real network, we observe that many corner cases are defined in RRM spec, so avoiding defining corner cases is a critical issue.

Proposal 1: The issues of Redundancy, Readability and avoiding corner cases shall be considered for 6G RRM Specification.

Potential improvements for RAN4 RRM specifications

Status in the WF [1]:

Issue 3-2-1: General principles and targets - Proposals - Proposal 1: - Proposal 1a: Follow agreements in R4-2420107 for better readability. - Proposal 1b: The agreements in R4-2420107 should be treated as a starting point for 6G RRM spec. - Proposal 2: Consider 6GR specification drafting rules (e.g. overall spec structure, hierarchy of indent, suffix rule, etc.). - Proposal 3: Enforcement of drafting rules: only CRs following drafting rules can be agreed. - Proposal 4: Differentiate whether the difference between two requirements is due to editorial issue or technical issue. - Proposal 5: For spec and CR drafting rules, follow agreed principles as in 5G RAN4 Meeting Efficiency Improvements (R4-2114691), i.e., big CR approach. - Proposal 6: Study further drafting rules to improve readability. - Proposal 7: - Proposal 7a: Avoid corner cases and focus on most typical and practical use cases. - Proposal 7b: Focus on enhancing the testing of RRM procedures to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios. - Proposal 7c: Study how to ensure that real UE implementations are tested as much as possible. - Proposal 7d: Discuss whether to define two threads requirements, one focus on the baseline and another focus on the strict performance with real field request. - Proposal 7e: Study how to define RRM requirements that allow UE implementation based on minimum requirements but also allow UEs that can outperform the minimum requirements the benefits from such better performance - Proposal 8: Consider a template for requirements. - Proposal 9: Use the following aspects as start point: - Reuse the Big CR procedure and RAN4 Chair and MCC’s rules of Big CR: no [], TBD, FFS clean up in the Big CR and specs. - Reuse the rules of “Forward section” to ensure consistent usage of frequently used terms, notation, abbreviations, CA configuration vocabulary, etc. - For new features, determine the common rule of whether to add a new sub-clause. If new sub-clauses are introduced: - Clearly declare the numbering corresponding to a feature in an appendix or designated location. - For situations where similar text needs to be repeated across multiple sections (or specifications), the general text should first be agreed upon as a reference and then used across different sections/CRs/specifications to improve consistency. - Proposal 10: Study at least following aspects or RRM spec improvement in 6GR - Better classification of L3 RRM measurement requirements - Consistent principles to address different collisions - Proposal 11: - Introduce a more intuitive and simpler way to define RRM requirements, and new forms of representation if necessary. - Adopt a more unified form to manage similar parameters and simplify as much as possible, avoiding the introduction of too many parameters with similar meanings and functions. - Proposal 12: Target for next generation technique innovation on RRM should be more efficiency and less energy consumption, higher throughput and less interruption. - Proposal 13: clear scope and goal / direction to be discussed /derived during 6G SI phase. - Proposal 14: Comprehensively consider the RRM design in 6G with some high-level principles: - Measurement bandwidth/Rx number vs implementation complexity - Power saving vs always-on RF chain - Gap-less design vs the number of searcher/idle RF chain - Measurement period vs measurement accuracy - TN&NTN integration vs implementation complexity - Proposal 15: Even without AI/ML-based operation, the 6G RRM specifications must provide improvements over NR - Proposal 16: Strive to establish quantifiable requirements to avoid the vague specification.

For potential improvements for RAN4 RRM spec, the principles need to be discussed firstly. The RRM spec improvements have been discussed in Rel-19 and the agreements have been achieved in R4-2420107, in our views, the agreements in R4-2420107 can be treated as a starting point for 6G RRM spec.

For RAN4 6G RRM spec, aligning the drafting rules shall be considered early, then the 6G RRM spec can be structured with the aligned drafting rules. Drafting rules including the spec structure, hierarchy of indent, suffix for features, unified/common configuration, etc., can be further discussed.

Considering the lessons from 5G standardization, it’s observed that many corner cases are defined in RRM spec, therefore, the cases and scenarios need to be carefully checked and the requirements on corner cases shall not be defined, we should focus on most typical and practical use cases.

Proposal 2: For general principles and targets, it’s proposed to consider the following aspects.

- The agreements in R4-2420107 can be treated as a starting point for 6G RRM spec.

- Consider 6GR specification drafting rules (e.g. spec structure, hierarchy of indent, suffix for features, unified/common configuration, etc.).

- Avoid corner cases and focus on most typical and practical use cases.

Status in the WF [1]:

Issue 3-2-2: Structural Options and Specification Organization - Proposals - Proposal 1: High-level structure for new 6G RRM specs - Proposal 1a: RAN4 to discuss the following for new 6G RRM specs: - a. High level structure (highest level sections: Idle, Inactive etc.) - b. UE requirements for a scalable 6G design - c. Any gain in further splitting the specification - d. Test case mapping (e.g. TC reference in core part) - Proposal 1b: Consider the following framework (preferred): - RRC_IDLE/INACTIVE state mobility - RRC_CONNECTED state mobility - Timing - Measurement procedure for RRC_CONNECTED state - RLM/BFD/CBD - PSCell/SCell management (if applicable by PHY/high layer design in 6GR) - Other UE-specific characteristic switching (if applicable by PHY design in 6GR) - Measurement performance - Proposal 1c: - For the 6G RRM spec, top level of sections 4, 5, 6, 7, 9, 10 in TS 38.133 can be reused. - Section 8 is used to capture procedure delay related requirements. - Scheduling restriction related requirements and interruption requirements are captured in one high-level section. - L1 measurement requirements (incl. RLM and link recovery) and L3 measurement requirements are captured in one high-level section. - Some distinct features (e.g. sidelink requirements) can be captured in a separate section. - Proposal 1d: The following skeleton as a starting point: - Requirements for RRC_Idle/Inactive - Requirements for RRC_Connected - Requirements for timing signal - Requirements for the measurement procedure - Reconsider if there should be RRC_INACTIVE. - Second-level heading to be discussed further: - More UE states - Intra/inter-frequency requirements separation - RRM unified requirement framework. - Proposal 1e: Specification structure for 6G test cases is similar to the requirements structure in the main part of the specification (at least top level, maybe second level). - Proposal 1f: the overall spec structure in 5GNR can be inherited such as: RRC_IDLE/INACTIVE/ CONNECTED state mobility, Timing, Signaling, Measurement. etc. - Decide the high-level principle to decide whether a new feature is introduced, new sub-clauses can be allowed or not. Preferable to category the clauses from procedures and different assumptions rather than UE types. - Even the named of procedures are the same as in 5GNR, it doesn’t mean the exactly same RRM requirements in 5GNR will be reused. - Achieve the common assumption of each component for different UE capabilities, including assumption of RF and BB processing, like: RF retuning time, AGC time, time for change bandwidth, time for BB processing, T/F tracking, number of searchers, etc - Proposal 2: The following alternatives can be considered in 6G to improve spec readability: - Option 1: a single spec for all UE features - Option 2: different sub-specs for common features and other vertical UE features (e.g. sidelink, NTN) - Option 3: different sub-specs for core, performance, TC separately. - Proposal 3: The basic functionalities and prioritize 6G day-1 typical cases’ requirements. - Proposal 4: Avoid duplication and repetition of UE requirements for different scenarios and use cases. - Proposal 5: - Proposal 5a: RAN4 to study how new features introduced in 6G are captured in RRM requirements specification. - Proposal 5b: New features introduced in later releases are captured either by incorporating in existing sections or by creating new sub-sections. Same suffix should be used for the same feature in different sub-sections. - Proposal 6: - Proposal 6a: Consider split RRM spec into two files for core part and performance part, respectively. - Proposal 6b: Avoid defining too huge spec like 38.133; consider splitting the 6G RRM spec into several specs covering core requirements, performance requirements, and test cases respectively.

For 6G RRM spec structure, high-level structure for new 6G RRM spec need to be discussed, e.g., which sections can be inherited from 5G NR RRM spec TS 38.133, which sections need to be updated and added compared with 5G NR RRM spec TS 38.133.

Another important thing on 6G RRM spec structure is whether to split core requirements, performance requirements, test cases into several specs.

Proposal 3: For Structural Options and Specification Organization, it’s proposed to consider the following aspects.

- High-level structure for new 6G RRM spec need to be discussed, e.g., which sections can be inherited from 5G NR RRM spec TS 38.133, which sections need to be updated and added compared with 5G NR RRM spec TS 38.133.

- Whether to split core requirements, performance requirements, test cases into several specs.

Status in the WF [1]:

Issue 3-2-3: Drafting approach - Proposals - Proposal 1: Adopt RAN2 pseudo-code approach in all sections. - Proposal 2: Uses block-based approach to define core requirements and test cases. - Proposal 3: A new tool, if possible, is used to capture tabulated test setup in test cases.

For Drafting approach, the agreements in R4-2420107 can be treated as a starting point for 6G RRM spec. In R4-2420107, it was agreed to adopt RAN2 pseudo-code approach for selected Rel-19 features with independent clauses, and put conditions before requirements. In our views, for 6G RRM spec, pseudo-code approach shall be adopted in all sections.

Proposal 4: For Drafting approach, pseudo-code approach shall be adopted in all sections.

Status in the WF [1]:

Issue 3-2-4: Readability and simplification - Proposals - Proposal 1: simplification on core requirements and tests by considering real demands, typical scenarios and realistic UE implementation - Proposal 1a: Study how to reduce the amount of RRM procedure and requirements are defined only for key RRM procedures. - Proposal 1b: Re-evaluate existing RRM core and performance requirements whether they still reflect state-of-the art UE implementations. - Proposal 1c: Study in the 6G study item whether the amount of RRM procedures can be reduced. Requirements should only be defined for key RRM procedures. - Proposal 1d: Investigate how the network can be enabled to follow true UE performance in its RRM procedures instead of assuming that all UEs just support minimal requirements. - Proposal 1e: Define necessary RRM requirements for key features and procedures. It is not mandatory to define RRM requirements for all features and procedures. To consider by two criteria: - Must to have actual impacts and guidance on implementation design. - Must to be tested and testable in conformance testing. - Proposal 1f: the feature with market demand are supported from 6G day-one. - Proposal 1g: Defining standardized and meaningful measurement metrics for 6G from day one, ensuring that these metrics are clearly specified and consistently implemented across vendors. - Proposal 1h: Define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature. - Proposal 2: - Proposal 2a: Include references or mapping tables in the core part requirements that point to the relevant test cases in 6G. - Proposal 2b: Include a reference to the corresponding test cases in the corresponding requirement clause (e.g., in the text or as a new sub-clause). - Proposal 2c: Test cases for specific applications or use cases can be in a separate section, but without breaking the main structure and mapping between core requirements and test cases: - Option 1: use cases added as separate sections after the main test case hierarchy. - Option 2: use cases grouped at a second level of the test case hierarchy. - Proposal 2d: If common configurations can be identified for different test cases, they can be collected in a common section (e.g. under A.3 RRM test configurations or similar). - Proposal 3: Reduce redundancy by referencing common descriptions instead of repeating descriptions in multiple places. - Proposal 4: Editorial modifications in NR can be used as the baseline for future optimization toward 6G. - Proposal 5: Study any changes to the synchronization signal design and other related reference signals and the impact on cell detection and measurement requirements.

For simplification, it’s important to consider real demands, typical scenarios and realistic UE implementation, and corner cases should not be defined. For readability, the relation between core requirements, performance requirements and test cases should be included, e.g., references or mapping tables in the core part requirements that point to the relevant test cases. Besides, the duplicated requirements and configurations should be avoided, e.g., referencing common descriptions instead of repeating descriptions in multiple places.

Proposal 5: For Readability and simplification, it’s proposed to consider the following aspects.

- Simplification on core requirements and tests by considering real demands, typical scenarios and realistic UE implementation.

- Include references or mapping tables in the core part requirements that point to the relevant test cases in 6G.

- Reduce redundancy by referencing common descriptions instead of repeating descriptions in multiple places.

- Conclusion

The contribution presents our views on RRM spec for RAN4 6G specification enhancement, with the following proposals:

Proposal 1: The issues of Redundancy, Readability and avoiding corner cases shall be considered for 6G RRM Specification.

Proposal 2: For general principles and targets, it’s proposed to consider the following aspects.

- The agreements in R4-2420107 can be treated as a starting point for 6G RRM spec.

- Consider 6GR specification drafting rules (e.g. spec structure, hierarchy of indent, suffix for features, unified/common configuration, etc.).

- Avoid corner cases and focus on most typical and practical use cases.

Proposal 3: For Structural Options and Specification Organization, it’s proposed to consider the following aspects.

- High-level structure for new 6G RRM spec need to be discussed, e.g., which sections can be inherited from 5G NR RRM spec TS 38.133, which sections need to be updated and added compared with 5G NR RRM spec TS 38.133.

- Whether to split core requirements, performance requirements, test cases into several specs.

Proposal 4: For Drafting approach, pseudo-code approach shall be adopted in all sections.

Proposal 5: For Readability and simplification, it’s proposed to consider the following aspects.

- Simplification on core requirements and tests by considering real demands, typical scenarios and realistic UE implementation.

- Include references or mapping tables in the core part requirements that point to the relevant test cases in 6G.

- Reduce redundancy by referencing common descriptions instead of repeating descriptions in multiple places.

- References

- R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.

- R4-2514518, Topic summary for [116bis][111] 6G operation efficiency, CATT, RAN WG4 Meeting #116bis, 13 – 17 October, 2025.