3GPP TSG-RAN WG4 Meeting #117	R4-2520868

# Dallas, USA, Nov. 17-21, 2025

Source: 	China Telecom

Title: 	Discussion on non-concurrent n5 DL and n8 UL for CA_n5-n8

Agenda Item:	5.9.2

Document for:	Approval

Introduction

In the last RAN4#116bis meeting, the RRM impact for realizing CA_n5-n8 with non-concurrent n5 DL and n8 UL has been discussed and get the following guidance. This contribution provides our views based on the RRM guidance and capability reporting.

<Way forward>:

Issue 2-1-1: Is there RRM impact to support 2UL/2DL CA with the solution described in WF R4-2511850?

Further discuss there is RRM impact w.r.t. scheduling restriction and interruption requirements

- Option 1: Handled by gNB scheduling, no RRM spec

- Option 2: RRM spec impact is required for supporting this

Discussion

As for whether there is RRM impact, in our views, there would be no RRM impact if UE is not scheduled to transmit on n8 UL in the slots where UE is configured to receive on n5 DL, e.g., UE is not scheduled to transmit on n8 UL in the slots where SSB and/or CSI-RS are configured on n5 DL.

Measurement and mobility requirements are typically defined based on SSB and/or CSI-RS, therefore, there is no RRM impact for measurement and mobility if UE is not scheduled to transmit on n8 UL in the slots where SSB and/or CSI-RS are configured on n5 DL.

As for selecting Option 1 or Option 2, in our view, it is just related to where to specify scheduling limitation for CA_n5-n8. One possible way is to specify in the note of the CA configuration table which is similar to that of non-simultaneous CA. For the detailed procedure on CA_n5-n8, as discussed in the WF [2], n5 acts as PCell for the band combination, that is, PCell is always n5, and n8 always serves as SCell. UL transmission via n8 is not scheduled to the UEs in slots where DL reception via n5 is scheduled or configured to the UEs, or DL reception via n5 is not scheduled to the UEs in slots where UL transmission via n8 is scheduled to the UEs.

Proposal 1: Specify the note in the CA configuration table to define scheduling limitation for CA_n5-n8, which is similar to that for non-simultaneous Rx/Tx.

Regarding capability reporting: Case 1 requires the BS to avoid allocating n5 DL resources to specific UE, meanwhile, case 2 requires the BS remove UL configuration of n8 UL to the specific UE via RRC. Although BS know the capability support when UE reporting UL CA_n5-n8, it is still beneficial to explicitly report this capability in case some UEs have other implementation for supporting UL CA_n5-n8 in future.

Proposal 2: Introduce UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Conclusion

In this contribution, we give our observation and proposal on spec impact and capability reporting for CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Proposal 1: Specify the note in the CA configuration table to define scheduling limitation for CA_n5-n8, which is similar to that for non-simultaneous Rx/Tx.

Proposal 2: Introduce UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

- References

- R4-2507910, WF on simultaneous Rx/Tx requirements, Huawei

- R4-2511850, WF on UL CA_n5-n8, CATT, Huawei, China Telecom, China Unicom, Anterix