3GPP TSG-RAN WG4 #116bis	R4-2520920

Dallas, USA, November 17-21, 2025

Agenda item:	6.12.3		Demodulation performance and CSI reporting requirements

Source:	Qualcomm Incorporated

Title:	Views on PMI performance requirements for Rel-19 MIMO Phase 5

Document for:	Discussion

# Introduction

In RAN4 #115, the work on Rel-19 MIMO Phase 5 demod requirements has been started. In RAN4#116-bis several key agreements have been made [1].

In this paper we provide our views for the open issues identified in the way forward in [1].

# Discussion

In the following sections we share our view on CSI reporting requirements for MIMO Phase 5.

## 128 CSI-RS Ports

In RAN4#116-bis, PMI requirements with Single-Panel-TypeI and eTypeII and 64 ports for both TDD and FDD were agreed. In this section, we provide few observations based on initial simulation results for settings agreed in [1].

Test settings

- TDD, 64 Tx ports, XP-Medium, Single Panel Type I, TDLA30-5, Rank 2

| Scenario | SNR (dB) at 90% maximum throughput | Gamma |
| --- | --- | --- |
| 2Rx, TDD | 9.2 | Infinity |
| 4Rx, TDD | 3.3 | Infinity |

- FDD, 64 Tx ports, XP-Medium, Single Panel Type I, TDLA30-5, Rank 2

| Scenario | SNR (dB) at 90% maximum throughput | Gamma |
| --- | --- | --- |
| 2Rx, FDD | 10.5 | Infinity |
| 4Rx, FDD | 3.7 | Infinity |

Observation 1: For all scenarios with XP-medium, the throughput achieved with random precoding is zero at SNR where 90% of maximum throughput by following PMI. This implies an infinite Gamma.

Observation 2: To have a finite and stable Gamma, an SNR with a higher BLER (e.g. 10%) with random precoder is required.

To choose a stable BLER region for random precoder, we propose to add an offset to the SNR at 90% of maximum throughput with follow PMI.

Proposal 1: RAN4 to add an offset of 5 dB to determine random precoder throughput with 4Rx for stable Gamma.

Proposal 2: RAN4 to add an offset of 3 dB to determine random precoder throughput with 2 Rx for stable Gamma.

If choosing different SNR points for follow and random precoders is not convenient for test procedures, then an alternative is to add an SNR offset for random precoder and choose a single and large Gamma that is lower than minimum among all Gamma from simulation results.

Proposal 3: RAN4 to choose a single Gamma based on post processing (offset to random precoder) of simulation results at 90% of maximum throughput SNR point for PMI requirements.

# Conclusions

In this paper we provided our views on several of the open issues identified in RAN4 #116-bis. Our observations and proposals are summarized as follows:

Observation 1: For all scenarios with XP-medium, the throughput achieved with random precoding is zero at SNR where 90% of maximum throughput by following PMI. This implies an infinite Gamma.

Observation 2: To have a finite and stable Gamma, an SNR with a higher BLER (e.g. 10%) with random precoder is required.

Proposal 1: RAN4 to add an offset of 5 dB to determine random precoder throughput with 4Rx for stable Gamma.

Proposal 2: RAN4 to add an offset of 3 dB to determine random precoder throughput with 2 Rx for stable Gamma.

Proposal 3: RAN4 to choose a Gamma based on post processing (offset to random precoder) of simulation results at 90% of maximum throughput SNR point by follow PMI.

# References

- R4-2514806, Way Forward of [116-bis][219] NR_MIMO_demod, October 2025