3GPP TSG-RAN WG4 Meeting # 117	R4-2520949

Dallas, US, 17th November – 21st November, 2025

Agenda item:		6.18.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for XR Phase 3

Document for:	Discussion

- Introduction

In the last RAN4 meeting, RAN4 discussed the RRM impacts of the WI with agreements and issues captured in the WF [1]. In this contribution, we provide our views on the potential impact on the performance part of XR.

- Discussion

Test target

During the last meeting, RAN4 discussed the test target for the XR devices. The following part is captured from the WF[1] as follows.

Issue 1-1-4: Test target <Way forward> Further discuss on the following proposals - Proposal 1: Further discuss on the UE processing timeline of receiving the DCI-based MG skipping indication - Proposal 2: Accuracy requirements - Proposal 3: Cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps

The test target of the XR device should focus on the DCI-based MG skipping function. This functionality of gap cancellation is the main target and feature of XR. It is necessary to introduce the test case for testing the timeline of DCI-based measurement gap skipping. For the processing time of DCI, the 3ms and 5ms depend on the UE capability and can be set in the test configuration. The 5ms capability is optional, the UE need to pass the test with a 3ms time offset by default.

Proposal 1: RAN4 defines the test of the UE processing timeline of receiving the DCI-based MG skipping indication.

- The DCI processing time offset depends on the UE capability in the test configuration.

For accuracy requirements and scheduling restrictions, in our view, these test cases are unnecessary to define. For instance, the accuracy requirements do not only rely on the UE side gap skipping achievement, but the network configuration of measurement gap cancellation will also affect the accuracy. In other words, the accuracy requirement needs a ratio of canaelled gap, but RAN4 does not have a consensus on this issue yet. Therefore, we propose further discussion of these issues.

Observation 1: The accuracy requirement needs a ratio of canaelled gap, but RAN4 does not have a consensus on this issue yet.

Proposal 2: The Accuracy requirements definition is related to the gap cancellation ratio issue and needs further discussion.

Cancellation model for XR test cases

Issue 1-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps <Way forward> Proposals for discussion - Option 1: Measurement gap cancellation is modelled as a fixed pattern for the test cases. - Option 2: Measurement gap cancellation is determined randomly by the test equipment.

The gap cancellation is random for the XR device, which is defined in the core requirement. For test cases, to verify the function and mechanism, it is possible to set a fixed pattern for needing cancellation gaps. RAN4 needs to discuss how to define this fixed cancellation gap pattern.

Proposal 3: RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern.

Cancellation ratio

Issue 2-3-3: Cancelation ratio <Way forward> Proposals for discussion - Proposal 1: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. - Proposal 2: For Type 1 MGs test cases: - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

In the core requirement part, RAN4 do not define the requirement for the gap cancellation ratio. So, in the performance part, only define the gap pattern for cancellation instead of defining the ratio of gap occasions.

Observation 2: There is no requirement defined in the core part for the cancellation ratio.

Proposal 4: It is not reasonable to define the cancellation ratio in the performance part of XR.

- Conclusions

Proposal 1: RAN4 defines the test of the UE processing timeline of receiving the DCI-based MG skipping indication.

- The DCI processing time offset depends on the UE capability in the test configuration.

Observation 1: The accuracy requirement needs a ratio of canaelled gap, but RAN4 does not have a consensus on this issue yet.

Proposal 2: The Accuracy requirements definition is related to the gap cancellation ratio issue and needs further discussion.

Proposal 3: RAN4 to design a fixed cancellation gap pattern for test cases, and further discuss the pattern.

Observation 2: There is no requirement defined in the core part for the cancellation ratio.

Proposal 4: It is not reasonable to define the cancellation ratio in the performance part of XR.

4. References

[1] R4-2514818 WF on RRM requirements for XR_Ph3