3GPP TSG-RAN WG4 Meeting #117	R4-2520951

Dallas, US, 17th November – 21st November, 2025

Agenda item:	7.4.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on (e)RedCap UE enhancement for Rel-20

Document for:	Discussion

- Introduction

The enhancement of the (e)RedCap UE in Release 20 is intended to achieve lower cost, reduced complexity and a smaller battery size for (e)RedCap devices. Building on the objectives outlined in WID [1], the new RRM requirement supersedes the previous RRM specifications of RLM and BFD and supports channel bandwidths of less than 5 MHz. In this contribution, we provide our views on the enhancement of (e)RedCap.

- Discussion

Relaxed RLM/BFD RRM requirements

Criteria for relaxation requirements

During the last meeting, RAN4 have two agreements on the power serving on the relaxed RAM/BFD for (e)RedCap UE. The agreements are captured from the WF[1] as follows.

Issue 2-2-1: Scope of relaxed RLM/BFD for (e)RedCap UE Agreement: RAN4 only considers the relaxed RLM/BFD requirements on PCell when the DRX cycle is no longer than 80 ms. Issue 2-2-2: Relaxed RLM/BFD for 2Rx (e)RedCap UE Agreement: - Reuse the existing R17 RLM/BFD relaxation requirements for both FR1 and FR2, and for full-duplex FDD and TDD.

The DRX length and applicability rule have already been discussed and have the agreement, we would like to contribute to the other remaining issues in this paper. To extend the Rel-17 RLM/BFD relaxation measurement to (e)RedCap in Rel-20, the criteria should consider the following aspect for 1Rx (e)RedCap and 2Rx (e)RedCap:

- Serving cell quality criteria of RLM/BFD relaxation

- Mobility criterion

Observation 1: To extend the Rel-17 RLM/BFD relaxation measurement to (e)RedCap in Rel-20, the criteria should consider the following aspect for 1Rx (e)RedCap and 2Rx (e)RedCap:

- Serving cell quality criteria of RLM/BFD relaxation

- Low mobility criterion

For the serving cell quality criteria, the relaxation for the serving cell is when the quality of the serving cell is “good”. As Rel-17 power serving is defined, the threshold for the serving cell quality criteria of RLM/BFD relaxation is Qin + offset for RLM and BFD. The offset in the threshold may not be applicable for both (e)RedCap UE 1Rx and 2Rx.

Proposal 1: For (e)RedCap, the criteria of RLM/BFD relaxation for serving cell quality reuse the legacy requirement: Qin + offset. Further discuss how to define the offset for 1Rx and 2Rx (e)RedCap UE.

The low‑mobility criterion remains applicable to (e)RedCap devices because these devices include wearables. In the Rel‑17 power‑serving discussion, no new low‑mobility criteria were introduced, so the Rel‑16 low‑mobility criterion – which is based on L3 RSRP measurement variation – can be reused. In our view, there is no issue with applying the Rel‑16 low‑mobility criterion, using L3 RSRP measurement variation, to Rel‑20 (e)RedCap UEs.

Proposal 2: Reuse Rel-16 low mobility criterion based on L3 RSRP measurement variation for Rel-20 (e)RedCap UE.

Support of less than 5 MHz channel bandwidth

HD-FDD mode

During the last meeting, RAN4 discussed the scope and outline of how to apply the current less than 5MHz requirement to the Rel-19 (e)RedCap. There are some open issues in the WF[1] that need to be further discussed. The following content are captured from the WF[1].

Issue 2-1-2: LT5 for 2Rx (e)RedCap UE Agreement: - The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with full-duplex FDD, TDD modes. - Handover - RLM/BFD - Intra-frequency/Inter-frequency measurement - Further check for HD-FDD

To determine how to enable the HD‑FDD mode on a Release‑19 (e)RedCap UE, we should first analyse how the feature was implemented in the Release‑17 HD‑FDD (e)RedCap UE. The Release‑17 RRM enhancement introduced the (e)RedCap UE, which supports both FDD and HD‑FDD modes. While the standard FDD mode presents no problems, the HD‑FDD mode may be subject to certain limitations.

The Rel-17 (e)RedCap UE HD-FDD mode supports the following additional conditions compared with FDD mode:

- SSB is available at the UE once every SMTC period during Tsearch

- One SSB is available during T∆

- One SSB is available during TIU.

- The HD-FDD UE shall meet the RRC connection release with redirection requirements: SSB is available at the UE once every SMTC period during Tsearch.

According to the above-listed elements, in our view, there is no impact on the applicability of less the 5MHz requirement on the (e)RedCap HD-FDD UE.

Proposal 3: The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with HD-FDD.

Coverage equivalence

About the (e)RedCap UE coverage equivalence and RLM requirement, the last meeting has agreed to further discuss. The following content is captured from the WF[1].

Issue 2-1-4: RLM/BFD impact of LT5 for 1Rx (e)RedCap UE Agreement: For discussing the PDCCH hypothetical parameters, RAN4 discuss whether it is feasible to target on coverage equivalence between 1Rx and 2Rx RedCap UEs.

To address the coverage equivalence, the aggregation level in the PDCCH transmission parameters is the curcial point. The following content is captured from TS 38.133 V19.2.0[2].

(e)RedCap Parameters: Table 8.1B.2.1-1: PDCCH transmission parameters for out-of-sync evaluation for RedCap UE Attribute; Value for BLER Configuration #0. DCI format; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 16; for RedCap UE with 1Rx branch. 8; for RedCap UE with 2Rx branches.. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4 dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4 dB. Bandwidth (PRBs); 48; for RedCap UE with 1Rx branch. 24; for RedCap UE with 2Rx branches.. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. NOTE: SCS = 60kHz is not applicable for FR1.. Less than 5MHz Parameters: Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for a UE operating on a cell with less than 5 MHz BW Attribute; Value for BLER Configuration #0. Channel BW; 3 MHz; 5 MHz. [DL Transmission BW]; 12 PRBs; 15 PRBs; 20 PRBs. DCI format; 1-0. Number of control OFDM symbols; 2; 3; 3. Aggregation level (CCE); 4; 8; 8. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4 dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4 dB. Bandwidth (PRBs); 12; 15; 20. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed; Non-DistributedNote 1; Distributed. NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication..

For the SSB bases RLM and BFD requirement, the Aggregation level in different bandwidths with different values, e.i, 4 and 8. For 2Rx (e)RedCap, this feature can be reused, and RAN4 is necessary to discuss the applicability of this to 1Rx (e)RedCap. Because the Aggregation level is related to coverage of (e)RedCap. To achieve the coverage equivalence on 1Rx and 2Rx (e)RedCap UE, the aggregation level should be designed to the same level.

Observation 2: The coverage is related to the Aggregation level in the PDCCH transmission parameters.

Because of the above rationale, the aggregation level for the 1Rx (e)RedCap UE must be increased when the Less‑than‑5 MHz requirement is applied. In practice, this means raising the AL from 8 to 16 or from 4 to 8, depending on the bandwidth configuration.

The reason for this increase is to maintain coverage equivalence: the Release 17 (e)RedCap specifications set the AL for the 1Rx (e)RedCap at twice the value used for the 2Rx (e)RedCap, thereby ensuring that both types of (e)RedCap equipment achieve comparable coverage.

Observation 3: To achieve the coverage equivalent, the aggregation level of the 1Rx (e)RedCap UE must be increased when the Less‑than‑5 MHz requirement is applied.

Therefore, RAN4 needs to discuss and evaluate the case of 1Rx (e)RedCap with 12 PRBs and 15 PRBs. In our view, the case where the PDCCH bandwidth is 12 PRBs with the aggregation level of 4 for the 1Rx (e)RedCap is possible to increase the AL by extending the number of control OFDM symbols from 2 to 3.

Proposal 4: RAN4 to discuss the feasibility of extending the number of control OFDM symbols from 2 to 3 for 1‑Rx (e)RedCap the BFD parameter with PDCCH bandwidth of 12 PRBs.

SSB based RLM/BFD

Given that the bandwidth for the 1Rx (e)RedCap is longer than the 2Rx (e)RedCap. The current PDCCH BW for 1Rx (e)RedCap is 48 PWB, which is wider than the 2Rx (e)RedCap. This design is to ensure the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap is the same. To apply the less than 5MHz feature, the bandwidth of the PDCCH will be 15PRBs and 12 PRBs for both 1Rx (e)RedCap and 2Rx (e)RedCap. This will cause the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap to be different.

Observation 4: Applying the less than 5MHz SSb based RLM and BFD will cause the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap to be different.

The TEvaluate_out_SSB,RedCap is defined in the requirement of SSB based RLM with different values for 1Rx (e)RedCap and 2Rx (e)RedCap. The details are captured from Chapter 8.1B of 38.133 V19.2.0[2]:

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx (e)RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(200, Ceil(10 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320 ms; Max(200, Ceil(15 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320 ms; Ceil(10 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx (e)RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(400, Ceil(20 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320 ms; Max(400, Ceil(30 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320 ms; Ceil(20 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length..

For the 1Rx (e)RedCap device, the detection SNR is lower than the 2Rx (e)RedCap. The reason why the TEvaluate_out_SSB,RedCap for 1Rx (e)RedCap is 20 is that working in a low SNR environment needs more samples to maintain the performance. In our view, if RAN4 introduces the less than 5 MHz feature to Rel-20 (e)RedCap while retaining the same aggregation level for both 1Rx and 2Rx (e)RedCap devices. The TEvaluate_out_SSB,RedCap can be set to the same value for both 1Rx (e)RedCap and 2Rx (e)RedCap.

Observation 5: If RAN4 introduces the less than 5 MHz feature to Rel 20 (e)RedCap while retaining the same aggregation level for both 1Rx and 2Rx (e)RedCap devices. The TEvaluate_out_SSB, RedCap can be set to the same value for both 1Rx (e)RedCap and 2Rx (e)RedCap.

Proposal 5: RAN4 to discuss the evaluation period once an agreement has been reached on the PDCCH transmission parameters for Rel-20 (e)RedCap UE supports the LT5 requirement.

- Conclusions

Observation 1: To extend the Rel-17 RLM/BFD relaxation measurement to (e)RedCap in Rel-20, the criteria should consider the following aspect for 1Rx (e)RedCap and 2Rx (e)RedCap:

- Serving cell quality criteria of RLM/BFD relaxation

- Low mobility criterion

Proposal 1: For (e)RedCap, the criteria of RLM/BFD relaxation for serving cell quality reuse the legacy requirement: Qin + offset. Further discuss how to define the offset for 1Rx and 2Rx (e)RedCap UE.

Proposal 2: Reuse Rel-16 low mobility criterion based on L3 RSRP measurement variation for Rel-20 (e)RedCap UE.

Proposal 3: The existing LT5 requirements for non-RedCap UE can be directly reused for 2Rx (e)RedCap UE with HD-FDD.

Observation 2: The coverage is related to the Aggregation level in the PDCCH transmission parameters.

Observation 3: To achieve the coverage equivalent, the aggregation level of the 1Rx (e)RedCap UE must be increased when the Less‑than‑5 MHz requirement is applied.

Proposal 4: RAN4 to discuss the feasibility of extending the number of control OFDM symbols from 2 to 3 for 1‑Rx (e)RedCap the BFD parameter with PDCCH bandwidth of 12 PRBs.

Observation 4: Applying the less than 5MHz SSb based RLM and BFD will cause the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap to be different.

Observation 5: If RAN4 introduces the less than 5 MHz feature to Rel 20 (e)RedCap while retaining the same aggregation level for both 1Rx and 2Rx (e)RedCap devices. The TEvaluate_out_SSB, RedCap can be set to the same value for both 1Rx (e)RedCap and 2Rx (e)RedCap.

Proposal 5: RAN4 to discuss the evaluation period once an agreement has been reached on the PDCCH transmission parameters for Rel-20 (e)RedCap UE supports the LT5 requirement.

- References

[1] R4-2514821 WF on NR_RRM_Ph6

[2] RP-252955 WID: NR Radio Resource Management (RRM) Phase 6

[3] 3GPP TS 38.133 V19.1.0