3GPP TSG-RAN WG4 Meeting # 117	 R4-2520952

Dallas, US, 17th November – 21st November, 2025

Agenda item:	7.10.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM core requirement of MIMO phase 6 for Rel-20

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [2]. RAN4 discussed the RRM impacts with the objective captured in WID[2]. In this contribution, we further provide our views on the remaining issues on enhancing UL capacity and coverage and enhancing DL CSI acquisition. In this paper, we will provide our views on the DL CSI acquisition in a case-by-case manner.

- Discussion

Cross-slot SRS

The intention of this objective is to improve the frequency detection granularity of SRS and achieve cross-slot multiplexing without destroying the existing SRS mechanism. At the same time, a special study is conducted on phase continuity to ensure that the new function can provide more detailed measurement capabilities in actual deployment without causing compatibility or phase error issues.

The following is the agreement from the RAN1 discussion.

Agreement Support at least the following scenarios for cross-slot SRS transmission: - Scenario 1: a periodic, semi-persistent, or aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted across a first S slot and a second consecutive U slot. - FFS whether Scenario 2 below is supported. Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot. Agreement For a P/SP cross-slot SRS resource, the slot offset configured to the SRS resource refers to the first of the two slots spanned by the SRS resource.

The discussion in the RAN1 is processing on the slot and the indication of how to design the cross-slot, like Scenario 1 mentioned above. Regarding this agreement, the legacy interruption requirement of SRS antenna port switching may not be applicable. The current requirement is based on a single slot and has a restriction that SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.

Observation 1: The legacy requirement of SRS antenna port switching is based on a single slot and has a restriction that SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.

Proposal 1: RAN4 to discuss whether or not to introduce a new interruption requirement for cross-slot SRS antenna port switching.

Early SRS/CSI/CSI-RS triggering

When a UE moves from IDLE/INACTIVE to CONNECTED mode, or when an SCell is transitioned from a deactivated or dormant state to an active state, the network must schedule DL data immediately in order to complete the connection setup. However, the UE may not have yet completed the measurement cycle required to acquire and report CSI‑RS measurements for the target SCell. Until the UE finishes the configured measCycleSCell and sends a valid CSI‑Report, the network has no prior CSI on which to base closed‑loop precoding for MIMO arrays.

Because closed‑loop precoding relies on an accurate CSI report, transmitting data with an unavailable precoding matrix can lead to severe beam mispointing and an unacceptably high block error rate. To guarantee reliable delivery during this interim period, the legacy use of a fallback transmission scheme.

Early triggering via RACH

The design of the procedure for early CSI/CSI-RS/SRS triggering via RACH will be based on the RAN1 agreements. The RAN1 agreements are shown as follows:

RAN1 #122 meeting Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately RAN1 #122bis meeting Agreement For early triggering of SRS-AS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple SRS configurations based on one or multiple ‘xTyR’ UE capability assumptions. - At least one SRS configuration(s) resource sets for SRS-AS in an SRS configuration is provided for a same ‘xTyR’ - FFS: Information to be provided in SRS configuration(s) - FFS: Whether Configuration information could be SRS configuration or the CSI report configuration for PMI-free report. - FFS: Whether SRS configuration can be associated with CSI report configuration for PMI-free report. - In Step-2, UE reports through MSG3 which/whether the SRS configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the SRS configuration(s) provided in SIBx is/are supported. Agreement: For early triggering of aperiodic CSI reporting and the associated CSI-RS for CSI when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple CSI report configurations based on one or multiple UE capability assumptions - Each CSI reporting configuration is associated with a CSI resource configuration for channel measurement. - The CSI-RS resource set(s) in the CSI resource configuration for channel measurement is provided with a same number of CSI-RS ports - Each CSI reporting configuration is associated with a CSI resource configuration for CSI-IM based interference measurement. - FFS: Support of NZP CSI-RS based interference measurement - FFS: Information to be provided in a CSI report/resource configuration - In Step-2, UE reports through MSG3 which/whether the CSI report configuration(s) provided in SIBx is/are supported. FFS: How to report which/whether the CSI report configuration(s) provided in SIBx is/are supported.

In the RAN1 agreements, the procedure of the early reporting trigger is based on the 4-step RACH. The trigger action happens during the RACH procedure, and the UE will finish the report at the end of the RACH.

Observation 2: The early trigger action happens during the procedure of RACH, and the UE will finish the report at the end of the RACH.

In terms of the RAN1 agreements, this report is a periodic report and tied to the procedure of the RACH. Although the timeline of the SRS-AS transmission, aperiodic CSI-RS reception, and aperiodic CSI reporting is FFS, the report moment is fixed at the final step of RACH. Therefore, RAN4 do need to define a new RRM delay requirement for early trigger report via RACH.

Proposal 2: No need to define a new RRM delay requirement for early CSI/CSI-RS/SRS trigger during transition from IDLE/INACTIVE to CONNECTED mode.

Early triggering via SCell activation

Similarly, the design of the early trigger procedure needs to reference the RAN1 agreements. The RAN1 agreements of the last meeting are shown as follows:

RAN1 #122 meeting Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is associated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? FFS: Whether/what new information is provided in SCell activation command? RAN1 #122bis meeting Agreement: On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting: - Alt-1 (Implicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration - Alt-2 (Explicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.

In the RAN1 #122 meeting agreements, it is mentioned that the requirement is up to RAN4. In the RAN1 #112bis meeting, they further provide contribution on the design of the timeline of the triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for an SCell. In our view, the timeline discussion is based on the RAN1 and RAN4 only need to discuss the requirement of delay on early triggering via SCell activation.

Here is one principle that the CSI measurement and report are supposed to be conducted after the UE finishes DL synchronisation. The reason is that only after the UE achieves precise DL synchronisation, the UE can correctly demodulate the CSI‑RS and avoid channel estimation errors caused by clock deviation. Similarly, for early triggering of SRS, it is necessary to ensure that the triggered SRS transmission is conducted after the UE finishes DL synchronisation, since the UL synchronisation is based on DL synchronisation.

Observation 3: The CSI/CSI-RS/SRS measurement and report are supposed to be conducted after the UE finishes DL synchronisation.

To figure out the requirement of the new early report/measurement, RAN4 needs to discuss which activation of SCell is applicable. Here is the analysis of the Rel-15 SCell activation and Rel-17 fast SCell activation.

The Rel‑15 SCell activation MAC‑CE only provides the basic SCell activation functionality. The UE’s DL synchronisation relies on measuring the SSB. In this situation, the network is unaware of the moment the UE completes DL synchronisation, because the network does not know which SSB the UE has used. Moreover, the UE may need to observe several SSB periods for AGC adjustment and for DL synchronisation, and the network has no information about the SSB periodicity at which the UE finishes this process.

In contrast, the Rel‑17 fast SCell activation assumes that the UE has already performed coarse synchronisation of the SCell and only needs to carry out precise synchronisation. To enable this, the enhanced MAC‑CE specifies a set of TRS resources for each activated SCell. Because the transmission times of the TRS are known to the network, the network can determine exactly when the UE will complete DL synchronisation.

Based on this analysis, we recommend adopting the Rel‑17 fast SCell activation for early SRS/CSI/CSI‑RS triggering. The Rel‑15 MAC‑CE could also be considered, but only if a method can be found to guarantee the timing requirement.

Observation 4: It is possible to adopt the Rel‑17 fast SCell activation for early SRS/CSI/CSI‑RS triggering. The Rel‑15 MAC‑CE could also be considered, but only if a method can be found to guarantee the timing requirement.

The discussion of applicable scenario(s) is unclear in RAN1. RAN4 should wait for the RAN1 progress on this. For the start point of the requirement discussion, the early report of SRS-AS and CSI/CSI-RS will be triggered based on the legacy SCell activation command, the legacy SCell procedure with three stages, shown as follows:

- Stage 1: Activation indication, which includes SCell activation MAC-CE transmission and processing. The time for this stage is THARQ.

- Stage 2: SCell activation, which mainly includes DL synchronisation based on SSB and/or TRS. The time for this stage is Tactivation_time.

- Stage 3: CSI acquisition, which includes CSI measurement and reporting. The time for this stage is TCSI_Reporting.

RAN4 to start the discussion of the delay requirement of the early triggering SCell activation, the start point can be the SCell being activated, no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

Observation 5: Based on the current RAN1 agreements, the TCSI_Reporting will be impacted by the early report triggering.

Proposal 3: RAN4 to start the discussion of the delay requirement of the early triggering SCell activation, the start point can be the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length. And the update of TCSI_Reporting will be based on the RAN1 further progress.

For the timeline discussion in RAN4, from our perspective, it is not needed for now. But if companies want to discuss the timeline of early report of SCell in RAN4, there is one principle that needs to be followed: The CSI/CSI-RS/SRS measurement and report are supposed to be conducted after the UE finishes DL synchronisation.

Proposal 4: If RAN4 starts the discussion of the timeline, there is one principle that needs to be followed: the CSI/CSI‑RS/SRS measurement and reporting should be conducted after the UE has completed DL synchronisation.

Early triggering when switching out of SCell dormancy

We recommend waiting for the RAN1 design for the early reporting indication or CSI/SRS request. RAN4 wait for further agreement in RAN1 for early triggering when switching out of SCell dormancy.

Proposal 5: Due to no explicit product has been defined in RAN1 for early triggering when switching out of SCell dormancy, RNA4 need to wait for further agreement in RAN1.

- Conclusions

Observation 1: The legacy requirement of SRS antenna port switching is based on a single slot and has a restriction that SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.

Proposal 1: RAN4 to discuss whether or not to introduce a new interruption requirement for cross-slot SRS antenna port switching.

Observation 2: The early trigger action happens during the procedure of RACH, and the UE will finish the report at the end of the RACH.

Proposal 2: No need to define a new RRM delay requirement for early CSI/CSI-RS/SRS trigger during transition from IDLE/INACTIVE to CONNECTED mode.

Observation 3: The CSI/CSI-RS/SRS measurement and report are supposed to be conducted after the UE finishes DL synchronisation.

Observation 4: It is possible to adopt the Rel‑17 fast SCell activation for early SRS/CSI/CSI‑RS triggering. The Rel‑15 MAC‑CE could also be considered, but only if a method can be found to guarantee the timing requirement.

Observation 5: Based on the current RAN1 agreements, the TCSI_Reporting will be impacted by the early report triggering.

Proposal 3: RAN4 to start the discussion of the delay requirement of the early triggering SCell activation, the start point can be the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length. And the update of TCSI_Reporting will be based on the RAN1 further progress.

Proposal 4: If RAN4 starts the discussion of the timeline, there is one principle that needs to be followed: the CSI/CSI‑RS/SRS measurement and reporting should be conducted after the UE has completed DL synchronisation.

Proposal 5: Due to no explicit product has been defined in RAN1 for early triggering when switching out of SCell dormancy, RNA4 need to wait for further agreement in RAN1.

4. References

[1] R4-2514825 WF on NR_MIMO_Ph6_RRM

[2] RP-252510 Revised WID - NR_MIMO_Ph6