3GPP TSG-RAN WG4 Meeting #117								    		 R4-2520968

Dallas, USA, 17th – 21st Nov, 2025

Agenda Item: 8.4.3

Source: MediaTek Inc.

Title: Initial evaluations on 6G coexistence study

Document for: Discussion

# 1	Introduction

Following the 6G SI [1] approved in RAN#108, the first 6G discussion starts from this RAN4 meeting. In RAN4#116b, a WF [2] was agreed to further study on the need of redoing coexistence study. The corresponding agreements are captured below.

### Issue 2-1-1: Coexistence studies Way Forward: List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

In this paper, we provide our views and some initial evaluation results on the 6G coexistence study.

# 2		Coexistence study

Some evaluation results in TR 38.922 are based on the FR1-like UE with omni-directional antenna. In our understanding, omni-directional antenna is not a realistic assumption. In any practical UE implementation, it is not possible to really achieve omni-directional antenna. To be more realistic, a simple way is to apply a certain front-to-back ratio. Such a front-to-back ratio can help to avoid overestimation of the interference signal power coming from an opposite direction to the antenna direction, which is especially useful when UE has more than 1 antennas. An illustrative figure is provided below.

Proposal 1: RAN4 to consider whether and how to add front-to-back ratio even for omni-directional UE antenna assumptions.

In the following, we provide some initial coexistence simulation results to check the impact of changing certain parameters. The simulation assumptions are in general the same as TR 38.922 with some particular assumptions provided in Table 1. The required ACIR levels at 5% UL throughput loss in BS for average throughputs and 5%-ile throughputs by aggressor 6G UEs are provided in Table 2. The entire curves for UL throughput loss vs ACIR level are provided in the Annex.

Table 1. Some simulation assumptions for coexistence study

| Scenario |  | UMa |
| --- | --- | --- |
| Deployment |  | Non-collocated |
| Direction |  | UL-to-UL (UE-to-BS) |
| Carrier frequency |  | 7GHz |
| CBW |  | 200MHz |
| Indoor UE ratio |  | 20% |
| BS settings | Am | 30 dB |
|  | SLAv | 30 dB |
|  | j3dB | 90 deg. |
|  | Q3dB | 65 deg. |
|  | GE,max | 6.4 dBi |
|  | Msub | 3 |
|  | dv,sub | 0.7l m |
|  | qsubtilt | 3 deg. |
|  | {M, N} | {8, 16} and {16,24} |
|  | dh | 0.5l m |
|  | dv | 2.1l m |
|  | qetilt | 90≤θescan≤100 deg. |
|  | Jescan | -60≤φescan≤60 deg. |
|  | R | 1 |
|  | qmech | 6 deg. |
| UE settings | # of Tx antenna | 1, without front-to-back ratio |
|  | Power class | PC3 and PC2 |

Table 2. Required ACIR levels (dBc) at 5% UL throughput loss

| {M, N} | Tput | PC3 | PC2 |
| --- | --- | --- | --- |
| {8, 16} | Average | 13.1 | 13.3 |
|  | 5%-ile | 18.2 | 18.2 |
| {16, 24} | Average | 11.6 | 12.2 |
|  | 5%-ile | 17.7 | 17.9 |

From the initial evaluation results, we can observe:

- Changing from PC3 to PC2 lead to a very minor increase of the required ACIR level. In our understanding, those UEs who really needs to transmit at the maximum power are indoor UEs in this scenario. However, indoor UE suffers from the additional high penetration loss which applies equally to the target BS and any neighboring BSs. Therefore, the impact of increasing UE’s maximum power brings only minor difference.

- Changing {M, N} from {8, 16} to {16, 24} can lower the required ACIR level. With more antennas, BS can form narrower Rx beams which helps in reducing the impact of interference from unwanted directions.

Observation 1: At around 7GHz, PC2 and PC3 UEs lead to very similar required ACIR levels.

As we know, a higher frequency band usually suffers from a higher pathloss, which demands HPUE in order to maintain acceptable UL coverage. Since the initial results shows that PC2 requires very similar ACIR levels as PC3, we think this kind of means that PC2 could be considered as a default power class for around 7GHz. We suggest companies to also study whether it is feasible and where there is any problem identified to assume PC2 as the default power class for around 7GHz.

Proposal 2: RAN4 to study whether PC2 can be assumed as the default power class for around 7GHz.

# 3	Summary

In this paper, we provide our view on the 6G coexistence study. We have the following observations and proposals.

Proposal 1: RAN4 to consider whether and how to add front-to-back ratio even for omni-directional UE antenna assumptions.

Observation 1: At around 7GHz, PC2 and PC3 UEs lead to very similar required ACIR levels.

Proposal 2: RAN4 to study whether PC2 can be assumed as the default power class for around 7GHz.

# 4	References

- [1] RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator)

- [2] R4-2514638, WF on 6G BS RF and co-existence, feature lead (Ericsson)

# 5	Annex

| - UL Tput loss vs ACIR levels for {M,N}={8,16} |  |
| --- | --- |
| - PC3 (23dBm) | - PC2 (26dBm) |
| - UL Tput loss vs ACIR levels for {M,N}={16,24} |  |
| - PC3 (23dBm) | - PC2 (26dBm) |