3GPP TSG-RAN WG4 Meeting #117			R4-2520977

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	4.25.3

Source: 	CMCC

Title: 			   Discussion on the RRM requirement maintenance for NTN phase3

Document for:	Discussion

- Introduction

In RAN4#116 meeting, the core part of NTN phase 3 has been finished, but some maintenance issue still need to be further discussed[1][2].

Besides, a LS was received from RAN2#131 meeting[3], which inform RAN4 about additional agreements on SMTC enhancements. RAN4 need to take them into account when define the RRM requirements.

For downlink coverage enhancement, RAN2 further made the following agreements related to SMTC enhancement: - We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. - In connected mode the NW will only configure up to 4 STMC. - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. RAN2 kindly request RAN4 to take above agreement into account when designing RAN4 RRM requirement for SMTC enhancement and provide feedback if needed.

In this contribution, we further discuss the maintenance issues.

- Maintenance for Downlink coverage enhancements

## [Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6)

RAN4 to further discuss the following in RAN4#116-bis:

For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant:

- The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE.

- Note: The SMTC selection rule may be further discussed and specified or left to UE implementation.

In RRC_IDLE/Inactive mode, we support the rule in above first bullet, that the SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE.

Regarding the SMTC selection rule, RAN2 agreed in RAN2#131 meeting that:

- RAN2 introduce a capability to support STMC enhancements to FR1

- If the NW configures more than 4 SMTCs the UE in idle/inactive the UE selects up to 4 SMTCs (or less, depending on the UE capability) based on location and only considers them for measurements.

- RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band).

The details of two capabilities are as follows, which defined in TS38.306:

Location based SMTC selection for RRC_IDLE/RRC_INACTIVE It is optional for the UE in RRC_IDLE/RRC_INACTIVE to support SMTC selection based on UE position and associated location for each SMTC in NTN, as specified in TS 38.331 [9].

twoSMTC-Periodicities-r19 Indicates whether the UE supports NTN SSB based RRM measurements on target cells using two SMTC periodicities on a single frequency carrier. A UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17.

Based on RAN2’s further agreement, although how to select the SMTC to up to 4 (or less, depending on the UE capability) is up to UE implementation, but the result after the selection is clear, that the number of selected SMTCs will not exceed the UE capability. Such selection result shall be captured in RAN4 spec.We propose following wording on top of the existing specification,

For intra-frequency cell re-selection measurement:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if UE does not support twoSMTC-Periodicities-r19 and the number of SMTCs with a periodicity equal to the serving cell SMTC periodicity doesn’t exceed the parallelSMTC-r17, the requirements in this clause apply. Otherwise, UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected

For inter-frequency cell measurement:

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected.

Proposal 1: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for intra-frequency cell re-selection measurement is proposed:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if UE does not support twoSMTC-Periodicities-r19 and the number of SMTCs with a periodicity equal to the serving cell SMTC periodicity doesn’t exceed the parallelSMTC-r17, the requirements in this clause apply. Otherwise, UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected

Proposal 2: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for inter-frequency cell re-selection measurement is proposed:

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

cell re-selection measurement is proposed:

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, if UE does not support twoSMTC-Periodicities-r19 and the number of SMTCs with a periodicity equal to the serving cell SMTC periodicity doesn’t exceed the parallelSMTC-r17, the requirements in this clause apply. Otherwise, UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in table 4.2C.2.3-1 and table 4.2C.2.3-2 is expected

Proposal 2: In RRC-IDLE/Inactive, update the SMTC selection rule, following wording for inter-frequency cell re-selection measurement is proposed:

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise

-	For UE which support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE shall select the SMTC to be used of all the configured SMTCs up to the parallelSMTC-r17, the selection is up to UE implementation, and the requirements in this clause apply.

-	For UE which does not support Location based SMTC selection for RRC_IDLE/RRC_INACTIVE, UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2C.2.4-2 is expected.

- Reference

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R2-2506298, LS on additional agreements on SMTC enhancements, RAN2