3GPP TSG RAN WG4 Meeting #117	R4-2521009

Dallas, USA, 17 – 21 November, 2025

Agenda item:			6.18.2

Source:	vivo

Title:	On RRM performance requirements for XR

Document for:	Discussion

# Introduction

In the RAN4#116-bis meeting, RAN4 discussed the performance requirements for Rel-19 XR. Following agreements were achieved and captured in the WF [1].

### Issue 1-1-1: Scenarios for the definition of the test cases <Agreement> - At least for measurement procedure RAN4 RRM to define test cases for NR-SA for both FR1 and FR2-1 - For FR1 TCs, - all cells are in FR1 - For FR2 TCs - all cells in FR2-1 - FFS if all TCs also cover FR2-1 - Test cases are specified only for per-UE MGs ### Issue 1-1-2: Type 1 and Type 2 gaps <Agreement> - Test cases are defined only for Type 1 gaps ### Issue 1-1-3: DRX proposals <Agreement> - Each type of test case is specified for either DRX or non-DRX but not both. ### Issue 1-1-4: Test target <Agreement> Proposals 1, 2, and 3 are addressed in the same test case - Proposal 1: measurement delay extension with gap cancellation - Proposal 2: Reception/transmission without scheduling restrictions during cancelled gap occasions - Proposal 3: The UE processing timeline of receiving the DCI-based MG skipping indication <Way forward> Further discuss on the following proposals - Proposal 1: Further discuss on the UE processing timeline of receiving the DCI-based MG skipping indication - Proposal 2: Accuracy requirements - Proposal 3: Cancellation of scheduling restrictions due to L3 and L1 measurements without gaps during cancelled gaps ### Issue 1-1-5: List of test cases <Agreement> - Define test case for: - Inter-frequency measurements procedure with gaps - Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1 - FFS whether to define at least one accuracy test case in FR1 and in FR2-1

In this contribution, we further provide our views on RRM performance requirements for Rel-19 XR.

# Discussion

The following open issues need to be solved for defining test cases for XR measurement gap cancellation.

### Issue 1-2-1: SSB time index detection <Way forward> Proposals - Proposal 1: For measurement procedure, define test cases without SSB time index detection. - Proposal 2: Use SSB index detection for intra-/inter-frequency measurement TCs, use no SSB index detection with lower priority for intra-/inter-frequency measurement TCs.

The purpose of the test is to verify UE handling measurement gap cancellation. SSB time index detection has no impact on measurement gap cancellation. To simplify the test design, it is suggested SSB time index detection is not considered in the tests for XR.

Proposal 1: SSB time index detection is not considered in the tests for XR.

### Issue 1-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps <Way forward> Proposals for discussion - Option 1: Measurement gap cancelation is modelled as a fixed pattern for the test cases. - Option 2: Measurement gap cancelation is determined randomly by the test equipment. ### Issue 2-3-3: Cancelation ratio <Way forward> Proposals for discussion - Proposal 1: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. - Proposal 2: For Type 1 MGs test cases: - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

The measurement gap cancellation was target for XR services which somehow is not random in terms of gap cancellation. However, since it is DCI triggered dynamic gap cancellation there would be no difference between randomly cancellation and fixed pattern cancellation. What matters is the ratio of gap cancellation which would have impact on mobility performance and test time. Following is proposed for ratio of gap cancellation.

Proposal 2: Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 5ms Toffset.

### Issue 1-4-1: Toffset between DCI and start of MG <Way forward> Proposals for discussion - Option 1: The processing time 3ms/5ms shall be verified in the test cases. - Option 1a: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition - Option 2: Define test case for 5ms offset only. ### Issue 1-4-2: Types of DCI with cancellation indication <Way forward> Proposals for discussion - Option 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation. - Option 2: Define RAN4 test cases using DCI 1-1 for the canceling indication. ### Issue 1-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’ <Way forward> Proposals for discussion - Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. - Proposal 2: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows: - Test Option 2 in every test case. - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

The UE capability for measurement gap cancellation is as follows.

| 64. NR_XR_Ph3 | 64-1 | Enabling TX/RX during measurement gap scheduling restriction | 1. Reception/transmission during measurement gaps/scheduling restrictions is enabled based on explicit indication by DCI 2a. DCI format 0_1 and 1_1 for enabling reception/transmission in gap/restriction 2b. Additional supported DCI formats for enabling reception/transmission in gap/restriction 3. UE behaviour for indication field value of “0” - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as of Rel-18 behaviour. 4. The minimum time offset from the end of PDCCH carrying the gap/restriction cancellation to the start of cancelled gap/restriction |
| --- | --- | --- | --- |

In addition, candidate values of component 2b: 3-bit bitmap {DCI 0_2 and 1_2, DCI 0_3, DCI 1_3} and candidate values of component 3: {Option 1, Option 2}.

DCI format 0-1 and 1-1 is mandatorily supported by the UE supporting measurement gap cancellation. Then in RAN4 test cases, DCI 1-1 can be used for the gap cancellation indication.

For time offset, it is based on UE capability whether UE supports either 5ms or 3ms. It would be better to verify the UE supporting 3ms time offset by indicating gap cancellation with 3ms time offset.

UE behaviour for indication field value of “0” is based on UE capability. To ensure different type of UE can be tested, both options should be tested.

In one test, different time offset and different option for value ‘0’ indication can be addressed. Different UE capability can use different test configuration but it is still one test.

Proposal 3: DCI 1-1 is used in the test for gap cancellation indication.

Proposal 4: Time offset of both 3ms and 5ms are tested in one test, where time offset for DCI triggering is based on UE capability.

Proposal 5: UE bbehavior of Option 1 and Option 2 for indication of value ‘0’ are tested in one test, where test requirement on measurement delay is based on UE capability.

# Summary

In this contribution, we further provided views on RRM performance requirements for XR in Rel-19. Following proposals and observations are present.

Proposal 1: SSB time index detection is not considered in the tests for XR.

Proposal 2: Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 5ms Toffset.

Proposal 3: DCI 1-1 is used in the test for gap cancellation indication.

Proposal 4: Time offset of both 3ms and 5ms are tested in one test, where time offset for DCI triggering is based on UE capability.

Proposal 5: UE bbehavior of Option 1 and Option 2 for indication of value ‘0’ are tested in one test, where test requirement on measurement delay is based on UE capability.

- References

- R4-2514869 WF on NR_XR_Ph3_RRM, Nokia