3GPP TSG-RAN WG4 Meeting #117	R4-2521061

Dallas, US, 17 – 21, November 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on IoT-NTN PC1.5 single Tx

# Agenda item:	7.6.3

# Document for:	Approval

# 1 Introduction

In RAN#109 meeting, a new Work Item [1] was approved on introduction of HPUE for NTN with the following RAN4 objective. In this contribution, we want to share some further views about IoT-NTN PC1.5 single Tx from UE RF requirements perspective.

HPUE for NTN Core Part - Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

# 2 Discussion

- Issue 2-1-6: Capability signalling for PC1.5 NB-IoT NTN UE Agreement: - The signaling to indicate support for PC1.5 is needed to be added to LTE RRC signalling with preference on early implementation.

Regarding capability signalling for PC1.5 NB-IoT NTN UE, we reached an agreement as in the above. Per the agreement, a LS is needed to send to RAN2. For preference on early implementation, we didn’t see any motivation to do this. Moreover, whether to introduce the capability in early implementation manner should be discussed in RAN2 instead of RAN4. In RAN4, we can discuss that whether to introduce PC1.5 in a release independent manner for IoT-NTN.

Proposal 1: Need to send a LS to RAN2 and whether to introduce the capability in early implementation manner should be discussed in RAN2.

- Issue 2-2-3: UE RF front end assumptions and related accommodations WayFoward: - FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements) - The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118.

In last meeting, one company raised power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm, and proposed to use duty cycle scheme to solve the problem. During online discussion, many companies showed their concern about duty cycle scheme. The main reason includes that in Rel-19 NTN HPUE discussion, we agreed to use P-MPR solution instead of duty cycle. Additionally, although duty cycle solution was introduced in RAN4 spec, it was never used in field as the exact evaluation period is not clear and no duty cycle solution introduced for FDD bands.

Furthermore, for the UE architecture, P-MPR solution was used for PC1 and PC2 NB-IoT based IoT-NTN for both handheld and non-handheld UEs. Same logic for PC1.5, we can use P-MPR solution if power handling challenges exist.

Proposal 2: To use P-MPR solution if power handling challenges exist.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for IoT-NTN PC1.5 and the proposals are made as follows:

Proposal 1: Need to send a ls to RAN2 and whether to introduce the capability in early implementation manner should be discussed in RAN2.

Proposal 2: To use P-MPR solution if power handling challenges exist.

# 4 Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)