3GPP TSG-RAN WG4 Meeting #117	R4-2521070

Dallas, US, 17 – 21, November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-2 | CR | 0088 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TEI19-BDaT_simp_improvement) CR to TS 38.176-2 spec structure simplification for co-location and co-existence requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the RAN task, the spec structure simplification for transmitter spurious emission in subclause 6.7.5.4.5.1 and 6.7.5.5.5.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The table is simplified to include the general requirement level. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the specification cannot be improved as requested by RAN task. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.7.5.4.5.1 and 6.7.5.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is the resubmission of the endorsed draftCR R4-2515110. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.174: "NR; Integrated access and backhaul radio transmission and reception".

[3]	3GPP TS 38.176-1: " NR; Integrated Access and Backhaul (IAB) conformance testing; Part 1: Conducted conformance testing".

[4]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[5]	3GPP TS 38.141-1: "NR, Base Station (BS) conformance testing, Part 1: Conducted conformance testing".

[6]	3GPP TS 38.141-2: "NR, Base Station (BS) conformance testing, Part 2: Radiated conformance testing".

[7]	3GPP TS 38.211: "NR; Physical channels and modulation".

[8]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[9]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[10]	Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain".

[11]	ERC Recommendation 74-01: "Unwanted emissions in the spurious domain".

[12]	Recommendation ITU-R M.1545, "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[13]	Recommendation ITU-R SM.328: "Spectra and bandwidth of emissions".

[14]	"Title 47 of the Code of Federal Regulations (CFR)", Federal Communications Commission.

[15]	3GPP TR 25.942: "RF system scenarios".

[16]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[17]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[18]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[19]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations".

[20]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations".

[21]	IEC 60 721: "Classification of environmental conditions".

[22]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold".

[23]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat".

[24]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)".

[25]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements".

[26]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[27]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[28]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 standalone".

[29]	3GPP TS 38.521-2: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 standalone”.

[30]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

[31]	Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band

[32]	3GPP TS 25.104: "Base Station (BS) radio transmission and reception (FDD) "

[33]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

==============Next change==============

6.7.5.4.5	Test requirement

6.7.5.4.5.1	Test requirement for IAB type 1-O

The power of any spurious emission shall not exceed the test limits in table 6.7.5.4.5-1 for a IAB where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the table 6.7.5.4.5-1 apply for each supported operating band.

Table 6.7.5.4.5.1-1: IAB-DU and IAB-MT spurious emissions basic limits for co-existence with systems operating in other frequency bands

| System type to co-exist with (Note 4) | Frequency range for co-existence requirement (MHz) (Note 5) | IAB-DU type 1-O Test limits (dBm) | IAB-MT type 1-O Test limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 | -45.4 | -45.4 +Y | 100kHz | Note 1 |
|  | 824 - 849 | -49.4 | -49.4 +Y |  |  |
| GSM900 | 921 - 960 | -45.4 | -45.4 +Y |  |  |
|  | 876 - 915 | -49.4 | -49.4 +Y |  |  |
| DCS1800 | 1805 - 1880 | -35.4 | -35.4 +Y |  |  |
|  | 1710 - 1785 | -49.4 | -49.4 +Y |  |  |
| PCS1900 | 1930 - 1990 | -35.4 | -35.4 +Y |  |  |
|  | 1850 - 1910 | -49.4 | -49.4 +Y |  |  |
| UTRA, E-UTRA or NR | Uplink operating band 22 and UTRA FDD Band XXII | -37 | -37 +Y | 1MHz | Note 1 |
|  | Downlink operating band 22, 42, 43 n48/48, n77, n78 and UTRA FDD Band XXII | -40 | -40 +Y |  |  |
|  | n46/46, 47, n79, n96, n102, n104 | -39.5 | -39.5 +Y |  |  |
|  | n82, n89 | -45.4 | -45.4 +Y |  |  |
|  | Downlink operating band n83, n85/85, n86 | -49.4 | -49.4 +Y |  |  |
|  | Uplink operating band n84, n85/85 | -35.4 | -35.4 +Y |  |  |
|  | n81, uplink operating band n93 and other downlink operating band | -40.4 | -40.4 +Y |  |  |
|  | Downlink operating band n93 and other uplink operating band | -37.4 | -37.4 +Y |  |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause the co-existence requirements in table 6.7.5.4.5.1-1do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.7.5.4.5.1-1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:   Y = - 9 + 10log10(NTXU,OTApercell) dB

NOTE 4:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104 [33], table 5.5-1.

NOTE 5:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 25.104 [32] clause 5.2, TS 36.104 [33] clause 5.5 and TS 38.104 [4] clause 5.2, respectively.

==============Next change==============

6.7.5.5.5.1	Test requirement for IAB type 1-O

These requirements may be applied for the protection of other IAB receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a IAB Node.

The requirements assume co-location with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

This requirement is a co-location requirement as defined in clause 4.9, in TS 38.174 [2], the power levels are specified at the CLTA output.

The output of the CLTA of any spurious emission shall not exceed the test limit in table 6.7.5.5.5.1-1.

For a multi-band RIB, the exclusions and conditions in the table 6.7.5.5.5.1-1 apply for each supported operating band.

Table 6.7.5.5.5.1-1: IAB-DU and IAB-MT spurious emissions basic limits for co-location with BS or IAB-Node

| Frequency range of uplink operating band of | System type to | Test limits (dBm/100kHz) (Note 1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| the co-located BS or IAB-Node (MHz) (Note 5) | co-locate with (Note 4) | WA IAB-DU | WA IAB-MT | MR IAB-DU | LA IAB-DU | LA IAB-MT |
| 824 - 849 | GSM850 or CDMA850 | -115.9 | -115.9 +Y | -108.9 | -87.9 | -87.9 +Y |
| 876 - 915 | GSM900 | -115.9 | -115.9 +Y | -108.9 | -87.9 | -87.9 +Y |
| 1710 - 1785 | DCS1800 | -115.9 | -115.9 +Y | -108.9 | -97.9 | -97.9 +Y |
| 1850 - 1910 | PCS1900 | -115.9 | -115.9 +Y | -108.9 | -97.9 | -97.9 +Y |
| n51/51 | E-UTRA or NR | N/A | N/A | N/A | -105.9 | -105.9 +Y |
| n46/46, n53/53 | E-UTRA or NR | N/A | N/A | -108.9 | -105.9 | -105.9 +Y |
| n96, n102 | NR | N/A | N/A | -107.6 | -104.6 | -104.6 +Y |
| n104 | NR | -112.6 | -112.6 +Y | -107.6 | -104.6 | -104.6 +Y |
| n79 | NR | -113.6 | -113.6 +Y | -108.6 | -105.6 | -105.6 +Y |
| UTRA FDD Band XXII or E-UTRA Band 22, 42, 43, n48/48, n77, n78 | UTRA, E-UTRA or NR | -113.7 | -113.7 +Y | -108.7 | -105.7 | -105.7 +Y |
| Other operating band | UTRA, E-UTRA or NR | -113.9 | -113.9 +Y | -108.9 | -105.9 | -105.9 +Y |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.3-1 do not apply for the frequency range extending ΔfOBUE immediately outside the transmit frequency range of a IAB-MT and IAB-DU. The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB antenna to antenna minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [15].

NOTE 2:	Table 6.6.5.2.3-1 assumes that two operating bands, where the corresponding transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:   Y = - 9 + 10log10(NTXU,OTApercell) dB.

NOTE 4:	Does not apply for co-location with V2X operation defined in TS 36.104 [33], table 5.5-1.

NOTE 5:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 25.104 [32] clause 5.2, TS 36.104 [33] clause 5.5 and TS 38.104 [4] clause 5.2, respectively.

==============End of change==============