3GPP TSG-RAN WG4 Meeting #117														    R4-2521089

Dallas, USA, Nov. 17-21, 2025

Agenda item:			8.9

Source:	Samsung

Title:	Discussion on MRSS and inter-RAT mobility for 6GR

Document for:	Approval

1	Introduction

In RAN4#116bis meeting, 5G-6G spectrum sharing topic was initially discussed including both MRSS (Multi-RAT Spectrum Sharing) and inter-RAT mobility.

Regarding MRSS for migration, it was agreed to take following aspects as starting point for RAN4 study [1, R4-2514646]:

#### Issue 1-2-0: General consideration Agreement - To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study - System parameters - RF requirements - RRM requirements - Interference handling

Regarding mobility between 5G NR and 6GR, the discussion is focusing RRM aspects like inter-RAT handover interruption, inter-RAT measurement including with and without gaps.

In this paper, we share our views on both MRSS and mobility from RAN4 perspective.

2	Discussion

2.1	MRSS – defination and scope

MRSS (Multi-RAT Spectrum Sharing) has been discussed for quite a period before it was captured into the 6G WG level SID [2, RP-252912], however, there is no clear definition for this concept yet. So, the detailed implication of MRSS need to be figured out by RAN work groups. E.g.

- Semi-static MRSS or dynamic MRSS;

- Whether or not reusing NR signals/channels for 6GR

Fundamentally speaking, these aspects are up to RAN1 design, however, different approach has different RAN4 impacts. To facilitate RAN4 study on MRSS, it was also agreed in last RAN4 meeting that RAN4 discussion will be based on the agreed hypothesis before RAN4 receives concrete design from RAN1/2.

Semi-static MRSS relies on FDM/TDM between NR and 6GR and RRC-based rate-matching (RM) can be adopted as necessary, while dynamic MRSS relies on dynamic RM without fixed FDM/TDM between NR and 6GR. Following the 5G DSS experience, we should not assume all UE mandatorily supporting dynamic rate matching. Consequently, the semi-static MRSS should be the baseline for MRSS as a spectrum sharing migration.

Proposal 1:	It is proposed that RAN4 assumes RRC-based semi-static MRSS as baseline for RAN4 MRSS study.

Whether or not reusing NR signals/channels (e.g., SSB) for 6GR is also under RAN1 discussion. A basic understanding is that the sharing-based MRSS (reusing NR signals/channels for 6GR) would obviously restrict the 6GR design. SSB is expected to have minimal overhead and re-use of NR SSB may lead to multiple SSB designs for 6GR.

Moreover, sharing-based MRSS (reusing NR signals/channels for 6GR) can be seen as a special option under RAN1 discussion. A traditional MRSS with separate signals/channels for each RAT should be followed as baseline.

Based on above considerations, it is better for RAN4 to assume not reusing NR signals/channels for 6GR as hypothesis.

Proposal 2:	It is proposed that RAN4 assumes not reusing NR signals/channels for 6GR as hypothesis for RAN4 MRSS study.

In last RAN4 meeting, companies also involved NTN into the MRSS discussion.

#### Issue 1-1-3: Spectrum sharing with NTN - Proposals from companies - P1 (CATT): The MRSS should incorporate NTN network in the scope, including 6G TN + 6G NTN, NR TN + 6G NTN, as well as 6G TN + NR NTN - P2 (Xiaomi): - RAN4 shall study potential solutions and impact on co-channel interference handling and mobility which also related to regulation update including following scenarios: - NTN operation use TN bands/spectrum - 5G NTN and 6G NTN share dedicated NTN spectrum - P3 (ZTE): for MRSS BS, propose to consider the TN BS with 5G-6G TN MRSS in the existing TN MSR specification and NTN SAN with 5G-6G NTN MRSS in the new NTN MSR specification - Way forward - Further discuss

For spectrum sharing between TN and NTN, the work on using TN band for NTN service is needed. RAN plenary has discussed the new work item for using TN bands for NTN service, which has not yet been approved. Regarding detailed scenarios, in our understanding the base stations for TN and NTN are at different places respectively and not sure if there is applicable scenario for multi-RAT spectrum sharing. Further clarification on applicable scenario is needed. Especially for 6G TN + 6G NTN, actually it is spectrum sharing within same RAT and thus not belongs to multi-RAT spectrum sharing.

For spectrum sharing between 5G NTN and 6G NTN, it seems more practical scenario and the group should discuss if this scenario is in scope and whether RAN4 focus on TN MRSS firstly. In RAN4 6G thread for spectrum, when discussing TN and NTN frequency range harmonization, it was agreed that RAN4 will initially focus the discussion on frequency ranges based on TN deployment [3, R4-2514626].

Proposal 3:	It is proposed that proponents for MRSS with NTN further clarify the detailed applicable scenarios especially for TN and NTN spectrum sharing.

2.2	MRSS – frequency range or band

In last RAN4 meeting, the applicable frequency range or band was also discussed, and the agreement is as following [1]

- Agreements - For operating frequency range or band for MRSS between 6GR and NR - Start RAN4 6G-5G MRSS study in FR1 - Whether to consider 6G-5G MRSS in FR2-1 is further discussed

Looking back at the 5G DSS, it was observed that the DSS feature was band-specific. DSS was enabled based on operator demand and was enabled for demanded band by dedicated work items, indicated as following Figure.

Fig. 1 DSS enabled with band-specific manner

Observation 1:	DSS in 5G was a band-specific feature. DSS was enabled based on operator demand and was enabled for demanded band by dedicated work items.

Given there is obvious throughput degradation, DSS was not commercially successful. Similarly, for 6G-5G MRSS, it should not be a feature for all bands by default. Especially for U6G band, it potentially could enjoy more advanced 6G characteristics. Now U6G band has not commercial deployment yet and there would be short gap between 5G deployment and 6G deployment for this band, it seems not necessary to enable MRSS for U6G bands.

Observation 2:	U6G band potentially could enjoy more advanced 6G characteristics. There is no 5G commercial deployment yet and there would be short gap between 5G deployment and 6G deployment.

Proposal 4:	MRSS should be enabled in per-band manner based on operator demand in RAN4.

Proposal 5:	It is not necessary to support 6G-5G MRSS for U6G bands.

For 6G-5G MRSS in FR2-1, in last meeting majority view is that FR1 can be prioritized and FR2-1 can be further discussed. Different from FR1 where DSS was mainly for single carrier spectrum sharing e.g., at FDD low bands, the existing 5G FR2-1 deployment is mostly based on Nx100MHz carriers, the scheduling of 5G and 6G can be based on different 100MHz carriers, therefore it is not necessary to enable MRSS which brings no much benefits for FR2 scenario but has to afford throughput degradation unnecessarily.

Observation 3:	Existing FR2-1 deployment is mostly based on Nx100MHz carriers which indicates no 6G-5G MRSS necessity in the same carrier (MRSS in such scenario brings no much benefits but has to afford throughput degradation unnecessarily).

Proposal 6:	It is not necessary to support MRSS for FR2 bands.

2.3	MRSS – System paramter impacts

System parameter is important for MRSS. In last RAN4 meeting, it was agreed that for some system parameters (e.g. SCS, sync raster, channel raster etc.) separated agenda will be designated for the scenarios of with (in spectrum sharing) and without (in system parameter) MRSS considered. In this agenda, the system parameters to be studied are as following [1]

- Taking following table as starting point to discuss which system parameters are impacted by MRSS. System parameters; Whether impacted by MRSS. Numerology; Maybe impacted. Channel raster; Maybe impacted. Sync raster; Maybe impacted. Waveform; FFS. Modulation; FFS. Channel bandwidth; FFS.

For numerology, same SCS is precondition for MRSS operation. Based on latest RAN1 agreement and considering practical SCS in 5G deployment, it is reasonable to consider 15kHz SCS for FDD and 30kHz SCS for TDD in 5G-6G MRSS.

Proposal 7:	The SCS for 5G and the SCS for 6G should be the same in 5G-6G MRSS. 15kHz SCS for FDD and 30kHz SCS for TDD can be considered.

For channel raster, different from 4G-5G DSS where 7.5kHz uplink shifting was applied to align the orthogonality of LTE and NR uplink sub carriers, it is obviously not necessary to apply 7.5kHz uplink shifting for 5G-6G MRSS. This seems common understanding and can be agreed in RAN4.

Proposal 8:	7.5kHz uplink shifting is not needed for 5G-6G MRSS.

For sync raster which indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition when explicit signalling of the synchronization block position is not present, it is highly dependent on SSB design in RAN1. It is difficult for RAN4 to make a hypothesis in this stage. It would be better to wait some time for RAN1 progress on SSB design.

Proposal 9:	RAN4 to evaluate sync raster pending on RAN1 progress on SSB design.

For waveform, RAN1 has already agreed that the waveforms defined in 5G NR are supported as the basis for 6GR. For other waveforms, even there would be, it does not impact MRSS operation if the MRSS operation is restricted to 5G compatible waveforms. Similarly, for modulation, even there would be higher order modulation than 5G, no much RAN4 impacts to MRSS is foreseen.

Observation 4:	Based on RAN1 agreement, no much RAN4 impacts foreseen from waveform aspect and modulation aspect.

For channel bandwidth, technically speaking all CBW can be applicable for MRSS as long as rate-matching is feasible. However, the smaller CBW, the higher degradation for MRSS efficiency. It was a lesson that 5G DSS is commercial failure due to high throughput loss. Thus, it is reasonable not to consider MRSS for small channel bandwidths, e.g., 3MHz and 5MHz.

Proposal 10:	RAN4 not to consider 5G-6G MRSS for small channel bandwidths. FFS the boundary above which MRSS can be applied.

2.4	MRSS – RF requirement impacts

According to the experience of 5G DSS, the only RAN4 core requirement impacts were the 7.5kHz uplink shifting in both UE and BS specifications.

For SUL bands except n95, n97, n98 and for the uplink of all FDD bands defined in Table 5.2-1, and for TDD bands n34, n39, n48, n90, n38 and n40 FREF, shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz. where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [7]. For Band n34, n38, n39, n40, n48 FREF, shift is only applicable to uplink transmissions using a 15 kHz SCS.

Given 7.5kHz uplink shifting is deemed not necessary any more for 5G-6G MRSS, no other RF requirement impacts are foreseen at this stage.

Observation 5:	No RF requirement impacts are foreseen at this stage for 5G-6G MRSS.

2.5	MRSS – RRM requirement impacts

It is the same for RRM if there would be no special enhancement for 5G-6G MRSS. There were no RRM impacts in 5G DSS. From the RRM perspective, there are so many key measurements in RRM which are based on SSB. According to current RAN1 discussion, there is a high probability that the SSB design in 6GR is different from that in 5G. From the system design, it leads the restriction on the design of 6GR sync signals to use the same sync signals in 5G NR. Based on this assumption, RAN4 RRM will be similar as what we encountered in 5G DSS. RAN4 can focus on the RRM requirements without MRSS in 6GR, if nothing new enhancements in RAN1 special for MRSS, the same set RRM requirements can be applied in MRSS as well. Then the new RRM requirements are only for Inter-RAT mobility.

In system parameter part discussion of last meeting, RAN4 has agreed to strive for unified system parameters for the scenarios with and without MRSS, if feasible [1]. It would be also preferred to define unified RRM requirements for the scenarios with and without MRSS from RAN4 perspective.

Proposal 11:	RAN4 strives to define unified RRM requirements for the scenarios with and without MRSS.

From RAN4 perspective, it is better to minimize the RRM impact due to MRSS operation. Sharing NR signals/channels for 6GR in MRSS operation is contradicted to this principle. From this sense, the MRSS operation should be transparent to UE as possible, to minimize the RRM impacts.

Proposal 12:	MRSS operation should be transparent to UE as possible, to minimize the RRM impacts.

2.6	inter-RAT mobility – RRM requirements

#### Issue 1-1-1: Migration scheme - Agreements - RAN4 focus on MRSS between 6GR and NR until further RANP guidance - RAN4 focus on inter-RAT mobility between 6GR and NR

According the agreements in RAN-P and RAN4#116bis meeting, RAN4 only focus on inter-RAT mobility between 6GR and NR.

As the key factor of wireless system, the mobility performance is very important for system. When UE is moving in an area with 5GNR cells and 6GR cells, the RRM requirements for inter-RAT mobility shall ensure the UE for smart and fast connection. From the RRM perspective, many categories of RRM requirements for inter-RAT mobility shall be investigated and defined.

- In RRC_Idle mode, cell reselection is the key RRM procedure for mobility. When a user is moving, the signal quality of camped cell and neighbouring cells change constantly. Cell reselection ensures the UE proactively finds and connects to a better cell (to 6GR cell or to 5GNR cell) before the UE fails in connection.

- In RRC_Connected mode, HO is the key RRM procedure for mobility. When a user is moving, the signal quality of serving cell and neighbouring cells change constantly. HO or HO-like ensures the UE proactively finds and connects to a better cell (to 6GR cell or to 5GNR cell) before the UE fails in connection.

- For RRM measurement, inter-RAT measurement shall be also considered.

For the details of each RRM requirements, since the fundamental RRM framework is still ongoing. It’s too early to discuss each component of RRM requirements. As the purpose to support inter-RAT measurement without gap, we prefer to use 5G later releases as the baseline and take the consideration in the general Gap design as well.

Proposal 13:     For inter-RAT mobility support, RAN4 shall focus on the discussion to support:

- Cell reselection for inter-RAT cells

- Handover to other RATs for inter-RAT mobility

- Inter-RAT measurement

RAN4 shall consider the inter-RAT measurement without GAP with capability as start point and discuss the Gap design in general. In addition, RAN1 inputs of synchronization signals are needed for further discussion.

3	Conclusion

Proposal 1:	It is proposed that RAN4 assumes RRC-based semi-static MRSS as baseline for RAN4 MRSS study.

Proposal 2:	It is proposed that RAN4 assumes not reusing NR signals/channels for 6GR as hypothesis for RAN4 MRSS study.

Proposal 3:	It is proposed that proponents for MRSS with NTN further clarify the detailed applicable scenarios especially for TN and NTN spectrum sharing.

Observation 1:	DSS in 5G was a band-specific feature. DSS was enabled based on operator demand and was enabled for demanded band by dedicated work items.

Observation 2:	U6G band potentially could enjoy more advanced 6G characteristics. There is no 5G commercial deployment yet and there would be short gap between 5G deployment and 6G deployment.

Proposal 4:	MRSS should be enabled in per-band manner based on operator demand in RAN4.

Proposal 5:	It is not necessary to support 6G-5G MRSS for U6G bands.

Observation 3:	Existing FR2-1 deployment is mostly based on Nx100MHz carriers which indicates no 6G-5G MRSS necessity in the same carrier (MRSS in such scenario brings no much benefits but has to afford throughput degradation unnecessarily).

Proposal 6:	It is not necessary to support MRSS for FR2 bands.

Proposal 7:	The SCS for 5G and the SCS for 6G should be the same in 5G-6G MRSS. 15kHz SCS for FDD and 30kHz SCS for TDD can be considered.

Proposal 8:	7.5kHz uplink shifting is not needed for 5G-6G MRSS.

Proposal 9:	RAN4 to evaluate sync raster pending on RAN1 progress on SSB design.

Observation 4:	Based on RAN1 agreement, no much RAN4 impacts foreseen from waveform aspect and modulation aspect.

Proposal 10:	RAN4 not to consider 5G-6G MRSS for small channel bandwidths. FFS the boundary above which MRSS can be applied.

Observation 5:	No RF requirement impacts are foreseen at this stage for 5G-6G MRSS.

Proposal 11:	RAN4 strives to define unified RRM requirements for the scenarios with and without MRSS.

Proposal 12:	MRSS operation should be transparent to UE as possible, to minimize the RRM impacts.

Proposal 13:       For inter-RAT mobility support, RAN4 shall focus on the discussion to support:

- Cell reselection for inter-RAT cells

- Handover to other RATs for inter-RAT mobility

- Inter-RAT measurement

RAN4 shall consider the inter-RAT measurement without GAP with capability as start point and discuss the Gap design in general. In addition, RAN1 inputs of synchronization signals are needed for further discussion.

Reference

[1] R4-2514646, WF for [116bis][108] 6G spectrum sharing, RAN4#116bis, CMCC

[2] RP-252912, Revised SID: Study on 6G Radio, RAN#109

[3] R4-2514626, WF on 6G Spectrum, RAN4#116bis, Nokia