3GPP TSG-RAN WG4 Meeting #117	R4-2521098

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on core requirements maintenance for Rel-19 NES

Agenda Item:	4.21.1

Document for:	Discussion

- Introduction

In this contribution, we provide analysis on RRM impact on OD-SSB with SSB-less operation regarding RAN2 LS [1].

- Discussion

Based on RAN2 discussion about the relationship between OD-SSB and SSB-less SCell, RAN2 conclude on following agreements and send LS [1] to RAN4:

Agreements:

1.	When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is not configured, the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation).

2.	When AO-SSB in reference cell is present (i.e. absoluteFrequencySSB is present) and OD-SSB is deactivated, RAN2 assume that the UE can obtain timing from SpCell or another SCell (i.e. SSB-less operation as legacy Rel-18 behavior).

3.	The field description of absoluteFrequencySSB is updated as follows:

Frequency of the SSB to be used for this serving cell. SSB related parameters (e.g. SSB index) provided for a serving cell refer to this SSB frequency unless mentioned otherwise. The CD-SSB of the PCell is always on the sync raster. Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15] or TS 38.101-5 [75]). If the field is absent, the SSB related parameters should be absent, e.g. ssb-PositionsInBurst, ssb-periodicityServingCell and subcarrierSpacing in ServingCellConfigCommon IE. If the field is absent and od-ssb-r19 is absent in SCellConfig, or if the field is absent and od-ssb-r19 is present in SCellConfig but OD-SSB is not activated, the UE obtains timing reference from the intra-band SpCell or intra-band SCell if applicable as described in TS 38.213 [13], clause 4.1, or from the SpCell or an SCell indicated by referenceCell, or from the reference serving cell defined in TS 38.133 [14]. This is supported in case the SCell for which the UE obtains the timing reference is in the same or different frequency band as the cell (i.e. the SpCell or the SCell, respectively) from which the UE obtains the timing reference. For PCell, this field corresponds to the CD-SSB.

Based on above agreements, it could be observed that SSB-less SCell operation can still apply when OD-SSB is configured but not activated.

Observation 1: Based on RAN2 agreements, SSB-less SCell operation can still apply when OD-SSB is configured but not activated.

Based on above understanding, we would like to further analyse the potential RRM impacts and issues in following part.

Q1: Which set of requirements to apply (i.e. SSB-less SCell activation or OD-SSB based SCell activation)

For a Rel-19 UE supporting both SSB-less (taking Rel-18 inter-band SSB-less as example) and OD-SSB, it could be further categorized into following subcase:

- Case 1: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is absent

- Case 2: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is not activated.

- Case 3: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is activated.

Proposal 1: RAN4 to discuss which set of SCell activation requirements to apply (i.e., SSB-less SCell activation or OD-SSB based SCell activation) for following cases:

- Case 1: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is absent

- Case 2: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is not activated.

- Case 3: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is activated.

For Case 1, it is straightforward that legacy SSB-less activation conditions and requirements and directly apply.

For Case 2, the conditions and requirements may need further clarifications. The existing conditions for Rel-18 SSB-less are shown as follows:

For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference cell and A-TRS/P-TRS of SSB-less SCell, and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

The highlighted part is QCL conditions for legacy SSB-less operation. The assumption is that all RS(s) are configured to TRS(s) which are further QCL-ed to an inter-band reference Cell. However, for the case described above, it is not realistic to assume that all RS(s) are QCL-ed to an inter-band reference Cell with OD-SSB configuration. The reason is that: if gNB wants to activate OD-SSB, gNB has to reconfigure the QCL source all these RS(s) before OD-SSB activation via RRC message. In other words, there is no sense to allow such case (i.e., UE is configured with OD-SSB but it is deactivated) since gNB has to perform RRC reconfiguration before OD-SSB activation. As mentioned in previous RAN4 discussion, the main use case is that gNB can decide to activate the SCell based on SCell when the SSB-less condition (e.g. RTD and power difference) cannot be guaranteed. To serve this purpose, gNB can provide multiple sets of QCL source (e.g. some of RS(s) are QCL-ed to inter-band reference Cell and some of RS(s) are QCL-ed to the OD-SSB of the SCell), and if gNB decide to activate the SCell via SSB-less procedure, gNB and indicate TCI states/RS to be measured which are based QCL-ed to inter-band reference Cell; and if gNB decide to activate the SCell based on OD-SSB, gNB and indicate TCI states/RS to be measured which are based QCL-ed to OD-SSB of serving Cell.

Observation 2: For the case “UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is not activated”, the main use case is that gNB to activate SCell based on OD-SSB when SSB-less conditions are not guaranteed.

Observation 3: Based on observation 2, the legacy conditions about QCL assumptions for SSB-less operation may needs to be reconsidered.

Based on above analysis, for case 2, we would like to discuss whether to clarify the existing QCL assumptions to allow that not all RS(s) are QCL-ed to the inter-band reference Cell. In details, some RS(s) are configured to be QCL-ed to OD-SSB of SCell. If the SCell is activated as an OD-SSB SCell, UE is only required to measure the RS(s) which are QCL-ed to inter-band reference Cell (e.g., the RS of active TCI states of PDCCH/PDSCH, the CSI-RS that UE is required to measure).

Proposal 2: For case 2, UE shall follow SSB-less SCell activation procedure and requirement, and RAN4 to discuss whether to clarify the existing QCL assumptions to allow that not all RS(s) are QCL-ed to the inter-band reference Cell.

- E.g., some RS(s) are configured to be QCL-ed to OD-SSB of SCell and some RS(s) are configured to be QCL-ed to reference Cell before SCell activation.

For case 3, it is straightforward that UE shall follow OD-SSB based SCell activation since gNB activates OD-SSB on purpose. One thing needs to be further clarified is the timeline between OD-SSB activation and SCell activation, from our understanding, if the OD-SSB is activated at the same time as SCell activation or OD-SSB is activated before SCell activation and kept activated during and after SCell activation, UE shall follow OD-SSB based SCell activation procedure.

Proposal 3: For case 3, if the OD-SSB is activated at the same time as SCell activation or OD-SSB is activated before SCell activation and kept activated during and after SCell activation, UE shall follow OD-SSB based SCell activation procedure and requirements.

Another issue we should be careful with is whether UE is assumed to support this combination of SSB-less and OD-SSB if UE support Rel-15/Rel-18 SSB-less and Rel-19 OD-SSB. For instance, if UE supports Rel-18 SSB-less feature, UE assumes that the QCL source can only be inter-band reference Cell. But if UE support OD-SSB, this UE does not only need to support OD-SSB operation, but also enhanced Rel-18 SSB-less (i.e., the QCL source could be inter-band reference cell or OD-SSB SCell conditionally), which may cause some NBC issue as UE has to modify the implementation of a Rel-18 feature. Thus, RAN4 shall also discussed whether UE is mandated to support the combination of SSB-less and OD-SSB if UE support Rel-15/Rel-18 SSB-less and Rel-19 OD-SSB.

Proposal 4: RAN4 to discuss whether a UE is mandated to support the combination of SSB-less and OD-SSB if UE support Rel-15/Rel-18 SSB-less and Rel-19 OD-SSB.

- Conclusion

Observation 1: Based on RAN2 agreements, SSB-less SCell operation can still apply when OD-SSB is configured but not activated.

Proposal 1: RAN4 to discuss which set of SCell activation requirements to apply (i.e., SSB-less SCell activation or OD-SSB based SCell activation) for following cases:

- Case 1: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is absent

- Case 2: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is not activated.

- Case 3: UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is activated.

Observation 2: For the case “UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration AND od-ssb-r19 is present but OD-SSB is not activated”, the main use case is that gNB to activate SCell based on OD-SSB when SSB-less conditions are not guaranteed.

Observation 3: Based on observation 2, the legacy conditions about QCL assumptions for SSB-less operation may needs to be reconsidered.

Proposal 2: For case 2, UE shall follow SSB-less SCell activation procedure and requirement, and RAN4 to discuss whether to clarify the existing QCL assumptions to allow that not all RS(s) are QCL-ed to the inter-band reference Cell.

- E.g., some RS(s) are configured to be QCL-ed to OD-SSB of SCell and some RS(s) are configured to be QCL-ed to reference Cell before SCell activation.

Proposal 3: For case 3, if the OD-SSB is activated at the same time as SCell activation or OD-SSB is activated before SCell activation and kept activated during and after SCell activation, UE shall follow OD-SSB based SCell activation procedure and requirements.

Proposal 4: RAN4 to discuss whether a UE is mandated to support the combination of SSB-less and OD-SSB if UE support Rel-15/Rel-18 SSB-less and Rel-19 OD-SSB.

References

[1] R2-2507768	LS on OD-SSB