3GPP TSG-RAN WG4 Meeting #117	R4-2521099

Dallas, USA, Nov. 17-21, 2025

Source: 	vivo

Title: 	Discussion on RRM performance for Rel-19 NES OD-SSB

Agenda Item:	6.15.2.1

Document for:	Discussion

- Introduction

The perf part of Rel-19 NES was initially discussed in last meeting with agreements captured in [1]. In this contribution, we provide our analysis on remaining issues on performance part for OD-SSB.

- Discussion

The performance part has been initially discussed in last meeting with following agreements.

Issue 1-1-1: General – Case 1 and Case 2 Agreement: - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Case #1: scenario #2, scenario #2A. - Case #2: scenario #2, scenario #2A and scenario #3B. Issue 1-1-2: General – OD-SSB parameter update (scenario #3B) Close the issue. Issue 1-1-3: General – Different OD-SSB and AO-SSB relation in Case 2 Agreement: RAN4 to define TCs for Alt Time-C1. Meanwhile, - Option 1: Not define TC for Alt Time-C2 - Option 2: Select one test for Alt Time-C2. For UE supporting Alt Time-C2, the corresponding test case for Alt Time-C1 can be skipped. Further discuss in one test case or separate test cases. Issue 1-1-4: Scenario – Scell activation scope Agreement: RAN4 to only define the test cases for single Scell activation, Direct SCell activation. Issue 1-1-5: Scell activation scope – known/unknown Close the issue. Issue 1-1-6: T2 for deactivated SCell measurement Agreement: Define one test case in case 1 to verify the UE measurement behaviour when the OD-SSB is reactivated relevant to T2. Issue 1-3-1: General principle Agreement: RAN4 to apply the following principles for OD-SSB test - Both FR1 and FR2 - Test non-DRX mode, and select one test case with DRX configuration for deactivated SCell measurement. - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - Only NCD-SSB for OD-SSB test case

The test cases list was also agreed as captured in the WF. There are three candidate test cases in FR1 which are still FFS which is shown as follows.

- SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB case 1

- Direct SCell activation at SCell addition based on OD-SSB case 1

- Intra-frequency measurement (deactivated SCell) based on OD-SSB

For the first test case, as TC1 is already agreed to be introduced which is the DRX based SCell activation for case 1, the only difference is whether DRX is configured. For SCell activation procedure, UE shall activated SCell based on OD-SSB periodicity regardless of DRX configuration. If the motivation is to verify deactivated SCell measurement in scenario 2, TC7 can already serve the purposed.

Proposal 1: Do not define following test case since the performance can be verified by TC1 and TC7.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

For the second test case, as RAN4 already agreed to have TC6 which is Direct SCell activation at SCell addition based on OD-SSB case 2, the only difference is OD-SSB case 1 and case 2. As commented in last meeting, the test procedure is that OD-SSB is activated upon RRC configuration, and the SCell is activated via the same RRC message. Thus, from UE perspective, there is no big difference between case 1 and case 2. UE will activate the SCell based on dense SSB periodicity (i.e. OD-SSB for case 1 and union of OD-SSB and AO-SSB in case 2). Thus, there is also no need to define another TC only for difference OD-SSB case.

Proposal 2: Do not define following TC:

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |

The last one is whether to define TC for FMW for OD-SSB case 2 in FR1. RAN4 already agreed to define TC for TC6 which is FMW for OD-SSB case 1 in FR2. From our understanding, the UE behaviour on FMW ongoing legacy measurement with AO-SSB may need to be verified additionally.

Proposal 3: RAN4 to define following TC.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FFS FR1 |

- Conclusion

Proposal 1: Do not define following test case since the performance can be verified by TC1 and TC7.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB | SCell activation Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1 |

Proposal 2: Do not define following TC:

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Direct SCell activation at SCell addition based on OD-SSB | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a FR1 |

Proposal 3: RAN4 to define following TC.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| FFS | Intra-frequency measurement(deactivated SCell) based on OD-SSB | Deactivated SCell measurement; Interruption during deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW FFS FR1 |

References

[1] R4-2514808	WF on Netw_Energy_NR_enh_RRM_Part1