# 3GPP TSG-RAN WG4 Meeting #117	R4-2521146

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.17.2.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on performance requirements for CLTM

Document for:	Approval

Introduction

The RRM core requirement on Mobility work item has been summarized, which include

- Event triggered L1 measurement reporting

- CSI-RS measurements for LTM procedures

- (Subsequent) conditional LTM

This contribution provides our view on performance requirements for CLTM.

Discussion

- For (subsequent) conditional LTM, we should define test cases based on R18 LTM. In R18 LTM, we defined the test cases for RACH-based/RACH-less, intra-frequency/inter-frequency, PCell/PSCell switch from FR1 to FR1 or FR2 to FR2. During Release 18, we have done the down selection when designed the test cases. And in this meeting, we should discussed whether without early DL sync is needed for CLTM.

- Issue 1-7: early DL sync < Way forward > - Option 1: With early DL sync. - Option 2: With and without early DL sync. - Option 3: For each FR, one test case with and without early DL sync, with early DL sync only for other test cases.

- In Rel-18, UE capabilities on early DL sync have been introduced by RAN1. It is means that early DL sync will be supported by advance UE and also allow UE doesn’t to excute early DL sync. Meanwhile, even UE supports early DL sync, T/F fine tracking may still need for some cases. Based on this, we think it is better to defined test cases for the CLTM without early DL sync.

- Proposal 1: Prefer to define test cases for CLTM without early DL sync.

- Issue 2-1: Test case list for Subsequent CLTM < Way forward > - Option 1: Define test cases for subsequent CLTM. - For FR1: - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM. - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM. - For FR2: the same proposal as FR1. - Option 2: Not define test case for subsequent CLTM as CLTM.

For subsequent CLTM, the main difference compared to CLTM is that the starting point of procedure and subsequent CLTM does not require to receive RRC configuration. Therefore, the testing design for subsequent CLTM can be based on CLTM. For UE which can pass subsequent CLTM, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure.

In the last meeting, some companies proposed that there is no need to define test cases for subsequent CLTM and UE behavior can be verified through the test cases for CLTM. We know the point is to reduce the number of test cases for the similar requirements and subsequent CLTM is supported if UE supporting CLTM. However, in our understanding, we have defined the requirements for CLTM and subsequent CLTM in clauses 6.3.2.2 and 6.3.2.3, respectively. It is better to define a use case for subsequent CLTM. Option 1 is a good way to consider the trade off between requirements and the number of test cases.

Proposal 2: Prefer to define test cases of subsequent CLTM, and the follow rule can be used

      - For FR1:

      - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM.

      - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM.

      - For FR2: the same proposal as FR1.

Conclusions

In this contribution, we put forward the following proposals on performance part for mobility enhancements.

- Proposal 1: Prefer to define test cases for CLTM without early DL sync.

Proposal 2: Prefer to define test cases of subsequent CLTM, and the follow rule can be used

      - For FR1:

      - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM.

      - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM.

      - For FR2: the same proposal as FR1.

References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2514816, WF on NR_Mob_Ph4_RRM_Part2, CATT