# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2521174

# Dallas, USA, Nov. 17-21, 2025

# Title: 	Discussion on UE demodulation performance requirements for 6Rx

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.2

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the revised WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx

In the following section, we will provide the detailed discussions for 6Rx UE demodulation requirements.

# 2 Discussion

In the last meeting, RAN4 discussed the technical issues in detail, but still have some open issues[2]. Next, we would like to address in the following for 6Rx UE demodulation requirements.

General

Test applicability rule for 6Rx 6 layers performance requirements

- Agreement: - Agree to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. - Proposals: - Discuss whether to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE” in 38.101-4

In the last meeting, RAN4 agreed to define the applicability rules that 6 MIMO layers performance requirements are applicable for UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6. Currently, the device type FWA UEs has been explicitly captured in the RAN2 specification to support 6 MIMO layers. Therefore, we don’t think that to define the explicitly capability rules that 6 layers requirements are only applicable for FWA UE in 38.101-4 is necessary.

Proposal 1. Propose to do not explicitly capture the capability rules that 6 layers requirements are only applicable for FWA UE in 38.101-4.

SDR requirements for 6Rx

MCS value for 256QAM/ 6 layers with scaling factor 1

- Proposals - Option 1: 27 - Option 2: 24 - Option 3: 25

Regarding MCS selection for 256QAM with 6 layers, we have the following simulation results for candidate options.

Table 1. Simulation results for SDR test with 256QAM/6 layers

| MCS | Target SNR (dB) @85% of maxTP |
| --- | --- |
| 24 | 21.6 |
| 25 | 23.2 |
| 27 | 25.2 |

From our simulation results, we think that the candidate options also can be testable, even RAN4 considers additional margins. Therefore, we prefer to consider MCS 27 as our preference.

Proposal 2. Propose to consider MCS 27 for 256QAM/ 6 layers with scaling factor 1.

MCS value for 64QAM/ 6 layers with scaling factor 1

- Proposals - Option 1: 28 - Option 2: 26

Regarding MCS selection for 256QAM with 6 layers, we have the following simulation results for candidate options.

Table 2. Simulation results for SDR test with 64QAM/6 layers

| MCS | Target SNR (dB) @85% of maxTP |
| --- | --- |
| 26 | 17.2 |
| 28 | 19.2 |

From our simulation results, we think that the candidate options also can be testable, even RAN4 considers additional margins. Therefore, we prefer to consider MCS 28 as our preference.

Proposal 3. Propose to consider MCS 28 for 64QAM/ 6 layers with scaling factor 1.

MCS value for 1024QAM/ 2 layers with scaling factor 1

- Proposals - Option 1: 23 - Option 2: 24

Regarding MCS selection for 1024QAM with 2 layers, we have the following simulation results for candidate options.

Table 3. Simulation results for SDR test with 1024QAM/2 layers

| MCS | Target SNR (dB) @85% of maxTP |
| --- | --- |
| 23 | 22.8 |
| 24 | 24.8 |

From our simulation results, we think that the candidate options also can be testable, even RAN4 considers additional margins. Therefore, we prefer to consider MCS 24 as our preference.

Proposal 4. Propose to consider MCS 24 for 1024QAM/ 2 layers with scaling factor 1.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx requirements, the conclusions are:

Proposal 1. Propose to do not explicitly capture the capability rules that 6 layers requirements are only applicable for FWA UE in 38.101-4.

Proposal 2. Propose to consider MCS 27 for 256QAM/ 6 layers with scaling factor 1.

Proposal 3. Propose to consider MCS 28 for 64QAM/ 6 layers with scaling factor 1.

Proposal 4. Propose to consider MCS 24 for 1024QAM/ 2 layers with scaling factor 1.

# 4 References

- RP-250780 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4. Huawei, HiSilicon.

- R4-2514788, Way Forward for [116bis][215] NR_ENDC_RF_Ph4_Demod_6Rx.