# 3GPP TSG-RAN WG4 Meeting # 117                                  R4-2521190

Dallas, USA, Nov. 17-21, 2025

# Title: 	Discussion on performance requirements for 6Rx UE with interference

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.5.4

# Document for:	Approval

# 1 Introduction

In RAN #109 meeting, the new WID of UE performance requirements for 6Rx UE devices (both handheld and FWA), which was approved in [1], wherein the objectives related to 6Rx UE are listed as below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver - Use the Rel-17 interference model for 2/4Rx UE as a baseline - Up to 4 MIMO layers for target cell - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference. - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Use the existing interference model for 2/4Rx UE as a baseline - Up to 2 MIMO layers for each user

In this contribution, we give some initial discussions on the UE performance requirements for 6Rx devices (both handheld and FWA), which include inter-cell interference cancellation and intra-cell inter-user interference cancellation.

# 2 Discussion

As we all know, co-channel inter-cell and intra-cell inter-user interference cancellation performance requirements were introduced in Rel-17. And in Rel-19, RAN4 introduced 6Rx demodulation requirements for PDSCH and CSI performance. With 6Rx, larger performance gain can be verified, due to larger spatial domain freedom compared to 2/4Rx. As a consequence, from demodulation perspective, there are more benefits to define UE performance requirements for 6Rx UE devices under interference scenarios.

## General

Coverage of UE types

- Observation: - It is clearly stated in the approved WID that the 6Rx requirements with interference will cover both FWA and handheld UEs. - Proposals: - Proposal 1: FFS whether to define one set or two sets of requirements for handheld and FWA UEs until there is clear agreement in Rel-19. - Proposal 2: Define only a single requirement for both handheld and FWA devices. - Proposal 3: Define separate requirements for handheld and FWA devices

At the last meeting, it was agreed that one test case is applicable to both the FWA and the handheld in Rel-19 6Rx. Based this agreement, we think that the Rel-20 6Rx with interference could be follow this rule to define one test case under inter-cell and intra- cell interference scenarios.

Proposal 1. Propose to define only one test case for both FWA and handheld UE devices.

Antenna correlation

- Candidate options: - Proposal 1: Follow the decision in the Rel-19 WI ‘NR_ENDC_RF_Ph4-Perf’. - Proposal 2: Evaluate the performance with α = 0 and β = 0.07179 for handheld UE and low correlation for FWA - Proposal 3: For initial simulation alignment, cover both ULA Low and ULA “non-Low” as per outcome of NR_ENDC_RF_Ph4_Demod_6Rx.

In Rel-19 discussion, which agreed to consider α = 0 and β = 0.07179 as default antenna correlation for 4 MIMO layers scenarios. Also, we evaluated the ULA low and ULA with α = 0 and β = 0.07179 scenarios in our contribution [3]. From our simulation results, both ULA low and ULA with α = 0 and β = 0.07179 also achieve the SNR operation point of 70 % maximum throughput. From this point of view, we believe that both ULA low and ULA with α = 0 and β = 0.07179 propagation conditions can be considered as default assumption. However, considering the Rel-19 discussion, we think it is better to align the 6Rx discussion.

Observation 1, From our simulation results, both ULA low and ULA with α = 0 and β = 0.07179 also achieve the SNR operation point of 70 % maximum throughput.

Proposal 2. Propose to follow the decision of antenna correlation in the Rel-19 6Rx.

  - Inter-cell interference cancellation scenario

Propagation condition

- Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

As discussed before, we don’t think that there is necessary to define the separate test case is defined for FWA and handheld UE from Rel-19 6Rx conclusion. Further, when defining 8Rx with interference scenarios in Rel-18, only TDLA 30-10 propagation condition is agreed for all inter-cell and intra-cell test cases. Therefore, from our point of view, we prefer to only consider TDLA30-10 propagation condition for all interference test cases.

Observation 2. Only TDLA30-10 propagation condition is agreed for Rel-18 8Rx with interference and Rel-19 6Rx topics.

Proposal 3. Propose to only consider TDLA 30-10 as default propagation condition for Rel-20 6Rx interference scenarios.

MCS

- Candidate options: - Option 1: MCS 17 for HomNet and MCS 13 for HetNet - Option 2: MCS 17 - Option 3: MCS 17 for rank 2 and MCS 13 for rank 4

In general, we want to define different MCSs for different MIMO layers. MCS 17 for rank 2 and MCS 13 for rank 4 have been verified the performance in our results.

Proposal 4. Propose to consider MCS 17 for rank 2 and MCS 13 for rank 4 on inter-cell interference scenarios.

  - Intra-cell inter-users interference cancellation scenario

Propagation condition

- Candidate options: - Option 1: TDLA30-10 - Option 2: Consider TDLC300-100 and TDLA30-10 for handheld UE. For FWA devices, consider TDLA30-10 only. - Option 3: TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2

As discussed before, we don’t think that there is necessary to define the separate test case is defined for FWA and handheld UE from Rel-19 6Rx conclusion. Further, when defining 8Rx with interference scenarios in Rel-18, only TDLA 30-10 propagation condition is agreed for all inter-cell and intra-cell test cases. Therefore, from our point of view, we prefer to only consider TDLA30-10 propagation condition for all interference test cases.

Proposal 5. Propose to only consider TDLA 30-10 as default propagation condition for Rel-20 6Rx interference scenarios.

Precoding method for the co-scheduled UE

- Candidate options: - Option 1: Random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests - Option 2: Orthogonal precoding for rank 2+2 tests

Based the last meeting agreement, we evaluated the Rank 1+1 and Rank 2+2 test cases, from our simulation results, random precoding with rank 1+1 and orthogonal precoding with rank 2+2 test cases also can achieve the SNR operation point of 70% maximum throughput for IRC receivers.

Proposal 6. Propose to consider orthogonal precoding for rank 2+2 test cases and random precoding with rank 1+1 cases.

MCS for the target UE

- Candidate options: - Option 1: MCS 17 (Table 1) for 2+2 case, and MCS19 (Table 1) for 1+1 case - Option 2: MCS 17 (Table 1) for 2+2 case - Option 3: Cover MCS 13 and MCS 17 for evaluation

We evaluated the different MCSs in our contribution, MCS 13 and MCS 19 for rank 1+1 cases, MCS 13 and MCS 17 for rank 2+2 test cases. All test cases can be achiever the SNR operation point under ULA low and ULA α = 0 and β = 0.07179. Considering the test purpose, we would like to consider higher MCS for rank 1+1 case and low MCS for rank 2+2 test cases. Therefore, MCS 17 (Table 1) for 2+2 case and MCS19 (Table 1) for 1+1 case are our preference.

Proposal 7. Propose to consider MCS 17 (Table 1) for 2+2 cases and MCS19 (Table 1) for 1+1 cases.

  - CQI reporting requirements with inter-cell interference

Whether to introduce CQI reporting requirements with inter-cell interference

- Candidate options: - Option 1: No matter how RAN4 will design the CQI reporting tests for basic 6Rx, there is necessity to define CQI reporting requirements with inter-cell interference. - Option 2: Don’t define CQI reporting requirements with inter-cell interference for 6Rx

In the last meeting, CQI reporting requirements with Rank 4 under static channel is agreed to define in Rel-19 6Rx.  In Rel-18 8Rx with interference, CQI reporting requirements with inter-ell interference is also defined. Therefore, we think that define CQI reporting requirements with inter-cell interference is reasonable.

Proposal 8. Propose to define CQI reporting requirements with inter-cell interference.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for 6Rx UE with interference, the conclusions are:

Observation 1, From our simulation results, both ULA low and ULA with α = 0 and β = 0.07179 also achieve the SNR operation point of 70 % maximum throughput.

Observation 2. Only TDLA30-10 propagation condition is agreed for Rel-18 8Rx with interference and Rel-19 6Rx topics.

Proposal 1. Propose to define only one test case for both FWA and handheld UE devices.

Proposal 2. Propose to follow the decision of antenna correlation in the Rel-19 6Rx.

Proposal 3. Propose to only consider TDLA 30-10 as default propagation condition for Rel-20 6Rx interference scenarios.

Proposal 4. Propose to consider MCS 17 for rank 2 and MCS 13 for rank 4 on inter-cell interference scenarios

Proposal 5. Propose to only consider TDLA 30-10 as default propagation condition for Rel-20 6Rx interference scenarios.

Proposal 6. Propose to consider orthogonal precoding for rank 2+2 test cases and random precoding with rank 1+1 cases.

Proposal 7. Propose to consider MCS 17 (Table 1) for 2+2 cases and MCS19 (Table 1) for 1+1 cases.

Proposal 8. Propose to define CQI reporting requirements with inter-cell interference.

# 4 References

- RP- 252957, WID: NR demodulation performance: Phase 6.

- R4-2514823, WF on NR_demod_Ph6_Ph5.

- R4-251xxxx, Simulation results on performance requirements for 6Rx UE with interference, ZTE Corporation, Sanechips.