3GPP TSG-RAN WG4 Meeting # 117 	    R4-2521204

Dallas, US, 17 – 21 November 2025

Agenda Item:	6.11.7

Source: 	Ericsson

Title:	On test cases for UE-based positioning case 1

Document for:	Approval

# Introduction

In the last meeting it was agreed to define test cases for validation of reporting delay requirement for AI/ML based positioning case 1 [1]. In this contribution details related to the test cases are discussed.

Agreement from RAN4#116bis [1]:

Introduce “case 1” tests only for reporting delay, one for FR1 and one for FR2

- Only connected mode

- Only reporting delay to be tested

- Only for the case with gaps

# Discussion

To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification can be reused. And to define the test case for UE-based positioning case 1 in FR2, test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in the existing specification can be reused. The number of cells in the tests shall however be updated to 4 and the configuration that are not relevant for UE-based positioning case 1 in Table A.6.6.12.1.1-2, Table A.6.6.12.1.1-3, Table A.7.6.9.1.1-2, and Table A.7.6.9.1.1-3 shall be removed. It should also be noted that depending on the signaling designed by RAN2, new parameters that are relevant for testing need to be considered. Moreover, TDL-A and TDL-C channel models shall be considered for FR1 and FR2 testing respectively.

Proposal 1: 4 cell setup is considered for testing UE-based positioning case 1 reporting delay requirement in FR1 and FR2.

Proposal 2: To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3) may need to be considered.

Proposal 3: To define the test case for UE-based positioning case 1 in FR2, test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3) may need to be considered.

Proposal 4: FR1 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-A propagation condition.

Proposal 5: FR2 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-C propagation condition.

# Summary

In this contribution, testing of UE-based positioning case 1 is discussed. The proposals below summarize the discussion presented in this paper.

Proposal 1: 4 cell setup is considered for testing UE-based positioning case 1 reporting delay requirement in FR1 and FR2.

Proposal 2: To define the test case for UE-based positioning case 1 in FR1, test configurations in Table A.6.6.12.1.1-1, general test parameters in Table A.6.6.12.1.1-2, and cell specific test parameters in Table A.6.6.12.1.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.6.6.12.1.1-2 and Table A.6.6.12.1.1-3) may need to be considered.

Proposal 3: To define the test case for UE-based positioning case 1 in FR2, test configurations in Table A.7.6.9.1.1-1, general test parameters in Table A.7.6.9.1.1-2, and cell specific test parameters in Table A.7.6.9.1-3, for NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in the existing specification is reused as a baseline keeping the following aspects in mind:

- configuration that are not relevant for UE-based positioning case 1 in Table A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3 shall be removed.

- depending on the signalling designed by RAN2, new parameters (not existing in A.7.6.9.1.1-2 and Table A.7.6.9.1.1-3) may need to be considered.

Proposal 4: FR1 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-A propagation condition.

Proposal 5: FR2 testing for UE-based positioning case 1 reporting delay requirement is done under TDL-C propagation condition.

# References

- R4-2514631, WF on [116bis][112] R19 AI for air interface, Qualcomm Incorporated.