3GPP TSG-RAN WG4 Meeting # 117	R4-2521207

Dallas, US, 17 – 21 November 2025

Agenda Item:	7.12.3

Source:	Ericsson

Title:	On RRM measurement prediction for AI/ML aided mobility

Document for:	Approval

# Introduction

In this contribution, issues related to RRM measurement prediction for mobility are discussed. Temporal domain prediction case-A and case-B from the core requirement point of view, aspects related to performance monitoring, data collection, and accuracy requirement for predicted measurements are particularly discussed.

# Discussion

## Temporal domain prediction

Temporal domain prediction use cases that are within the scope of Rel. 20 WI are shown in Figure 1 and Figure 2. In temporal domain prediction case-A, the UE performs multiple measurements without skipping any measurement occasion within observation window (OW) to predict RRM measurements to be valid for the time instances within the prediction window (PW). Different to this, in temporal domain prediction case-B the UE may skip performing measurements on some of the measurement occasion within the OW to predict RRM measurements to be valid for the time instances within the PW.

Figure 1. Temporal domain prediction case-A.

From the core requirement point of view, for the scenario where the number of measurement occasions within OW and the number of time instances for which the measurements are to be predicted by the UE within the PW remains same, temporal domain prediction case-B becomes one of the special cases of temporal domain prediction case-A. In this regard, from the core requirement point of view there is no need to differentiate between temporal domain prediction case-A and temporal domain prediction case-B unless a need to differentiate between them is identified based on RAN2 signalling design.

Figure 2. Temporal domain prediction case-B.

From the core requirement point of view there is no need to differentiate between temporal domain prediction case-A and temporal domain prediction case-B for RRM measurement prediction for mobility unless a need to differentiate between them is identified based on RAN2 signalling design.

## Performance monitoring

For performance monitoring of UE-sided model, the WID highlights two key aspects. First, the model related choices for mobility purposes can be up to the UE’s implementation. Second, the performance monitoring of AI/ML model at UE can be performed at the network side. For performance monitoring to be performed at the network, performance metric reported by the UE to the network is vital. Given that the model related choices can be up to the UE’s implementation, ensuring that the metric reported by different implementation choices made by UE vendors are consistent with each other, and defining the requirement such that the performance metric reporting on performed by UE on a timely manner to the network falls within the RAN4 scope. RAN4 should therefore identify the performance metric to be reported for performance monitoring, specify delay requirement for performance metric reporting, and discuss the accuracy of the reported metric for AI/ML aided mobility purposes. The accuracy of the reported metric can be defined in the performance part of the WI, but the other aspects highlighted above can already be discussed as a part of the core requirement.

Model related choices can be up to the UE’s implementation, ensuring that the metric reported by different implementation choices made by UE vendors are consistent with each other, and defining the requirement such that the performance metric reporting on performed by UE on a timely manner to the network falls within the RAN4 scope.

For UE sided model for AI/ML mobility, RAN4 to specify performance metric to be reported for performance monitoring at network side.

RAN4 to define delay requirement for performance metric reporting for performance monitoring of UE sided model at the network node.

## Data collection

Data collection is another important aspect when it comes to training or retraining the model deployed at UE. To perform data collection UE may need some assistance data from the network. Depending on the assistance data and data collection configuration provisioned by the network UE can perform the data collection for which corresponding requirement needs to be specified. One of the important aspects that needs to be considered is the impact of data collection on the any ongoing other RRM procedures at UE. When UE is configured to perform other RRM measurements, for example RLM, data collection procedure shall be down prioritized by the UE.

Depending on the assistance data and data collection configuration provisioned by the network UE can perform the data collection for which corresponding requirement needs to be specified.

When UE is configured to perform other RRM measurements, for example RLM, data collection for UE sided model is down prioritized by UE.

## Accuracy requirement for predicted measurement(s)

One of the open issues from the last meeting is the accuracy requirements for predicted measurement(s). The list of options in WF from RAN4#116bis meeting is copied below for reference purposes.

Way forward from RAN4#116bis [1]

  - Further discuss based on the open issues in [2] and the following way forward:

    - Discuss whether the prediction accuracy has the same or different accuracy requirements as non-AI/ML RRM measurement.

    - Discuss whether the following parameters need to be the side condition of the accuracy requirements:

      - OW/PW length

      - MRRT

      - Skipping pattern

      - UE speed

      - L1 filtering approach and sample number

AI/ML aided mobility may not be supported by all 5G UEs. Contrary to this, all 5G UEs should be capable of performing RRM measurement for mobility and report to the network for mobility decisions to be made. In practice this means that the network will have to manage mobility of UEs that are capable of predicting RRM measurement and UEs that don’t have this capability. From the network point of view, it is important that the measurements reported by UEs, regardless of if they are AI/ML capable or not, meet the same accuracy requirement so that consistent mobility decisions can be made by network for both types of UEs. From the performance requirement point of view accuracy requirement for non-AI/ML-based RRM measurements should therefore also apply to AI/ML-predicted RRM measurements. RAN4 can further discuss how to reuse:

- accuracy requirement for intra-frequency measurement to define accuracy requirement for RRM measurement prediction in temporal domain and;

- accuracy requirement for inter-frequency measurement to define accuracy requirement for RRM measurement prediction in frequency domain for co-located cells.

Network will have to manage mobility of UEs that are capable of predicting RRM measurement and UEs that don’t have this capability.

From the network point of view, it is important that the measurements reported by UEs, regardless of if they are AI/ML capable or not, meet the same accuracy requirement so that consistent mobility decisions can be made by network for both types of UEs.

Accuracy requirement for non-AI/ML RRM measurement apply to predicted RRM measurements. Further discuss how to reuse:

- accuracy requirement for intra-frequency measurement to define accuracy requirement for RRM measurement prediction in temporal domain and;

- accuracy requirement for inter-frequency measurement to define accuracy requirement for RRM measurement prediction in frequency domain for co-located cells.

# Summary

Temporal domain prediction case-A and case-B from the core requirement point of view, aspects related to performance monitoring, data collection, and accuracy requirement for predicted measurements are discussed in this paper. The observations and proposals below summarize the discussion presented in this paper.

- From the core requirement point of view there is no need to differentiate between temporal domain prediction case-A and temporal domain prediction case-B for RRM measurement prediction for mobility.

- Model related choices can be up to the UE’s implementation, ensuring that the metric reported by different implementation choices made by UE vendors are consistent with each other, and defining the requirement such that the performance metric reporting on performed by UE on a timely manner to the network falls within the RAN4 scope.

For UE sided model for AI/ML mobility, RAN4 to specify performance metric to be reported for performance monitoring at network side.

RAN4 to define delay requirement for performance metric reporting for performance monitoring of UE sided model at the network node.

Depending on the assistance data and data collection configuration provisioned by the network UE can perform the data collection for which corresponding requirement needs to be specified.

When UE is configured to perform other RRM measurements, for example RLM, data collection for UE sided model is down prioritized by the UE.

Network will have to manage mobility of UEs that are capable of predicting RRM measurement and UEs that don’t have this capability.

From the network point of view, it is important that the measurements reported by UEs, regardless of if they are AI/ML capable or not, meet the same accuracy requirement so that consistent mobility decisions can be made by network for both types of UEs.

Accuracy requirement for non-AI/ML RRM measurement apply to predicted RRM measurements. Further discuss how to reuse:

- accuracy requirement for intra-frequency measurement to define accuracy requirement for RRM measurement prediction in temporal domain and;

- accuracy requirement for inter-frequency measurement to define accuracy requirement for RRM measurement prediction in frequency domain for co-located cells.

# References

R4-2514827, WF on NR_AIML_Mob_RRM, OPPO.

R4-2513532, Topic summary for [116bis][226] NR_AIML_Mob_RRM, Moderator (OPPO).