3GPP TSG-RAN WG4 Meeting #117	R4-2521215

Dallas-Texas, United States of America, November 17th – 21st, 2025

Source: 	Ericsson

Title:  	On TA command adjust timing for NB-IoT NTN

Agenda Item:	4.26.2

Document for:	Discussion

# Introduction

In this contribution we present our view on “TA command MAC-CE in NB-IoT NTN”.

In RAN1, there is a discussion on a misalignment TS 36.213 and TS 36.133 for the following clauses [1-2]:

| TS 36.213 |  |
| --- | --- |
| 16.1.2 Timing synchronization … For a timing advance command reception ending in DL subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the first available NB-IoT uplink slot following the end of n+12 DL subframe and the first available NB-IoT uplink slot is the first slot of a NPUSCH transmission. |  |

| TS 36.133 |  |
| --- | --- |
| 7.22A.2.1 Timing Advance adjustment delay UE shall adjust the timing of its uplink transmission timing at sub-frame n+12+ k-Offset-r17+1 for a timing advance command received in sub-frame n, where k-Offset-r17 is specified in [2]. |  |

Observation 1: On the “TA command MAC-CE” procedure, beyond the non-consistent use of Koffset, part of the misalignment between TS 36.213 and TS 36.133 is that the procedures are described with respect to different time units. That is, whereas the former one uses "slot", the latter one does it with respect to a "subframe," which is something that needs to be aligned.

This contribution outlines our point of view on on the underlying impacts to RRM requirements.

# Discussion

In our view, updating the RAN1 specification might be better, since when Koffset is used as in the RAN4 spec, the network will know exactly when (i.e., in which uplink subframe) the UE updates the TA. If the timing of the RAN1 spec is used, then the application time depends on the TA, which is unknown to the network.

Observation 2: On the “TA command MAC-CE” procedure description and the misalignment between TS 36.133 and TS 36.213. In our view, updating the RAN1 specification is recommended, since when Koffset is used (as in the RAN4 spec), the network will know exactly when (in which uplink subframe) the UE updates the TA.

Moreover, after having performed an investigation on how this procedure is handled in other RATs across RAN1 and RAN4, we realized that the corresponding RAN1 and RAN4 specs for LTE-MTC-NTN [1-2] and NR-NTN [3-4] are consistently using Koffset, as shown below:

| LTE-MTC-NTN |  |
| --- | --- |
| TS 36.213 clause 4.2.3: “For a BL/CE UE, for a timing advance command received on subframe n, the corresponding adjustment of the uplink transmission timing shall apply for the uplink PUCCH/PUSCH/SRS transmissions in subframe n+6+Koffset.” |  |
| TS 36.133 clause 7.28A.2.1: “The UE shall adjust the timing of its uplink transmission timing at sub-frame n+6+Koffset for a timing advance command received in sub-frame n.” |  |

| NR-NTN |  |
| --- | --- |
| TS 38.213 clause 4.2: “For a timing advance command received on uplink slot n, […] , the corresponding adjustment of the uplink transmission timing applies from the beginning of uplink slot ” |  |
| #### TS 38.133 clause 7.3C.2.1: “UE shall adjust the timing of its uplink transmission timing from the beginning of uplink time slot n+ k+1+2µ for a timing advance command received in time slot n” |  |

Observation 3: On the “TA command MAC-CE” procedure description and the misalignment between TS 36.133 and TS 36.213. Other NTN Radio Access Technologies (RATs) do not have such a misalignment, both LTE-MTC-NTN and NR-NTN consistently use Koffset as in the RAN4 spec.

Based on the previous analysis, to amend the misalignment between TS 36.133 and TS 36.213 on the “TA command MAC-CE” procedure description, we have the following proposal:

Proposal 1:  Send an LS to RAN1 to request RAN1 to update Koffset in TS 36.213 clause 16.1.2 to align with that in TS 36.133 clause 7.22A.2.1 of the RAN4 spec.

# Conclusion

Observation 1: On the “TA command MAC-CE” procedure, beyond the non-consistent use of Koffset, part of the misalignment between TS 36.213 and TS 36.133 is that the procedures are described with respect to different time units. That is, whereas the former one uses "slot", the latter one does it with respect to a "subframe," which is something that needs to be aligned.

Observation 2: On the “TA command MAC-CE” procedure description and the misalignment between TS 36.133 and TS 36.213. In our view, updating the RAN1 specification is recommended, since when Koffset is used (as in the RAN4 spec), the network will know exactly when (in which uplink subframe) the UE updates the TA.

Observation 3: On the “TA command MAC-CE” procedure description and the misalignment between TS 36.133 and TS 36.213. Other NTN Radio Access Technologies (RATs) do not have such a misalignment, both LTE-MTC-NTN and NR-NTN consistently use Koffset as in the RAN4 spec.

Proposal 1:  Send an LS to RAN1 to request RAN1 to update Koffset in TS 36.213 clause 16.1.2 to align with that in TS 36.133 clause 7.22A.2.1 of the RAN4 spec.

# References

- TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures,” v 19.1.0.

- TS 36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management,” v19.2.0.

- TS 38.213, “NR; Physical layer procedures for control,” v 19.1.0.

- TS 38.133, “NR; Requirements for support of radio resource management,” v19.1.0.