3GPP TSG-RAN WG4 Meeting #117	R4-2521221

Dallas, Tx, USA, 17th – 21st November 2025

Source: 	Ericsson

Title:  	Discussion on performance requirements of FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	6.10.2.2

Document for:	Discussion

# Introduction

In this contribution, we herein present our views and proposals on the remaining open issues relevant to performance requirements.

# Discussion

### Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing CSSF

Issue 2-1-3: Cell configurations in TCs for CSSF solution 3

<Way Forward>: - Continue discussions on the test case when one inter-RAT E-UTRAN Cell configured and consider the impact to core requirements.

The issue raised in the last meeting concerns whether the CSSF Solution 3 is applicable to inter-RAT configurations. The previous agreements related to inter-RAT MOs are duplicated below for reference.

Issue 1-2-1: applicability limitation for solution 3 Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

We understand that the cause of the issue is related to the statement "CSSF does not apply for inter-frequency MO with no measurement gap", which may be interpreted in two ways:

- Option 1: CSSF Solution 3 is applied to SCCs, but not to inter-RAT MOs, within the inter-RAT configuration, or

- Option 2: CSSF Solution 3 is not applicable to inter-RAT configurations at all.

From our perspective, Option 1 is preferred, as CSSF Solution 3 provides benefits for SCCs within inter-RAT configurations.

Proposal 1:  CSSF Solution 3 is applied to SCCs, but not to inter-RAT MOs, within the inter-RAT configuration.

# Conclusion

Proposal 1:  CSSF Solution 3 is applied to SCCs, but not to inter-RAT MOs, within the inter-RAT configuration.

# References

- R4-2514800, WF for NR_RRM_Ph5_Part1, Ericsson