3GPP TSG-RAN WG4 Meeting #117	R4-2521226

Dallas, USA, 17 November – 21 November 2025

Agenda Item:	6.20.4

Source:	Ericsson

Title:	SAN demodulation requirements for IoT-NTN

Document for:	Discussion

# Introduction

RAN4#116bis agreed with the way forward on the SAN demodulation requirements for IoT-NTN phase 3 [1], where RAN4 is going to specify SAN demodulation requirement of NPUSCH format 1 with OCC2 by the SNR to achieve 70% of the maximum throughput for OCC sequence indexes 0 and 1.

This contribution provides our updated simulation result to define the NPUSCH format 1 demodulation requirements.

# Simulation results

Table 1 shows our updated simulation result of NPUSCH format 1 with OCC2 for alignment. According to the WF, the summary shows SNR to achieve 70% of the maximum throughput per OCC sequence index (UE1 and UE2).

Table 1	NPUSCH format 1 with OCC-2 simulation results for alignment.

| Tx antennas | Rx antennas | SCS | Sub-carriers | Propagation channel | FRC | Repetition number | SNR to achieve 70% of the peak rate (from UE1) | SNR to achieve 70% of the peak rate (from UE2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 2 | -4.12 | -4.17 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 | 2 | -7.41 | -7.38 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 8 | -12.14 | -12.24 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 8 | -15.13 | -15.19 |

It is observed from the simulation results summary in RAN#116bis [2] that there are large spans among companies’ simulation results. If the spans of the companies’ results are still large in RAN4#117, we propose to add extra margin as same as Rel-18 SAN NPUSCH format 1 demodulation requirements [3], to define the SAN demodulation requirements for NPUSCH format 1 with OCC2.

Proposal 1: If the span of the companies’ results is large for NPUSCH format 1 simulation result, add extra margin to specify the SAN demodulation requirements, as same as the SAN NPUSCH format 1 demodulation requirements in Rel-18.

For information, Table 2 shows our NPUSCH format 1 simulation result without OCC2 for SCS=15kHz single tone. Note this simulation configures 16 repetitions (cf. 8 repetitions for OCC2) because of no OCC2.

Table 2	NPUSCH format 1 without OCC-2 simulation results (for information).

| Tx antennas | Rx antennas | SCS | Sub-carriers | Propagation channel | FRC | Repetition number | SNR to achieve 70% of the peak rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 16 | -11.1 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 | 16 | -13.97 |

# Summary

Proposals:

Proposal 1: If the span of the companies’ results is large for NPUSCH format 1 simulation result, add extra margin to specify the SAN demodulation requirements, as same as the SAN NPUSCH format 1 demodulation requirements in Rel-18.

# References

- R4-2515160, “Way Forward for [116bis][312] Rel-19 Demodulation_Part2”, Nokia.

- R4-2514152, “Simulation summary on SAN demodulation requirements for IoT-NTN”, ZTE Corporation, Sanechips.

- R4-2312791, “Summary of SAN simulation results for IoT-NTN”, Ericsson.

# Appendix

Table A.7.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A7-1 | A7-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 |
| Number of allocated subcarriers | 1 | 12 |
| Diversity | No | No |
| Modulation | BPSK | QPSK |
| ITBS / IRU | 0 / 1 | 9 / 0 |
| Payload size (bits) | 32 | 136 |
| Allocated resource unit | 2 | 1 |
| Code rate (target) | 1/3 | 2/3 |
| Code rate (effective) | 0.29 | 0.56 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 |
| Number of code blocks - C | 1 | 1 |
| Total number of bits per resource unit | 96 | 288 |
| Total symbols per resource unit | 96 | 144 |
| Channel estimation length (ms) Note 1 | 16 | 2 (when repetition = 2) 4 (when repetition > 2) |

Table A.16.1-1: FRC parameters for NB-IoT NPUSCH format 1

| Reference channel | A16-1 | A16-2 | A16-3 | A16-4 | A16-5 | A16-6 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 3.75 | 15 | 15 | 15 | 15 | 15 |
| Number of allocated subcarriers | 1 | 1 | 3 | 6 | 12 | 12 |
| Diversity | No | No | No | No | No | No |
| Modulation | BPSK | BPSK | QPSK | QPSK | QPSK | QPSK |
| ITBS / IRU | 0 / 1 | 0 / 1 | 3 / 0 | 7 / 0 | 9 / 0 | 5/4 |
| Payload size (bits) | 32 | 32 | 40 | 104 | 136 | 424 |
| Allocated resource unit | 2 | 2 | 1 | 1 | 1 | 5 |
| Code rate (target) | 1/3 | 1/3 | 1/3 | 1/3 | 2/3 | 1/3 |
| Code rate (effective) | 0.29 | 0.29 | 0.22 | 0.44 | 0.56 | 0.34 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 0 | 0 | 0 | 0 | 0 |
| Number of code blocks - C | 1 | 1 | 1 | 1 | 1 | 1 |
| Total number of bits per resource unit | 96 | 96 | 288 | 288 | 288 | 1440 |
| Total symbols per resource unit | 96 | 96 | 144 | 144 | 144 | 720 |
| Channel estimation length (ms) Note 1 | 16 | 4 | 4 | 4 | 2 (when repetition = 2) 4 (when repetition > 2) | 4 |
| Note 1: Channel estimation lengths are included in the table for information only. |  |  |  |  |  |  |