3GPP TSG-RAN WG4 Meeting #117	R4-2521229

Dallas, USA, 17 November – 21 November 2025

Agenda Item:	6.16.3

Source:	Ericsson

Title:	UE demodulation requirements for LP-WUS/WUR

Document for:	Discussion

# Introduction

RAN4#116bis agreed with the way forward on LP-WUS demodulation requirements [1]. This contribution discusses the open issues on the UE demodulation requirements for LP-WUS/WUR.

# Discussion

## Receiver assumption

### Receiver type in FR2

Issue 2-1-8: Receiver type for FR2 - Agreement: Consider at least OFDM receiver for FR2. FFS OOK receiver. Issue 2-1-16: Whether to define single requirement for OFDM and OOK receivers Proposals - Option 1: Define two sets of requirements (one for each receiver type: OOK and OFDM). - Option 2: RAN4 to study the performance for both receiver types (OOK receiver and OFDM receiver) and further discuss based on simulation results if one or two sets of requirements are to be defined Issue 2-1-17: Whether to define applicability rules for OFDM and OOK receivers Proposals - Option 1: Don’t define any capability rules related to OFDM and OOK receivers for demodulation requirements. - Option 2: Defer the discussion of applicability rules until simulation results have been analysed for both receivers.

RAN4 endorsed the Draft CR to TR 38.774 on LP-WUS FR2 in RAN4#116bis [2], where it is mentioned that:

- For FR2, only OFDM based LR type is pursued, and the NF + IM value is agreed as 12dB which represents a 1dB degradation in NF compared with MR assumption at 28GHz.

Observation 1: RAN4 RF has defined the WUS receiver requirements for FR2 with OFDM-WUR only.

According to the RF agreements, we propose defining the WUS reception performance with OFDM-based LR only for FR2.

Proposal 1: Assume Type 2 (OFDM-WUR) only for WUS demodulation requirements in FR2.

### Requirements for OFDM-based receiver

Issue 2-1-13: Consideration of one-shot and repetition transmission mode for OFDM-based receiver Proposals - Option 1 - RAN4 to specify demodulation requirements for both one-shot and full-repetition cases for OFDM-based receivers.

Another open issue is how to define the WUS demodulation requirements for OFDM-based receivers. According to RAN1 design on WUS transmission, Nseq OFDM sequences can be overlaid per WUS ON segment to transmit the WUS information bits. For example, since RAN4#116bis agreed to configure Nseq=4 for FR1, each ON segment can transmit 4 difference overlaid sequences that correspond to 2 bits (log2(Nseq) = log2(4) = 2). When the information bits are 5 (K=5), we need 3 ON segments to transmit the information bits with the overlaid sequences. Since we agreed to configure 64 bits after line coding, there are 32 ON segments. As the results, it is possible to transmit the information bits 10 times (= floor(32/3)) with overlaid sequences.

Observation 2: Information bits can be transmitted 10 times with the overlaid sequences for Mwus=4, 16 OFDM symbols, Nseq=4, and 5 information bits (K=5).

Similarly, in the case of MWUS=1, 24 OFDM symbols, Nseq=2, and K=2, one WUS ON segment can transmit 1 bit and there are 12 WUS ON segments. Since K=2, it is possible to transmit the information bits 6 times (= floor(12/2)) with overlaid sequences.

Observation 3: Information bits can be transmitted 6 times with overlaid sequences for Mwus=1, 24 OFDM symbols, Nseq=2, and 2 information bits (K=2).

According to TR 38.774 6.3, RAN4 summarized the simulation results of OOK-based and OFDM-based receivers with the assumption of 4 overlaid sequences with Mwus=4 for FR1. The TR assumes the one-shot reception with OFDM-based receiver and then concludes to specify the same SNR to achieve 1% of MDR of LP-WUS for both OOK-based and OFDM-based receivers.

Observation 4: RAN4 RF assumed one-shot reception for OFDM-based receiver to define RF receiver requirements such as REFSENS.

Since there is no signaling from the network to indicate how to receive the overlaid sequences on WUS ON segment, it is up to UE implementation how to receive them. Considering specifying the minimum requirements, we propose defining the requirements with one-shot reception as same as UE RF RESENSE.

Proposal 2: For OFDM-based WUS receiver, specify the WUS reception requirements based on one-shot reception of overlaid sequences.

## Simulation results

Table 1 summarizes the current simulation assumptions for the wake-up signal reception requirements in our understanding. Table 2 clarifies the assumption of the center frequency for each frequency range and SCS configuration to derive the frequency offset. Table 3 also summarizes our simulation results.

Table 1	Simulation assumption for the wake-up signal reception requirements.

| Parameters | Mwus=1 | Mwus=4 |
| --- | --- | --- |
| MWUS | 1 | 4 |
| Wake-up information length (K) | 2 bits | 5 bits |
| Number of overlaid sequences (Nseq) | 2 | 4 |
| Number of root sequence numbers (Nroot) | 1 | 1 |
| root1 | 1 | 1 |
| Number of bits after line coding (G0 = Nos x MWUS, in TS 38.212 7.4.3) | 24 bits (1*24) | 64 bits (4*16) |
| Number of OFDM symbols | 24 | 16 |
| Code rate (K/G0) |  |  |
| Reception time offset | 0.5us | 1us |
| Reception frequency offset | 10ppm for OOK receiver 5ppm for OFDM receiver | 10ppm for OOK receiver 5ppm for OFDM receiver |
| Channel model | TDLA30-75 | TDLA30-10 |
| Frequency range and SCS | FR2 SCS=120kHz | FR1 SCS=15kHz FR1 SCS=30kHz |
| Antenna configuration | 1x1 | 1x1 |
| LP-SS | Configured | Configured |

Table 2	Assumption of carrier frequency for each numerology.

| Frequency range and SCS | Carrier frequency |
| --- | --- |
| FR1 SCS=15kHz | 900MHz |
| FR1 SCS=30kHz | 2.6GHz |
| FR2 SCS=120kHz | 28GHz |

Table 3	Summary of WUS reception simulation results.

| SCS | Mwus | Information bits | Number of bits after line coding | Channel model | Frequency offset | Receiver type | SNR to achieve 1% missed detection rate (MDR) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15kHz | 4 | 5 | 64 | TDLA30-10 | 9 kHz | OOK | TBD |
| 15kHz | 4 | 5 | 64 | TDLA30-10 | 4.5 kHz | OFDM single sequence | TBD |
|  |  |  |  |  |  | OFDM full sequence | TBD |
| 30kHz | 4 | 5 | 64 | TDLA30-10 | 26 kHz | OOK | TBD |
| 30kHz | 4 | 5 | 64 | TDLA30-10 | 13 kHz | OFDM single sequence | TBD |
|  |  |  |  |  |  | OFDM full sequence | TBD |
| 120kHz | 1 | 2 | 24 | TDLA30-75 | 280 kHz | OOK | TBD |
| 120kHz | 1 | 2 | 24 | TDLA30-75 | 140 kHz | OFDM single sequence | TBD |
|  |  |  |  |  |  | OFDM full sequence | TBD |

# Summary

Observations:

Observation 1: RAN4 RF has defined the WUS receiver requirements for FR2 with OFDM-WUR only.

Observation 2: Information bits can be transmitted 10 times with the overlaid sequences for Mwus=4, 16 OFDM symbols, Nseq=4, and 5 information bits (K=5).

Observation 3: Information bits can be transmitted 6 times with overlaid sequences for Mwus=1, 24 OFDM symbols, Nseq=2, and 2 information bits (K=2).

Observation 4: RAN4 RF assumed one-shot reception for OFDM-based receiver to define RF receiver requirements such as REFSENS.

Proposals:

Proposal 1: Assume Type 2 (OFDM-WUR) only for WUS demodulation requirements in FR2.

Proposal 2: For OFDM-based WUS receiver, specify the WUS reception requirements based on one-shot reception of overlaid sequences.

# References

- R4-2514813, “WF for [116bis][220] NR_LPWUS_Demod”, Moderator (Qualcomm).

- R4-2514611, “Draft CR to TR 38.774 on LP-WUS FR2 part update”, Samsung.

- TR 38.774, “Radio Frequency (RF) requirements for Low-Power Wake-up Signal and Receiver”, V19.0.0, 3GPP.