# 3GPP TSG-RAN WG4 Meeting #117		R4-2521242

# Dallas, USA, Nov. 17-21, 2025

Source: 	OPPO

Agenda Item:	8.8.1

Title: 	General issues for AI/ML in RAN4

Document for:	Approval

- Introduction

For 6G, the SID [1] of 6GR was approved in RAN#108 meeting. The objectives related to AI air interface for 6GR are listed as follows.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we would like to provide our viewpoints on some general issues for AI/ML in RAN4.

- Discussion

  - UE AI category

In recent years, more and more user devices are being equipped with AI capabilities. It is important to distinguish between devices with different levels of AI capability to define how they can best use AI, e.g., considering different computing capabilities, available computing resources, energy consumptions, memory capacities, supported models/backbones. This will help ensure that devices with varying AI capabilities can effectively support the AI functions they need.

Taking available computing resources as an example, in current terminal devices, most computing resources are located outside the modem. Computing resources inside the modem, especially dedicated processors like GPUs or NPUs, are very limited. Moreover, when a UE lacks dedicated AI processors or such resources are constrained, non-AI-specific processors (CPU) may be used to run simple neural network models, like simplified MLP models. As a result, for wireless AI use cases, available computing resources may fall into: (1) AI computing resource on device, but outside modem, e.g., NPU on device for AI operation. (2) Non-AI computing resource on device, but outside modem, e.g., normal computing resource on device. (3) AI computing resource inside modem, e.g., the NPU inside modem for AI operation. (4) Non-AI computing resource inside modem, e.g., normal computing resource inside modem

Figure 1: Computing resources on chip

Observation 1: 	For computing resources, in current terminal devices, most computing resources are located outside the modem. Computing resources inside the modem, especially dedicated processors like GPUs or NPUs, are very limited.

Observation 2: 	For wireless AI use cases, available computing resources may fall into:

- AI computing resource on device, but outside modem

- Non-AI computing resource on device, but outside modem

- AI computing resource inside modem

- Non-AI computing resource inside modem

Besides, another example could be UE with different AI capabilities may support different models or backbones. Some UE may be able to support relatively complex models, while others may only support simpler ones. If RAN4 define reference models without taking into account the diverse AI capabilities of UE, it could lead to AI features being available only to high-end devices.

Observation 3: 	If RAN4 defines reference models without taking into account the diverse AI capabilities of UE, it could lead to AI features being available only to high-end devices.

From our understanding, it would be helpful to define UE AI categories in RAN4. Following aspects could be considered:

- Computing Capability: Different tiers correspond to different computing capabilities supported by UE, e.g., the number of supported AI processing units by UE.

- Energy Consumption: Different tiers correspond to different power consumptions supported by UE, e.g., X TOPS/W is supported by UE.

- Memory Capacity: Different tiers correspond to different available memory size supported by UE, e.g., the number of memory units supported by UE.

- Types of Computing Resources: For example, non-AI specific resources (like CPUs) and AI-dedicated resources (like GPUs) supported by UE.

- Location of Computing Resources: For example, computing resources located inside and outside the modem.

- Supported Reference Models: This refers to the types of model architectures supported by UE, such as MLP, CNN, or Transformer-based models.

- Note: details related to AI categories (metrics related to AI/ML complexity supported by UE/NW) could be postponed to Q2 2026.

Proposal 1: 	For 6G, RAN4 should define UE AI categories for different devices. Details related to UE AI categories could be postponed to Q2 2026.

  - AI model complexity

In 5G-A, for the CSI compression use case, the introduction of 2-sided models has led RAN4 to study standardized models in order to ensure testability. Progress has been made in R19, and work will continue in R20.

In 6G, further research is needed on model standardization. From our understanding, the discussion about AI models in RAN4 should focus on following two aspects:

- Effective Model Design: As 6G marks the start of a new generation of communication systems, it is important to carefully compare and analyze the model designs for different use cases. This will help ensure that the reference models developed by RAN4 are more practical and feasible for field implementation.

- Constraints on Model Deployment: As a common issue for model design, RAN4 needs to study and define what makes a model "complex" or "simple”. An example is shown in Table 1, the complexity of a model can be determined by a combination of factors such as number of model parameters, model computing complexity, model size, training data requirement, inference latency, and model generalization. AI models with different complexities could be supported by UEs with different AI categories.

|  | "Simple model” | "Complex model" |
| --- | --- | --- |
| Num of model parameter | Low, e.g., <1M | High, e.g., 1M ~10M |
| Model computing complexity | Low, e.g., <1M FLOPs, or [1M FLOPs ~10M FLOPs] | High, e.g., [1M FLOPs ~10M FLOPs], or 10M FLOPs ~100M FLOPs |
| Model size | Small, e.g., <1MB | Large, e.g., 1MB ~10MB |
| [Inference latency] | Low | High |
| [Training data requirement] | Low | High |
| [Generalization] | Sufficient for simple tasks | High potential for complex tasks |

Proposal 2: 	RAN4 should define AI model complexities, by considering: number of model parameters, model computing complexity, model size, data requirement, inference latency, and model generalization.

Proposal 3: 	AI models with different complexities could be supported by UEs with different AI categories.

- Conclusion

In this contribution, general issues regarding the UE AI category and AI model complexity are analysed and got the following proposals/observations:

Proposal 1: 	For 6G, RAN4 should define UE AI categories for different devices. Details related to UE AI categories could be postponed to Q2 2026.

Proposal 2: 	RAN4 should define AI model complexities, by considering: number of model parameters, model computing complexity, model size, data requirement, inference latency, and model generalization.

Proposal 3: 	AI models with different complexities could be supported by UEs with different AI categories.

- References

- RP-251881, New SID: Study on 6G Radio, RAN#108, June 2025