3GPP TSG-RAN WG4 Meeting #117	R4-2521305

Dallas, USA 17th – 21st November 2025

Source: 	Nokia

Title: 					PC1.5 2Tx A-MPR simulations

Agenda Item: 			7.1.4

Document for:	Discussion

# 1	Introduction

We provide A-MPR simulation results on power class PC1.5 and 2Tx configuration for NS_03 and NS_05. We model the 2Tx configuration by using a 1Tx configuration and adding a 0.7 dB offset on top of it. We are assuming the offset does not change with the antenna isolation assumptions. The presented A-MPR triangles are valid for large FWA form factor i.e., MPR presented in Table 6.2D.2-3 [TS 38.101-1] is used while calculating the A-MPR.

# 2	Simulation assumptions and scenario

We model the 2Tx configuration by using a 1Tx transmitter and adding a 0.7 dB offset on top of it. We calibrate the 1Tx transmitter assuming the outer MPR for 1Tx PC1.5 is the same as for 1Tx transmitters of PC1, PC2 and PC3. Our A-MPR triangles are presented for the large FWA form factor, that is, the triangles show the A-MPR when it exceeds the MPR for large FWA form factor in Table 6.2D.2-3 of TS 38.101-1. We use the following simulation assumptions and parameters.

- NS_03 for n25 and NS_05 for n1

- WOLA processing

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc, CIM5 -70dBc

## 2.1	NS_03

Even though the NS_03 emission requirement includes 45 MHz CBW, we only simulate up to 40 MHz because we focus on n25, and in n25 45 MHz CBW is only for DL.

Table 6.5.2.3.3-1: Additional requirements for "NS_03" and “NS_03U”

| ΔfOOB MHz | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20, 25, 30, 35, 40, 45 |  |
| 0-1 | -13 | -13 | 1 % of channel BW |
| 1-6 | -13 |  | 1 MHz |
| 6-10 | -25 |  |  |
| 1-BWChannel |  | -13 |  |
| BWChannel-(BWChannel+5) |  | -25 |  |

## 2.2	NS_05

Table 6.5.3.3.4-1: Additional requirements for "NS_05" and “NS_05U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 |  |
| 1884.5 ≤ f ≤ 1910 | -30 | 1 MHz |
| 1910 ≤ f ≤ 1915.7 | -25 |  |
| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
|  | 10, 15, 20 |  |
| 1884.5 ≤ f ≤ 1906.6 | -30 | 1 MHz |
| 1906.6 ≤ f ≤ 1915.7 | -25 |  |

# 3	Simulation results

In this section we provide highlights of the simulation results. We provide the complete sets of A-MPR triangles in the Appendix. The triangles show the A-MPR when it exceeds the MPR for large FWA form factor in Table 6.2D.2-3 of TS 38.101-1.

## 3.1	NS_03

The simulated A-MPR exceeds the MPR only in very small parts of the inner region of the triangles and it is rather small, at 0.4 dB at maximum. In Fig. 1 we show some examples of this. CBWs smaller than 20 MHz do not have any A-MPR.

Figure 1: Examples of the simulated A-MPR to satisfy the NS_03 additional SEM.

Observation 1: Small A-MPR, 0.4 dB at maximum, is simulated for NS_03. The A-MPR is limited to very small part of the inner region and is only for CBWs of 20 MHz and larger.

## 3.2	NS_05

In Fig. 2 we show examples of the simulated A-MPR for OFDM QPSK at the lower edge of the operating band n1. No A-MPR is needed when the lower edge of the channel is at 1940 MHz or above.

Fc=1922.5MHz

Fc=1930MHz

Fc=1927.5MHz

Fc=1925MHz

Figure 2: Examples of the simulated A-MPR to satisfy the NS_05 additional spurious emission limit. The channels are at the lower edge of the operating band n1.

Observation 2: A-MPR is needed for NS_05 when the channel is at the lower edge of the operating band n1.

Observation 3: No A-MPR is needed for NS_05 when the lower edge of the channel is at 1940 MHz or above.

# 4	Conclusions

We have provided simulation results on PC1.5 2Tx A-MPR for NS_03 and NS_05. We are assuming the offset does not change with the antenna isolation assumptions. The presented A-MPR triangles are valid for large FWA form factor i.e., MPR presented in Table 6.2D.2-3 [TS 38.101-1] is used while calculating the A-MPR. We have made the following observations based on our results.

Observation 1: Small A-MPR, 0.4 dB at maximum, is simulated for NS_03. The A-MPR is limited to very small part of the inner region and is only for CBWs of 20 MHz and larger.

Observation 2: A-MPR is needed for NS_05 when the channel is at the lower edge of the operating band n1.

Observation 3: No A-MPR is needed for NS_05 when the lower edge of the channel is at 1940 MHz or above.

# 5	References

[1] R4-2515087, Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116bis

# 6	Appendix: A-MPR triangles

The triangles show the A-MPR when it exceeds the MPR for large FWA form factor in Table 6.2D.2-3 of TS 38.101-1.

## 6.1	NS_03

## 6.2	NS_05