3GPP TSG-RAN WG4 Meeting #117	R4-2521385

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	6.11.8

Source:	Huawei, HiSilicon

Title:	Discussion on demodulation and/or CSI reporting requirements for CSI prediction

Document for:	Discussion

# Introduction

According to WF in RAN4 #116bis [1], we discuss the remaining issues related to demodulation and/or CSI reporting requirements for CSI prediction.

# Simulation results

RAN4 #115 Updated simulation assumptions for AIML CSI prediction [2]: Table 1: Updated simulation assumptions for AIML based CSI prediction Parameter; Value. Duplex, Waveform; FDD, TDD, OFDM. Carrier frequency; 2GHz for FDD 4GHz for TDD. Subcarrier spacing; 15kHz for FDD, 30kHz for TDD. Bandwidth; 10MHz for FDD, 40MHz for TDD. Symbol; 1 symbol for step-1 and step-2 simulation. Number of subbands; 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD. gNB TX antennas; Baseline: 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements Optional: 32(N1,N2)=(4,4) as defined in Rel-15 type1. UE RX antennas; 4. Channel model; TDLA30 with XP medium correlation as baseline. Doppler spread; 20Hz, 50Hz. Channel estimation for CSI prediction; Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration). CSI-RS configuration; For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12).. Rank per UE; Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline. CSI feedback assumption; Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account. Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed. Test setup(timeline); Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration. SNR assumption; Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions. CSI feedback overhead; PC7 as baseline. Baseline; FFS - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements). KPI; Step-1: - check SGCS before CSI feedback - SGCS is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS after CSI feedback - Encouraged companies to provide the following two SGCS metrics: - SGCS 2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. - SGCS 3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-3: - check TPs after CSI feedback for both AI based CSI prediction and the baseline(Random PMI with Rel-15 Type I single panel codebook), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction. - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 - FDD with 32Tx4Rx: Table 6.3.3.1.4-1: - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 - TDD with 32Tx4Rx: Table 6.3.3.2.4-1:. Model input type; Raw channel matrix. Model out type; Raw channel matrix. Observation window; Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms. Prediction window; Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms. Training data and test data; Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz,); 20Hz Doppler spreads. 50Hz Doppler spreads.. Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models..

According to the baseline simulation assumptions for AI CSI prediction, we provide the simulation results of step-1 in the following.

Proposal 1: For step-1, SGCS performance results of AI CSI prediction are provided in Table 1.

Table 1. SGCS performance results of AI CSI prediction (step-1)

|  | 20Hz | 50Hz |
| --- | --- | --- |
| Rank 1 | 0.9985413 | 0.8673333 |
| Rank 2 | 0.99800384 | 0.8166052 |

It is observed that acceptable SGCS performance can be achieved with 20Hz Doppler spread. There is a degradation in SGCS performance at 50Hz. At 100Hz, there is an extremely significant degradation in SGCS performance.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For step-1, SGCS performance results of AI CSI prediction are provided in Table 1.

Table 1. SGCS performance results of AI CSI prediction (step-1)

|  | 20Hz | 50Hz |
| --- | --- | --- |
| Rank 1 | 0.9985413 | 0.8673333 |
| Rank 2 | 0.99800384 | 0.8166052 |

# References

- R4-2514631, “WF on AI ML for air interface study”, 3GPP RAN4 #116bis, 2025.

- R4-2508102, “Updated simulation assumptions for AIML CSI prediction”, 3GPP RAN4 #115, 2025.