3GPP TSG-RAN4 Meeting #117	R4-2521405

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.104 | CR | 0749 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.104 for Rel-19 BS MMSE-IRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Need to align the Notes in Table: 8.2.16.1-1 across 38.104, 38.141-1 and 38.141-2. - Need to add the missing applicability for FDD in Note2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the Notes in Table: 8.2.16.1-1, including adding the applicability of requirements for FDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for FDD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The corresponding draftCR R4-2514283 has been endorsed in RAN4#116bis |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

sssssssssssssssssssssssss

==============First change==============

### 8.2.16	Enhanced performance requirements for PUSCH with synchronous interference

#### 8.2.16.1	General

The performance requirement of PUSCH with enhanced performance requirements is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ retransmissions.

Table: 8.2.16.1-1 Test parameters with enhanced performance requirements

| Parameter |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 (Note 1) |  | Interferer 2 (Note 1) |
| Transform precoding |  | Disabled |  |  |  |
| Default TDD UL-DL pattern (Note 2) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |  |  |  |
| INR (dB, Note 3) | Set 1 | N/A | 5.35 for 2Rx tests 7.28 for 4/8 RX tests | N/A for 2Rx tests -2.52 for 4/8 RX tests |  |
|  | Set 2 | N/A | 9.82 for for 2Rx tests 12.38 for 4/8 RX tests | N/A for 2Rx tests -0.97 for 4/8 RX tests |  |
| Interference Model |  | N/A | As specified in B.9.2 |  |  |
| HARQ | Maximum number of HARQ transmissions | 4 | N/A |  |  |
|  | RV sequence | 0, 2, 3, 1 | N/A |  |  |
| DM-RS | DM-RS configuration type | 1 |  |  |  |
|  | DM-RS duration | single-symbol DM-RS |  |  |  |
|  | Additional DM-RS position | pos1 |  |  |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |  |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |  |  |
|  | DM-RS port | {0} |  |  |  |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 |  | NID0=2, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |  |  |  |
|  | Start symbol | 0 |  |  |  |
|  | Allocation length | 14 |  |  |  |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |  |  |  |
|  | Frequency hopping | Disabled |  |  |  |
| Code block group based PUSCH transmission |  | Disabled |  |  |  |
| NOTE 1: One explicit interferer, i.e., interferer 1, is modelled for tests with 2 RX antennas. Two explicit interferers are modelled for tests with 4 or 8 RX antennas. NOTE 2: The same requirements are applicable to FDD and TDD with different UL-DL pattern NOTE 3: The respective received energy of each interferer relative to noise is defined by its associated INR value as specified in Annex B.9.1. INR set 1 and set 2 are derived respectively for homogeneous and heterogeneous network scenarios. NOTE 4: All cells are time-synchronous. |  |  |  |  |  |

#### 8.2.16.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.16.2-1 to 8.2.16.2-8 at the given SNR. FRCs are defined in annex A.

Table 8.2.16.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-8 | pos1 | 14.2 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.1 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.1 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.1 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-10 | pos1 | 14.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 11.1 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.6 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.8 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-11 | pos1 | 14.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.8 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.4 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.5 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-14 | pos1 | 14.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.8 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.5 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-5: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-8 | pos1 | 14.0 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.6 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.4 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.2 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.1 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-6: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-10 | pos1 | 14.2 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.8 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.9 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.7 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.2 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-7: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-11 | pos1 | 14.3 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.7 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.5 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.7 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

Table 8.2.16.2-8: Minimum requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas (Note 1) | Number of RX antennas (Note 1) | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) |  | INR Set | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Tested signal | Interferer |  |  |  |  |  |
| 1 | 2 | Normal | TDLC300-100 Low | TDLC300-100 Low | Set 1* | 70% | G-FR1-A4-14 | pos1 | 14.5 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 14.4 |
|  | 4 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 10.8 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 10.8 |
|  | 8 |  | TDLC300-100 Low |  | Set 1* |  |  |  | 6.9 |
|  |  |  | TDLA30-10 Low |  | Set 2 |  |  |  | 6.6 |
| Note*: Not applicable for Local Area BS. |  |  |  |  |  |  |  |  |  |
| Note 1: Antenna configuration applies for each of the tested signal and interferers. Note 2: The propagation conditions for the tested signal and interferers are statistically independent. |  |  |  |  |  |  |  |  |  |

==============End of change==============