3GPP TSG-RAN WG4 Meeting #117	R4-2521412

Dallas, USA, November 17 – 21, 2025

Source:	Nokia

Title:	On PDSCH Requirements for Non-Collocated Intra-Band EN-DC/NR-CA Phase 2

Agenda item:	6.2.3

Document for:	Discussion

Introduction

The discussions on the Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2 during RAN4#116bis have led to agreements as documented in [1] and summarized below:

Topic #2: UE demodulation requirements ## Sub-topic 2-1 Test scope Agreement: Test scenario Define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario. - Specify FR1 TDD + FR1 TDD only. Agreement: Received time difference between two carriers Configure the timing offset of the SCell from the PCell is 33 µs. Agreement: Channel bandwidth and SCS Configure CBW=40MHz and SCS=30kHz for both CCs. ## Sub-topic 2-2 Simulation assumption Agreement: Channel model and antenna configuration Use the static channel model specified in TS 38.101-4 B.1.1. - 2x4 for CC with rank 2 - 4x4 for CC with rank 4 Agreement: How to choose MCS/rank The final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM). Agreement: Tx EVM assumption Assume Tx EVM=3% for both CCs. ## Sub-topic 2-3 Applicability Agreement: Separation of two CCs Send LS to RAN5 as same as Rel-18 NonCol_intraB_ENDC_NR_CA UE demodulation performance part in RAN4#117. Qualcomm volunteers to send LS to RAN5. Agreement: Test applicability Type 4b UE demodulation requirement is applicable for UEs with the capability of: - Supporting ‘intraBandNR-CA-non-collocated-r19’, - Providing ‘nonCollocatedTypeNR-CA-v1900’ with value ‘type4’.

There are still few open issues on simulation assumptions, with way forwards as in [1], which need to be addressed in RAN4#117 meeting. In this contribution, we provide our views on the remaining open issues for discussions during RAN4#117.

Discussion

In this Section, the issue on MCS and Rank will be first discussed, followed by the issue on antenna connection.

MCS and rank

The issue on MCS and rank has been discussed in RAN4#116bis with the following way forwards [1]:

Way forward: MCS/rank Down-select the MCS for the stronger CC based on the simulation results - For weaker CC: - Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1) - For stronger CC: - Option 1: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2) - Option 2: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2) - Other options are not precluded. Interested companies are encouraged to provide the simulation results (both alignment and impairment results) in the RAN4#117 to decide the rank/MCS.

As stated in the way forward above, the MCS and rank under consideration for the weaker CC are QPSK 0.3 (MCS4 in MCS index table 1) and Rank 2, respectively. The considered rank for stronger CC is 4, while the MCS itself is kept open for an option which will lead to 25 dB power difference between the two CCs.

In RAN4#117 meeting, we provide our preliminary simulation results in [2]. From the simulation results in [2], one may observe that the SNR for the weaker CC with Rank 2 and QPSK 0.3 (MCS4 in MCS index table 1) is -3.1 dB. For the stronger CC, the two options have two different SNR, namely, 20.9 dB for 256QAM 0.69 (MCS21 in MCS index table 2) and 22.6 dB for 256QAM 0.74 (MCS22 in MCS index table 2). Hence, the power difference between the two CCs are 24 dB for option 1 and 25.7 dB for option 2.

Having MCS 256QAM 0.69 (MCS21 in MCS index table 2) for the stronger CC leads to 24 dB power difference between the stronger and weaker CCs.

Having MCS 256QAM 0.74 (MCS22 in MCS index table 2) for the stronger CC leads to 25.7 dB power difference between the stronger and weaker CCs.

As stated in the way forward from RAN4#116bis, the final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM).

As the final choice of MCS and Rank should consider the maximum SNR difference of 25 dB including the additional margin (0.5dB for QPSK, 0.8dB for 256QAM), RAN4 to select 256QAM 0.69 (MCS21 in MCS index table 2) as the MCS for the stronger CC.

Antenna connection

The next open issue from RAN4#116bis is concerning antenna connection as follows [1]:

Way forward: Antenna connection Discuss whether to capture the antenna connection for Type 4b UE test in TS 38.101-4 5.1.1. - Example: - For Minimum requirements for non-collocated scenarios for intra-band non-contiguous NR-CA for type 4 UE in Clause xxxx, 4 out of 8Rx should be connected to one CC and the other 4 out of 8Rx should be connected to another CC, depending on UE’s declaration and AP configuration.

During the discussion in RAN4#116bis, the proponent of capturing the antenna connection for Type 4b UE test in TS 38.101-4 5.1.1, has mentioned that they will bring a corresponding CR for the Rel-18 in RAN4#117 which will provide more information on their proposals. RAN4 would then need to further discuss on the issue.

The proponent will submit a corresponding CR for Rel-18 in RAN4#117 to better explain their proposal on antenna connection; based on which RAN4 would need to discuss further on the issue.

Conclusion

In this contribution, we provide our views on Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2. The following Observations and Proposals were made:

Observation 1: Having MCS 256QAM 0.69 (MCS21 in MCS index table 2) for the stronger CC leads to 24 dB power difference between the stronger and weaker CCs.

Observation 2: Having MCS 256QAM 0.74 (MCS22 in MCS index table 2) for the stronger CC leads to 25.7 dB power difference between the stronger and weaker CCs.

Observation 3: As stated in the way forward from RAN4#116bis, the final choice of MCS and Rank should consider the maximum SNR difference of 25dB, where the SNRs are average of companies’ impairment results, plus additional margin (0.5dB for QPSK, 0.8dB for 256QAM).

Proposal 1: As the final choice of MCS and Rank should consider the maximum SNR difference of 25 dB including the additional margin (0.5dB for QPSK, 0.8dB for 256QAM), RAN4 to select 256QAM 0.69 (MCS21 in MCS index table 2) as the MCS for the stronger CC.

Observation 4: The proponent will submit a corresponding CR for Rel-18 in RAN4#117 to better explain their proposal on antenna connection; based on which RAN4 would need to discuss further on the issue.

- References

- R4-2514791, WF on NonCol_intraB_ENDC_NR_CA_Ph2_demod, 3GPP RAN4#116bis, Ericsson Prague, Czech Republic, October 13-17, 2025

- R4-2521413, Simulation Results of Non-Collocated Intra-Band NR-CA Phase 2, 3GPP RAN4#117, Nokia, Dallas, USA, November 17-21, 2025