3GPP TSG-RAN WG4 Meeting #117	R4-2521413

Dallas, USA, November 17 – 21, 2025

Source:	Nokia

Title:	Simulation Results of Non-Collocated Intra-Band NR-CA Phase 2

Agenda item:	6.2.3

Document for:	Information

Introduction

The discussions on the Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2 during RAN4#116bis have led to agreements as documented in [1]. Some of them, which relate to the simulation parameters, are shown below [1]:

Agreement: Test scenario Define PDSCH demodulation requirements for Type 4b UE with 2CC NR-CA scenario. - Specify FR1 TDD + FR1 TDD only. Agreement: Received time difference between two carriers Configure the timing offset of the SCell from the PCell is 33 µs. Agreement: Channel bandwidth and SCS Configure CBW=40MHz and SCS=30kHz for both CCs. Way forward: MCS/rank Down-select the MCS for the stronger CC based on the simulation results - For weaker CC: - Rank 2 + QPSK 0.3 (MCS4 in MCS index table 1) - For stronger CC: - Option 1: Rank 4 + 256QAM 0.69 (MCS21 in MCS index table 2) - Option 2: Rank 4 + 256QAM 0.74 (MCS22 in MCS index table 2) - Other options are not precluded. Interested companies are encouraged to provide the simulation results (both alignment and impairment results) in the RAN4#117 to decide the rank/MCS. Agreement: Channel model and antenna configuration Use the static channel model specified in TS 38.101-4 [3] B.1.1. - 2x4 for CC with rank 2 - 4x4 for CC with rank 4 Agreement: Tx EVM assumption Assume Tx EVM=3% for both CCs.

In this contribution, we provide the results of our simulation on Rel-19 intra-band non-collocated NR-CA.

Discussion

The simulation results of Rel-19 Non-Collocated Intra-Band are tabulated in Table 1 for the given parameters in [1] as summarized above.

Table 1. Simulation Results Non-Collocated Intra-Band NR-CA

|  |  | Antenna Configuration | Rank | MCS | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | Impairments |
| Weaker CC |  | 2x4 | 2 | QPSK 0.3 (MCS4 in MCS index table 1) | -3.1 | -0.6 |
| Stronger CC | Option 1 | 4x4 | 4 | 256QAM 0.69 (MCS21 in MCS index table 2) | 20.9 | 23.4 |
|  | Option 2 | 4x4 | 4 | 256QAM 0.74 (MCS22 in MCS index table 2) | 22.6 | 25.1 |

More on our observations and proposals can be found in [2].

Summary

In this contribution, we provide simulation results of the Rel-19 support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2. More on the observations and proposals can be found in [2].

- References

- R4-2514791, WF on NonCol_intraB_ENDC_NR_CA_Ph2_demod, 3GPP RAN4#116bis, Ericsson Prague, Czech Republic, October 13-17, 2025

- R4-2521412, On PDSCH requirements of Non-Collocated Intra-Band EN-DC NR-CA Phase 2, 3GPP RAN4#117, Nokia, Dallas, USA, November 17-21, 2025

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V19.0.0, 10-2025