# 3GPP TSG-RAN WG4 Meeting #117	R4-2521420

# Dallas, USA, Nov. 17-21, 2025

Title:	Discussion on NTN related BS RF and co-existence for 6GR

Source:	Samsung

Agenda item:	8.4.2

Document for:	Approval

Introduction

In last RAN4#116bis meeting, RAN4 had initially discussed the contributions for 6GR BS RF requirements and co-existence from multiple companies. The meeting had agreed on some high-level guidance on some aspects and documented in [1], but the detailed proposals from each company still require more discussion. In this paper, we will continue to provide our views on the NTN aspects for 6GR.

Discussion

For extensive coverage in areas without cellular networks, NR is increasingly relying on TN with NTN. NTN is capable of rapidly supplying basic services to remote areas and is applicable for a range of uses such as logistics, disaster management, and rescue through communication systems. Numerous TN operators have recently partnered with NTN operators and are now incorporating NTN capabilities into their services. Standards for IoT-NTN and NR-NTN are continuously evolving in LTE and NR, and it is anticipated that 6GR will support the respective functionalities within a single integrated framework.

In previous 5G NR co-existence study, the NR-NTN and IoT-NTN comes as a late comer in 3GPP environment and specifications. Thus, the co-existence studies were carried out in a way that most scenarios are conducted with the TN ACLR/ACS are fixed with existing requirements. In 6G new radio co-ex study, given NTN can be considered in early phase of developing the co-existence study and also the RF requirements for co-ex, it is possible that we can consider NTN scenario in 6G new radio co-ex study at relatively early phase and let its adjacent channel interference case to be considered in RF requirements for a harmonized TN/NTN structure/hardware.

Observation 1: In 5G NR/IoT NTN studies, the NR-NTN were introduced in a late phase where all TN requirements are ready, hence the interference transmitted and received from NTN stations are not fully addressed as the TN devices cannot be easily upgraded with new requirements.

Proposal 1: For 6G new radio study, it is possible for RAN4 to consider the NTN scenario in co-ex in relatively early phase in co-ex study, and in determine requirements for better co-existence for 6G Radio between NTN and TN.

Moreover, one of the key disputes in most NTN topics especially in Rel-19 is the isolation distance that RAN4 adopted in Rel-17 NR-NTN study in FR1-NTN carriers. In Rel-17, the assumption was that the RAN4 targeted on a common UE for both NTN and TN access. And with such target, RAN4 adopted an assumption that an UE connecting with NTN network will automatically connect to a TN network with a much better link conditions.

Isolation concepts in TR 38.863

However, in later releases, the concern from terrestrial operators is that the UE (even FR1 UE) may not be guaranteed can have the agreement between operators and even hardware support to be redirected or connect to a TN network. When that happens, the isolation distance assumption from the co-existence study will no longer be true, and so as the ACLR/ACS from the co-ex study may not guarantee the co-existence between NTN and TN operators.

In 6G Radio study, as 3GPP is pursuing a much more integrated network design for both NTN and TN, a much more general consideration for the NTN and TN co-existence may be conducted to study a more general co-existence case in adjacent channel between NTN and TN, and even NTN and NTN. Also the regulation from ITU-R and local/regional spectrum regulators could also have the limitation on the geographically overlapping of NTN and TN network in adjacent channel.

Proposal 2: The isolation distance between NTN and TN from Rel-17 should be re-evaluated when consider NTN in 6G Radio co-existence.

And, if we are targeting a more general 6G Radio co-existence with the existence of 6G NTN deployment in adjacent channel, the UE density of such 6G NTN network beam/cell may not be as many as the TN network assumptions. And it can also comes from the capacity limit of a SAN in downlink and uplink and in different beams.

Proposal 3: The UE density and its distribution in one and all beams/cell from a single SAN/satellite should be better modeled if we are targeting a more general overlapping co-ex between 6G NTN and TN.

Moreover, the NTN-NTN scenarios were not explicitly considered in RAN4 co-existence study, due to the facts that the interference and facilitation between NTN operators usually follows regulatory procedure, which is outside the 3GPP scope.

Observation 2: Previous RAN4 did not really study NTN-NTN co-existence, due to the facts that NTN-NTN interference management and negotiation usually follows regulatory framework and out of 3GPP scope.

But recently, many local/regional regulators files new rules to allow the trial to deploy the mobile service (MS) spectrum onto satellite for a better coverage purpose. Also, in ITU-R, there is an agenda item 1.13 study the regulatory framework on such activity. The candidate bands shown in the following table are from ITU-R discussion, and it shows a relatively broad impacts on both legacy TN and also NTN networks.

Table 2-1 FDD candidate bands in AI 1.13 of WRC-27

| UL | DL |
| --- | --- |
| 807-849 | 852-894 |
| 880-915 | 925-960 |
| 832-862 | 791-821 |
| 698-716 776-798 | 716-746 746-768 |
| 698-748 | 753-803 |
| 1 427-1 470 | 1 475-1 518 |
| 1 920-1 980 | 2 110-2 170 |
| 1 710-1 785 | 1 805-1 880 |
| 1 850-1 920 | 1 930-2 000 |
| 1 710-1 780 | 2 110-2 180 |
| 2 000-2 020 | 2 180-2 200 |
| 2 305-2 320 | 2 345-2 360 |
| 2 500-2 570 | 2 620-2 690 |

With not clear regulations published at the moment, in our view, we have two possible directions at least. One is that we leave it totally to other organizations to define the requirements and then 3GPP define additional requirements to guarantee co-existence. And the other way is that 3GPP can also study the interference between NTN and TN, and even NTN and NTN to understand what RF requirements should be targeted.

Observation 3: With a more flexible trend of spectrum usage between NTN and TN, along with a common design purpose of 6G Radio for NTN and TN, the co-existence between NTN and TN, and even between NTN and NTN can be more critical than 5G.

Another concept in NTN for 6G is the consideration of a het-net structure between SAN in different orbits. The experience from previous TN network is that the performance and interference for a co-channel deployment of het-network would be very hard to optimize. However, if the operators interested in pursuing a deployment of het-net between different orbits in adjacent channel, the interference should be studied.

Observation 4: If het-net (multi-orbits) structure between different satellite orbits are pursued as a key deployment in 6G NTN, the interference between NTN and TN, NTN and NTN may be also studied.

Conclusion

In this contribution, we proposed following for the 6G radio NTN:

Observation 1: In 5G NR/IoT NTN studies, the NR-NTN were introduced in a late phase where all TN requirements are ready, hence the interference transmitted and received from NTN stations are not fully addressed as the TN devices cannot be easily upgraded with new requirements.

Proposal 1: For 6G new radio study, it is possible for RAN4 to consider the NTN scenario in co-ex in relatively early phase in co-ex study, and in determine requirements for better co-existence for 6G Radio between NTN and TN.

Proposal 2: The isolation distance between NTN and TN from Rel-17 should be re-evaluated when consider NTN in 6G Radio co-existence.

Proposal 3: The UE density and its distribution in one and all beams/cell from a single SAN/satellite should be better modeled if we are targeting a more general overlapping co-ex between 6G NTN and TN.

Observation 2: Previous RAN4 did not really study NTN-NTN co-existence, due to the facts that NTN-NTN interference management and negotiation usually follows regulatory framework and out of 3GPP scope.

Observation 3: With a more flexible trend of spectrum usage between NTN and TN, along with a common design purpose of 6G Radio for NTN and TN, the co-existence between NTN and TN, and even between NTN and NTN can be more critical than 5G.

Observation 4: If het-net (multi-orbits) structure between different satellite orbits are pursued as a key deployment in 6G NTN, the interference between NTN and TN, NTN and NTN may be also studied.

Reference

[1] R4-2514638, WF on 6G BS RF and co-existence, Feature Lead (Ericsson)

[2] R4-2513045, Discussion on BS RF and co-existence for 6GR, Samsung