3GPP TSG RAN4 Meeting #117																	R4-2521454

Dallas, Texas US, 17th – 21st November 2025

Agenda item:		7.2.4

Source:	Nokia

Title:	On SBFD BS to SBFD BS adjacent channel coexistence

Document for:		Approval

# Introduction

A new Rel-20 WID [1] was agreed in RAN#109 regarding NR base station RF requirement evolution for FR1 and testing phase 2. Last RAN4#116bis meeting started discussion, including agreed work plan and way forward in [2]. In this contribution we continue discussion on SBFD BS to SBFD BS adjacent channel coexistence.

# Discussion

The following detail were agreed in way forward [2]:

- For next meeting further consider following options, with the intention to down-select: Option 1: UD (Operator 1) and DU (Operator 2) Option 2: DUD (Operator 1 and 2) Option 3: UD and DUD Provide more description on each specific case. - As baseline use simulation parameters defined in TR 38.858. For 6 GHz, new parameters for BS antenna should be established for SBFD.

## SBFD subbands configurations

Before discussing the different SBFD configuration options captured in the WF, a key consideration for this coexistence study is the selection of ACLR and ACS models. Previously, in Rel-18 TDD-SBFD coexistence studies, companies agreed to use a frequency-flat model for ACLR and ACS as captured in TR 38.858. One option for this new coexistence study is to maintain the same model assumptions. Alternatively, a frequency-dependent model for ACLR and ACS can be introduced. Note that a frequency-flat model is adopted, the SBFD subband arrangement among operators won’t have any effect.

Companies are encouraged to discuss whether ACLR/ACS frequency-flat or frequency-dependent models should be used for the SBFD-SBFD coexistence study.

Assuming a frequency-selective model and, regarding the different options captured in the WF, our preference is to select the one that results in the most challenging scenario from a gNB-to-gNB CLI point of view. If the study uses frequency-selective ACLR and ACS models, this will correspond to the SBFD arrangement that results in the minimum distance between the aggressor DL subband and the victim UL subband.

The coexistence study should be based on the SBFD configuration arrangement that results in the lowest frequency separation between the DL subband of the aggressor gNB and the UL subband of the victim gNB.

In the following, we provide an analysis of the different SBFD configurations by assuming a bandwidth split of 80-20 for the DU configuration and 40-20-40 for the DUD configuration, and 100 MHz carriers.

A bandwidth split of 80-20 and 40-20-40 is assumed for the DU and DUD SBFD configurations, respectively.

| SBFD configuration | Frequency distance between aggressor operator DL subband and victim operator UL subband [MHz] |
| --- | --- |
| UD (Operator 1) and DU (Operator 2) | 80 |
| DUD (Operator 1) and DUD (Operator 2) | 40 |
| UD (Operator 1) and DUD (Operator 2) | 80 (Operator 1 as victim), 40 (Operator 2 as victim) |

Based on this, we suggest prioritizing Option 2, i.e., DUD configurations for both Operator 1 and Operator 2.

It is proposed to assume DUD configuration for Operator 1 and Operator 2 as first priority SBFD configuration

## Antenna parameters for 6 GHz

For 6 GHz SBFD adjacent channel coexistence studies, we currently propose to use TR 38.858 antenna assumptions as a baseline.

It is propose to use TR 38.858 antenna assumptions as a baseline.

## Feasibility studies

WID [1] mentions “Specify necessary (and if feasible) BS TX and RX RF requirement …”. In this regard, part of the study in RAN4 should also be focused on discussing on whether the new unwanted emissions and in-band blocking requirements are possible to achieve. The preliminary study in R4-2507683 shows that ~36 dB improvement of ACIR (on top of the baseline 42.5) is needed for scenarios with 10% grid shift. This brings the question on whether it is possible to achieve such requirement in practical BS implementations, as this will likely requires operator-specific analog filters with little insertion loss. Also, full coordination between operators will be required to ensure that emissions between operators are minimised.

RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation.

# Conclusion

This continues discussions and present Nokia’s views on SBFD BS to SBFD BS adjacent channel coexistence. Following observations and proposals were introduced:

- Companies are encouraged to discuss whether ACLR/ACS frequency-flat or frequency-dependent models should be used for the SBFD-SBFD coexistence study.

The coexistence study should be based on the SBFD configuration arrangement that results in the lowest frequency separation between the DL subband of the aggressor gNB and the UL subband of the victim gNB.

A bandwidth split of 80-20 and 40-20-40 is assumed for the DU and DUD SBFD configurations, respectively.

It is proposed to assume DUD configuration for Operator 1 and Operator 2 as first priority SBFD configuration

It is proposed to use TR 38.858 antenna assumptions as a baseline.

RAN4 shall also discuss whether it is feasible to achieve the identified BS TX and RX RF requirements considering practical BS implementation.

# References

- RP-252905 “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, RAN#109, Beijing, China, 15th-18th September, 2025.

- R4-2515091 “Way-Forward for NR_BS_RF_Ph2”, RAN4#116bis, Prague, Czech, 13th – 17th October, 2025.