# 3GPP TSG-RAN WG4 Meeting #117	R4-2521515

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.3

Source: 	LG Electronics

Title: 	Views on 6G UE general RF and UE RF

Document for:	Discussion

Introduction

In the RAN#109, the study on “6G general RF and UE RF” was approved. In this discussion, we share our point of view for 6G general RF and UE RF based on the approved WF [1].

Discussion

Single carrier output power requirements

Topic #3: Single carrier output power requirements - Agreements - Study 6G output power requirements to enable more flexibility to different UE implementations to reflect the RF front-end capability for single band - Encourage inputs on how to allow UE to deliver more output power and how power class framework includes e.g. Tx diversity and UL-MIMO - Study improvements on configured maximum output power requirements and associated parameters - Strive to improve clarity on UE power ability under different UE condition (e.g. configured, activated, scheduled) in all output power requirements - Study the default power class for FDD and TDD bands for 6G - Study how to handle SAR/MPE for 6GR

PCMAX parameters

The equation for 5G's Configured Transmitted Power (PCMAX,f,c) is highly complex, making it difficult to understand, and the operation of some parameters included in the formula is unclear in the actual field. Therefore, there is a need to re-evaluate the necessity and definitions of these parameters during the initial study phase of 6G.

∆TC,c

∆TC,c is related to the RF characteristics based on the Operating Band and Operating Bandwidth. Consequently, it must be re-examined considering the 6G RF characteristics and new Channel Bandwidth (CBW).

ΔPPowerClass

∆PPowerClass was added to satisfy SAR regulations. It allows for the application of power backoff considering the duty cycle when the terminal operates at High Power. However, SAR regulation is an external (non-3GPP) requirement, and since it cannot be definitively assumed that the signal generated by the terminal is confined solely to 3GPP communication, there is ambiguity in actually implementing such a requirement within the 3GPP specification. Therefore, the necessity of ∆PPowerClass itself must be re-examined for 6G.

ΔPPowerBoost

∆PPowerBoost was introduced to support a higher output than the nominal rated power (PCMAX) under specific conditions. Since UL high power is also crucial in 6G to overcome coverage issues, the 6G study should investigate if there are more efficient and clear methods to support high power from the terminal.

ΔTIB,c

∆ TIB,c refers to the allowed power tolerance when operating with CA, DC, or SUL. Defining ∆TIB,c for each combination through a Basket Work Item in RAN4 is consuming enormous time and human resources, and as a result, the size of the standard document itself has increased immensely. Methods to remove or simplify ∆TIB,c should be discussed.

Proposal 1: A review of the 5G configured Tx power equation parameters is necessary for 6G

∆TC,c: Needs re-examination considering the 6G RF characteristics and the new Channel Bandwidth.

ΔPPowerClass: The necessity of the parameter itself should be fundamentally re-evaluated for exclusion from the 6G standard or replacement with a clearer alternative.

ΔPPowerBoost: Investigate through 6G studies whether there are more efficient and clear methods to support UL high power.

ΔTIB,c: Discussions should be held to find and implement ways to either remove or simplify this parameter.

On SAR/MPE impacts for 6GR

RAN4 discussed in 116bis meeting on potential solutions to manage the SAR and MPE as part of the 6G study item. Although the details of these metrics and requirements are not under 3GPP mandate we share below some high-level views and observations on this topic.

SAR, APD, and IPD are representative RF exposure metrics that respectively measure absorption rate, absorbed power density, and incident power density to assess electromagnetic exposure to the human body.

- SAR (Specific Absorption Rate) measures the rate at which electromagnetic energy is absorbed per unit mass of tissue (W/kg). (10 MHz ~ 6 GHz)

- APD (Absorbed Power Density) measures the electromagnetic power absorbed per unit area of tissue surface (W/m²). (6GHz ~ 10 GHz)

- IPD (Incident Power Density), on the other hand, measures the total incoming electromagnetic power per unit area before any absorption or interaction with the body (W/m²). It represents the exposure level in the environment, not the absorbed amount. (10 GHz ~ 300GHz)

For frequencies where penetration depth and tissue density are known, the Absorbed Power Density (APD) can be derived from the Specific Absorption Rate (SAR) using the relationship APD = (SAR × ρ) / δ, where ρ is the tissue density and δ is the penetration depth.  IPD is an independent metric indicating the RF power per unit area incident on the body surface before absorption, and cannot be derived from SAR or APD because it depends on environmental and transmission conditions.

SAR based Exposure ratio is

where:

- 𝑆𝐴𝑅𝑡 is the SAR value of the t-th transmitter/test frequency

- 𝑆𝐴𝑅𝑙𝑖𝑚𝑖𝑡,𝑡 is the basic restriction limit that is applicable for the t-th transmitter and

- 𝑓𝑡 is the operating frequency / test frequency of the t-th transmitter

Absorbed Power Desnsity (APD) based Exposure ratio is

where:

- APDv is the APD value for the v-th transmitter/test frequency

- APDlimit,v is the basic restriction limit that is applicable for the v-th transmitter and

- 𝑓𝑣 is the operating frequency / test frequency of the v-th transmitter

Incident Power Density (IPD) based Exposure ratio is

where:

- psPDx is the peak spatial-average power density value for the x-th transmitter

- psPDlimit,x is the applicable peak spatial-average power density reference level limit for the x-th transmitter and

- 𝑓𝑥 is the operating frequency / test frequency the x-th transmitter

and

where:

- psPDy is the peak spatial-average power density value for the y-th transmitter

- psPDlimit,y is the applicable peak spatial-average power density reference level limit for the y-th transmitter

- pPDy is the spatial peak power density value for the y-th transmitter

- pPDlimit,y is the applicable spatial peak power density reference level limit for the y-th transmitter and

- 𝑓𝑦 is the operating frequency/test frequency the y-th transmitter

Observation 1. For frequencies below 10 GHz, exposure assessment is primarily based on absorbed power metrics such as SAR or APD, whereas for frequencies above 10 GHz, the assessment is primarily based on incident power metrics such as IPD, reflecting the dominant interaction mechanism at higher frequencies.

Observation 2. At higher frequencies, uplink performance suffers due to greater propagation loss, prompting consideration of High Power UE configurations. However, higher transmit power increases also SAR and APD. This is attributed to the frequency-dependent increase in human tissue conductivity — 0.97 S/m @ 900 MHz, 1.5 S/m @ 2100 MHz, 2.91 S/m @ 3500 MHz, 6.07 S/m @ 6500 MHz, 8.46 S/m @ 8500 MHz, and 10.4 S/m @ 10000 MHz — which leads to greater RF energy absorption. Since conductivity increases by nearly a factor of 10 across this frequency range, SAR/APD risk can also increase by approximately 10× under the same electric field (E) conditions. Consequently, while higher power operation can improve communication performance, it also introduces additional challenges in meeting human RF exposure compliance requirements.

The table below presents the average power settings for the frequency ranges 1710 MHz – 1915 MHz, 2496 MHz – 2690 MHz, and 3300 MHz – 4200 MHz based on FCC filings that also shows the negative trend in average power with increasing frequency.

Observation 3: Measurements of commercial devices show also a negative trend in average power with increasing frequency.

Table 1- Average Power Settings for 10 recent 5G phones from two different companies

|  | n66 1710 - 1780 | B25/n25 1850 - 1915 | n7 or n41 2500-2570 | n77 3300 - 4200 |
| --- | --- | --- | --- | --- |
| Avg Power Set of A company | 20.3 dBm | 20.4 dBm | 19.3 dBm | 18.4 dBm |
| Avg Power Set of B company | 19.5 dBm | 19.3 dBm | 18.3 dBm | 17.4 dBm |

The figure below shows the data in the Table 1 and our estimation for maximum average power that meets the SAR under the assumption that all antennas have the same efficiency and that the amount of E-field absorbed by the human body is the same.

Figure 1. Estimated and measured average powers

Proposal 2: Take observations 1, 2 and 3 into account when discussing the UE transmission and output power related aspects during the study item.

Tx requirements

Topic #5: Tx requirements - Agreements - PA modelling is discussed in system parameter thread - In initial 6GR evaluations, use 5G NR unwanted emission requirements as starting point until requirement framework for 6GR is clear - Spurious emissions requirements, SEM and ACLR will be defined for 6GR - FFS for - OOB boundary - potential accommodations for different frequency sub-ranges - Encourage inputs on regulatory survey being conducted in spectrum agenda - Study the UE Tx impairment requirements/assumptions (e.g. carrier leakage, image rejection, CIM3 and CIM5) - In RAN4#117, prioritize discussion for UE Tx impairments and unwanted emission framework

The following baseline criteria were considered for MPR simulations in 5G NR:

- Post-PA Loss: 4 dB

- Carrier leakage: 28 dBc

- IQ Image: 28 dBc

- CIM3: 60 dBc

In 5G, for MPR simulation, the standards for IQ image and carrier leakage were raised to 34 dBc or higher only for high-order modulation conditions (e.g., 256 QAM). However, the 5G In-band emission requirements were defined without considering these aspects and do not adequately reflect the actual performance improvements of modern UEs.

Carrier leakage and IQ image

The introduction of high-order modulation, such as UL 1024 QAM being considered in 6G studies, makes the requirements for EVM performance much stricter. This directly necessitates the improvement of Tx impairments. The current standards for carrier leakage and IQ image of 28 dBc are significantly low to reflect the actual performance of modern RF Transceivers and the 6G high-performance requirements. To satisfy the high-order modulation performance requirements being considered in 6G, the carrier leakage and IQ image standards should be raised to 34 dBc or 40 dBc or higher.

CIM3

The current standard for CIM3 at 60 dBc seems appropriate based on the existing 100 MHz bandwidth. However, additional review is needed to account for the impact of bandwidth expansion to 200 MHz or more.

Post-PA Loss

The 5G standard uniformly assumed a 4 dB post-PA loss for both FDD and TDD. This value is understood to have been defined based on the worst-case scenario of FDD. This value also does not consider the additional RF front-end loss caused by CA.In 5G, the additional RF front-end loss was defined separately for each band combination, which resulted in a significant expenditure of human resources and time to define ΔTib/ΔRib. To alleviate this, it is proposed that in 6G, ΔTib/ΔRib due to CA/DC should be defined by considering them within the impairments. The following are the TDD Tx impairment parameters around 7 GHz.

RF front end block loss evaluation

- Diplexer: 1.5 dB

- Switch: 0.8 dB

- Additional switch loss subject to CA considering ∆ Tib: 0.5 or 1.5 dB (non-simultaneous/simultaneous Tx/Rx)

- TDD switch: 1dB

- Post PA loss (around 7 GHz): 3.8~5.3 dB (including CA impact)

Proposal 3: Consider Tx impairments for around 7 GHz

- Post-PA Loss: 3.8~5.3 dB (including CA impact)

- Carrier leakage: 34 dBc or 40 dBc

- IQ Image: 34 dBc or 40 dBc

- CIM3: 60 dBc

Rx requirement

Topic #6: Rx requirements - Agreements - Study if the RX requirements used in 5G are still relevant for respective 6G bands up to 100MHz CBW and how to define relevant RX requirements for >100MHz CBW’s - Study concept for RX requirements for new frequencies, especially those above 7.125GHz - Study how to specify 6G REFSENS in a scalable way considering that 6G will support devices with different implementations/capabilities (e.g. different number of Rx chains) potentially addressing different use cases - Study single carrier REFSENS requirements considering technology advancement as well as implementation constraints such as UEs may support CA as well - Study the 6G REFSENS level and respective UL RB allocations for FDD bands as well as other RX requirements using respective 5G requirements as reference - Study improvements to MSD framework for 6GR in parallel to progressing single carrier requirements - Study multi-carrier requirements (e.g. ACS, blocking) once single carrier work is more stable

The following REFSENS equation was used in 5G:

REFSENS (dBm) = 10log10(KTB) + NF + SNR – GDIV + IM

This equation is likely reusable in 6G research. However, parameters such as Noise Figure (NF) and IM (Implementation Margin) need to be updated to reflect advancements in RF components.

Noise Figure (NF) Calculation Considerations for the 6G around 7GHz TDD Band

- RF front end block loss evaluation

  - Diplexer: 1.5 dB

  - Switch: 0.8 dB

  - Additional switch loss subject to CA considering ∆ Rib: 1 dB

  - TDD switch: 1dB

  - Total RF front end loss (around 7 GHz): 3.3 dB (including CA impact)

- LNA

  - Gain: 15 dB

  - NF:3~7 dB

- Gain block

  - Gain:20 dB

  - NF: 7 dB

- NF calculation

  - Total noise figure: 6.6~ 10.4 dB (Typical temp), 9.6~13.4 dB (extreme temp)

The RF front end block loss can be calculated by considering the diplexer, switch, TDD switch, and additional switch loss for CA, resulting in approximately 3.3 dB. The LNA NF did not show significant differences based on operating mode but can vary depending on the LNA's band coverage, ranging from approximately 3 dB to 7 dB. Considering this, the total Noise Figure around 7 GHz can be 6.6 ~ 10.4 dB.

Implementation Margin (IM)

An Implementation Margin of 2.5 dB was commonly used in 5G. However, a clear standard or justification for this 2.5 dB was not found. For 6G, there is a need to more clearly define the necessity of this margin and its specific value.

Proposal 4: The REFSENS equation used in 5G can also be used in 6G.

REFSENS (dBm) = 10log10(KTB) + NF + SNR – GDIV + IM

Proposal 5: The noise figure around 7 GHz for 6G can be considered in conjunction with ∆ Rib

Noise Figure (NF) Calculation Considerations for the 6G around 7GHz TDD Band

- RF front end block loss evaluation

  - Diplexer: 1.5 dB

  - Switch: 0.8 dB

  - Additional switch loss subject to CA considering ∆Rib: 1 dB

  - TDD switch: 1dB

  - Total RF front end loss (around 7 GHz): 3.3 dB (including CA impact)

- LNA

  - Gain: 15 dB

  - NF:3~7 dB

- Gain block

  - Gain:20 dB

  - NF: 7 dB

- NF calculation

  - Total noise figure: 6.6~ 10.4 dB (Typical temp), 9.6~13.4 dB (extreme temp)

Proposal 6: The criterion for the IM (implementation margin) value needs to be more clearly defined in 6G.

Frequency range between FR1 and FR2-1

Topic #7: Frequency range between FR1 and FR2-1 - Agreements - Study UE RF front end characteristics considering potential differences between e.g. frequencies around 7 GHz and around 15 GHz.

In 5G, frequency ranges are divided into FR1 and FR2. FR1 (sub-6 GHz) typically requires conducted RF testing, while FR2 (mmWave) mandates OTA testing. This distinction arises from fundamental propagation characteristics: as frequency increases, directivity becomes higher and free-space path loss increases. To compensate for these effects, FR2 adopts beamforming as a key technique.
In 6G studies, frequencies around 7 GHz and 15 GHz are being considered as representative points. These frequencies lie in a transition region between the clearly separated FR1 and FR2 ranges in 5G. Consequently, UE RF front-end characteristics may exhibit mixed properties of both FR1 and FR2.

- Around 7 GHz: closer to FR1 characteristics; a baseline assumption could be similar to the 5G FR1 default architecture (e.g., 1T2R) but with a higher power class.

- Around 15 GHz: closer to FR2 characteristics; a baseline assumption could follow the 5G FR2 PC3 default architecture (e.g., 4-array, 1-panel) with beamforming capability.

The frequency range between FR1 and FR2 requires careful consideration of RF front-end design trade-offs, including antenna size, isolation, PA efficiency, and test methodology (conducted vs OTA).

Proposal 7: Consider UE RF front-end characteristics as shown below.

- Around 7 GHz: closer to FR1 characteristics; a baseline assumption could be similar to the 5G FR1 default architecture (e.g., 1T2R) but with a higher power class.

- Around 15 GHz: closer to FR2 characteristics; a baseline assumption could follow the 5G FR2 PC3 default architecture (e.g., 4-array, 1-panel) with beamforming capability.

∆TIB/∆RIB Simplification

Sub-topic 2-5: ∆TIB and ∆RIB in 6G The ∆TIB and ∆RIB defined per specific band combination roots in the first introduction of CA in LTE times. Therefore, some companies are now questioning whether there is really still a need for these in 6G. Additionally, there have been multiple attempts to align and clarify the rules for defining the ∆TIB and ∆RIB values, but there is still no unified way specified. This is wanted to be addressed by other companies. Issue 2-5-1: ∆TIB in 6G - Proposals - Option 1: RAN4 shall not consider ∆TIB for 6G - Option 2: RAN4 shall study if ∆TIB is needed, defined for 6G - Option 3: RAN4 shall continue to define ∆TIB for 6G but unify and specify how a value is determined. - WF: - RAN4 shall discuss ∆TIB requirements under the 6G general RF and UE RF agenda - It is not precluded to discuss band combination related structure/use of the ∆TIB requirements under this agenda, i.e. simplification of its use. Issue 2-5-2: ∆RIB in 6G - Proposals - Option 1: RAN4 shall not consider ∆RIB for 6G - Option 2: RAN4 shall study if ∆RIB is needed, defined for 6G - Option 3: RAN4 shall continue to define ∆RIB for 6G but unify and specify how a value is determined. - WF: - RAN4 shall discuss ∆RIB requirements under the 6G general RF and UE RF agenda - It is not precluded to discuss band combination related structure/use of the ∆TIB requirements under this agenda, i.e. simplification of its use.

The 5G NR system leverages advanced technologies like CA, DC, SUL, and SDL to maximize flexibility and performance through various Band Combinations. However, in multi-carrier environments like CA/DC, the complexity of the RF Front End (RFFE) inevitably increases. The higher number of components (switches, filters, duplexers) needed to support multiple bands can lead to additional loss. ∆ Tib/Rib Tolerances are defined to address this. They represent additional allowed tolerance on the UL Power and REFSENS requirements, respectively, taking into account the extra loss generated by the increased RFFE complexity in specific Band Combinations.

The 5G standardization process involved a massive effort to individually analyze and define the ∆Tib/Rib requirements for every possible Band Combination. This process led to a significant increase in the complexity of the standards and resulted in a huge consumption of time and engineering resources. The burden of repeating this analysis for every new Band Combination continues to grow. To mitigate this problem of standard complexity and resource drain, the simplification of the ∆Tib/Rib definition is essential.

Although ∆Tib/Rib was defined for multi-carrier environments (such as CA/DC), the increased RFFE complexity required for CA Band Combinations can also impact the RF performance of Single Carrier operation. The RFFE inherently carries the complexity needed for the Band Combination, even when only a single frequency is being used. Therefore, for 6G, we propose to integrate the impact of ∆Tib/Rib directly into the Single Carrier requirements, suggesting that ∆Tib/Rib should not be separately defined in the 6G standard to ensure manufacturers consider multi-carrier RFFE complexity early in the single-carrier design process, thus streamlining standardization and saving development time.

Proposal 8: Do not define the ∆Tib/Rib from 6G requirements and, instead, integrate its performance impact directly into the single carrier requirements.

Conclusion

Proposal 1: A review of the 5G configured Tx power equation parameters is necessary for 6G

∆TC,c: Needs re-examination considering the 6G RF characteristics and the new Channel Bandwidth.

ΔPPowerClass: The necessity of the parameter itself should be fundamentally re-evaluated for exclusion from the 6G standard or replacement with a clearer alternative.

ΔPPowerBoost: Investigate through 6G studies whether there are more efficient and clear methods to support UL high power.

ΔTIB,c: Discussions should be held to find and implement ways to either remove or simplify this parameter.

Observation 1. For frequencies below 10 GHz, exposure assessment is primarily based on absorbed power metrics such as SAR or APD, whereas for frequencies above 10 GHz, the assessment is primarily based on incident power metrics such as IPD, reflecting the dominant interaction mechanism at higher frequencies.

Observation 2. At higher frequencies, uplink performance suffers due to greater propagation loss, prompting consideration of High Power UE configurations. However, higher transmit power increases also SAR and APD. This is attributed to the frequency-dependent increase in human tissue conductivity — 0.97 S/m @ 900 MHz, 1.5 S/m @ 2100 MHz, 2.91 S/m @ 3500 MHz, 6.07 S/m @ 6500 MHz, 8.46 S/m @ 8500 MHz, and 10.4 S/m @ 10000 MHz — which leads to greater RF energy absorption. Since conductivity increases by nearly a factor of 10 across this frequency range, SAR/APD risk can also increase by approximately 10× under the same electric field (E) conditions. Consequently, while higher power operation can improve communication performance, it also introduces additional challenges in meeting human RF exposure compliance requirements.

Observation 3: Measurements of commercial devices show also a negative trend in average power with increasing frequency.

Proposal 2: Take observations 1, 2 and 3 into account when discussing the UE transmission and output power related aspects during the study item.

Proposal 3: Consider Tx impairments for around 7 GHz

- Post-PA Loss: 3.8~5.3 dB (including CA impact)

- Carrier leakage: 34 dBc or 40 dBc

- IQ Image: 34 dBc or 40 dBc

- CIM3: 60 dBc

Proposal 4: The REFSENS equation used in 5G can also be used in 6G.

REFSENS (dBm) = 10log10(KTB) + NF + SNR – GDIV + IM

Proposal 5: The noise figure around 7 GHz for 6G can be considered in conjunction with ∆ Rib

Noise Figure (NF) Calculation Considerations for the 6G around 7GHz TDD Band

- RF front end block loss evaluation

  - Diplexer: 1.5 dB

  - Switch: 0.8 dB

  - Additional switch loss subject to CA considering ∆Rib: 1 dB

  - TDD switch: 1dB

  - Total RF front end loss (around 7 GHz): 3.3 dB (including CA impact)

- LNA

  - Gain: 15 dB

  - NF:3~7 dB

- Gain block

  - Gain:20 dB

  - NF: 7 dB

- NF calculation

  - Total noise figure: 6.6~ 10.4 dB (Typical temp), 9.6~13.4 dB (extreme temp)

Proposal 6: The criterion for the IM (implementation margin) value needs to be more clearly defined in 6G.

Proposal 7: Consider UE RF front-end characteristics as shown below.

- Around 7 GHz: closer to FR1 characteristics; a baseline assumption could be similar to the 5G FR1 default architecture (e.g., 1T2R) but with a higher power class.

- Around 15 GHz: closer to FR2 characteristics; a baseline assumption could follow the 5G FR2 PC3 default architecture (e.g., 4-array, 1-panel) with beamforming capability.

Proposal 8: Do not define the ∆Tib/Rib from 6G requirements and, instead, integrate its performance impact directly into the single carrier requirements.

- References

# [1] R4-2514642, WF for [116bis][102] 6G general RF and UE RF(Qualcomm)

# [2] R4-2514626, WF on 6G Spectrum (Nokia)