3GPP TSG-RAN WG4 Meeting #117				                               R4-2521518

Dallas, USA, Nov. 17-21, 2025

Source:	LG Electronics

Title:	Discussion on co-existence evaluation for A-IoT outdoor scenario

Agenda item:	7.11.3

Document for:	Discussion

Introduction

In 3GPP RAN #109, SID[1] was approved. In this paper, we provide our views on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4. Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Discussion

Evaluation methodology and performance metric for formal simulation

The methodology should be same as the TR 38.769, as listed below

1) Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader based on coupling loss.

a)	The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other.

b)	The device can access the reader with best receiving power among all readers.

3)	Once association is done, round robin scheduling is used.

4)	For inter-system interference (between A-IoT and NR):

-	When AIoT as victim, if SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

- For SINR degradation, SINR refers to the 5% and 50% CDF SINR

-	When NR as victim, if throughput loss is smaller than and equal to 5%, it can be considered that inter-system interference is negligible.

Proposal 1: For NR system, use 5% throughput loss as performance metric as legacy.

Proposal 2: A-IoT system, use SINR degradation as performance metric.

On SINR definition, it can be as follows:

For D4T1-C:

SINR definition for D2R:

-	SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference

-	SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

SINR definition for R2D:

-Signal power of device to the noise and interference within 10MHz is baseline assumption

-	Consider 180KHz noise and interference bandwidth after BB LPF as optional

Proposal 3: Adopt the following A-IoT SINR definition.

SINR definition for D2R:

-	SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference

-	SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

SINR definition for R2D:

-Signal power of device to the noise and interference within 10MHz is baseline assumption

-	Consider 180KHz noise and interference bandwidth after BB LPF as optional

ACLR/ACS modeling

RAN4 should use the same ACLR model as Rel-19.

Proposal 4: For ACLR, RAN4 use ACLR model with applied scaling factor

Proposal 5: For ACS, RAN4 should choose one in following options

- Option1: Step function in TR36.942

- Option2: Flat model

- Option3: Consider ACS of adjacent channel

- Option4: Effective ACS

Deployment scenario

In last RAN4 #116bis meeting, we have approved following simulation assumptions for calibration purpose.

- The following assumptions are used for Rel-20 SI co-existence simulation calibration - Deployment scenario is considered - UMa scenario - Operating mode is considered with both in-band and standalone - In-band: only consider co-located - There is no grid shift between A-IoT BS and NR BS - Standalone: only consider non-co-located - 100% grid shift - Note: A-IoT BS and NR BS belonging to different operators are not deployed at the same site. The device may be far from its own cell and closer to the cell of a different operator, resulting in a near-far effect. - NR UE only outdoor is considered - A-IoT device location is considered as follow - Only outdoor - 100% outdoor 0 % in cargo trucks - Not preclude to provide simulation result in X% in cargo trucks - For example, X=20 - RAN4 consider as follow - both Device C and Device 2b Note. AIoT device 2b and C are simulated separately in the RAN4 co-existence simulation.

Proposal 6: For formal simulation, it should have same deployment scenario for calibration purpose

Deployment parameters assumptions for formal simulation

In last RAN4 #116bis meeting, we have approved following simulation assumptions for calibration purpose.

Table. Deployment parameters Parameter; Values for calibration purpose. Carrier frequency; 900MHz (Band 8). BW for NR; 10MHz with 15KHz SCS. R2D BW; 180kHz, for 15kHz SCS. D2R BW; 15kHz. Waveform; R2D: OFDM-based OOK D2R: BPSK. A-IoT DL power control; No. A-IoT UL power control; Use fixed maximum output power. Traffic model; Ful buffer. NR BS/A-IoT reader deployment (outdoor); Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-NR BS/A-IoT reader distance: [X]m - X= 750m (high priority) or 500m NR BS height: 25m the minimum 2D distance between macro NR BS/A-IoT reader and device 2b/C as 35 m.. Frequency reuse; option1) 1:3 option2) 1 (High priority). Device distribution; Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area - 100% outdoor 0 % in cargo trucks - Not preclude to provide simulation result in X% in cargo trucks - For example, X=20 Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 35m. NR UE dropping; For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell. MCL for NR BS to NR UE; MCL of 70 dB. Pathloss model; Use UMa pathloss model defined in TR38.901. penetration loss; OdB. Shadow fading margin; 6dB for UMa NLOS.

Proposal 7: For formal simulation, it should have same deployment parameter assumptions for calibration purpose

NR BS/UE simulation parameters for formal simulation

In last RAN4 #116bis meeting, we have approved following simulation assumptions for calibration purpose.

NR BS RF parameters Issue 2-3-4-1: NR BS RF parameters assumptions Agreement: NR BS Parameter; Values for calibration and formal simulation. Macro-BS Tx power (dBm); 46dBm. BS antenna pattern; Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am}. BS antenna gain (dBi); 15dBi including feeder loss. Height of macro NR BS (m); 25. NR Macro-BS Noise Figure(dB); 5. Network location; Outdoor. ACLR (in-band); Reuse R19 AIoT assumption: 30 dBc. ACLR (standalone); Same with legacy NR BS. ACS (in-band); Same with legacy NR BS. ACS(standalone); Same with legacy NR BS. NR UE RF parameters Issue 2-3-5-1: NR UE RF parameters assumptions Agreement: NR UE Parameter; Values for calibration and formal simulation. UE TX power in dBm; -40 to 23 dBm. NR UE Antenna gain (dBi); 0dBi. Height of UE antenna (m); 1.5m. NR UE Tx emission（dB） (standalone); Same with NR UE ACLR. NR UE Tx emission (dB) (inband); Same with NR UE IBE. NR UE ACS（dB）; 33dB. NR UE Noise Figure（dB）; 9dB. Antenna configuration; Omni direction antenna.

Proposal 8: For formal simulation, it should have same NR BS/UE parameter assumptions for calibration purpose

A-IoT reader/UE simulation parameters for formal simulation

In last RAN4 #116bis meeting, we have approved following simulation assumptions for calibration purpose.

A-IoT Reader RF parameters Issue 2-3-2-1: A-IoT Reader RF parameter assumptions Agreement A-IoT reader parameters; Values for calibration purpose. A-IoT reader total Tx power; Standalone; 43dBm. A-IoT reader receiver Noise Figure (dB); 5dB. A-IoT reader antenna gain (dBi); 15dBi including feeder loss. ACLR; 45dB. ACS; 46dB. Antenna configuration; Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am}. A-IoT Device RF parameters Issue 2-3-3-1: A-IoT Device RF parameters assumptions Agreement: A-IoT device parameters; Values for calibration purpose. Device 2b; Device C. A-IoT device effective antenna gain per Tx or Rx branch (dBi); 0; 0. A-IoT device output power（dBm）; -10; 5. A-IoT device noise figure (dB); 15; 13. ACLR(dB); 20; 25. ACS(dB); 30; 35. Guard band; 0PRB; 0PRB. Antenna configuration; Omni direction antenna; Omni direction antenna.

Proposal 9: For formal simulation, it should have same A-IoT reader/device parameter assumptions for calibration purpose except for some parameters(A-IoT reader total Tx power for in-band operation, ACLR. ACS)

Calibration results

This is simulation results for calibration. This table is just for information.

Based on above table, there is no big harm for most of cases and some cases is found with SINR degradation.

Observation 1: Based on calibration result, there is no harm for most of cases but some cases are found with SINR degradation. RAN4 need to further consider this cases in formal simulation.

- 2. Conclusion

In this contribution, we want to share some views on ambient IoT co-existence evaluioatn cases and simulation assumption and proposals are made as following:

Proposal 1: For NR system, use 5% throughput loss as performance metric as legacy.

Proposal 2: A-IoT system, use SINR degradation as performance metric.

Proposal 3: Adopt the following A-IoT SINR definition.

SINR definition for D2R:

-	SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference

-	SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

SINR definition for R2D:

-Signal power of device to the noise and interference within 10MHz is baseline assumption

-	Consider 180KHz noise and interference bandwidth after BB LPF as optional

Proposal 4: For ACLR, RAN4 use ACLR model with applied scaling factor

Proposal 5: For ACS, RAN4 should choose one in following options

- Option1: Step function in TR36.942

- Option2: Flat model

- Option3: Consider ACS of adjacent channel

- Option4: Effective ACS

Proposal 6: For formal simulation, it should have same deployment scenario for calibration purpose

Proposal 7: For formal simulation, it should have same deployment parameter assumptions for calibration purpose

Proposal 8: For formal simulation, it should have same NR BS/UE parameter assumptions for calibration purpose

Proposal 9: For formal simulation, it should have same A-IoT reader/device parameter assumptions for calibration purpose except for some parameters(A-IoT reader total Tx power for in-band operation, ACLR. ACS)

Observation 1: Based on calibration result, there is no big harm for most of cases but some cases are found with SINR degradation RAN4 need to further consider this cases in formal simulation.

- 3. Reference

RP-252964, “Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices” LG Electronics, RAN#109, Aug. 2025.

[2]  TR38.769, Study on solutions for ambient IoT (Internet of Things) V2.0.0