3GPP TSG-RAN WG4 Meeting #117	R4-2521526

# Dallas, USA, November 17th – 21st, 2025

Source:	Nokia

Title:	Discussion on Remaining Issues of Rel-19 Mobility Enhancements

Agenda item:	4.23.1

Document for:	Discussion

Introduction

In this contribution, we discuss the remaining issue related to the measurement time in the CLTM cell switch delay.

Discussion

CLTM cell switch delay

Tmeasure for SSB-based L1-RSRP measurement

In RAN4#116, RAN4 agreed to cover only the known target cell case for CSI-RS-based L1-RSRP measurements. As stated below, the current CLTM measurement time requirements when an SSB-based L1-RSRP measurement is used in the event include TPSS/SSS_sync_intra + TSSB_measurement_period_intra, which seems to always assume that the target cell is unknown.

##### 6.3.2.2.1 Measurement time The requirements in this clause apply under the following conditions: 1> The target cell is known stated in clause 6.3.1 for CSI-RS L1-RSRP triggered CLTM. The measurement delay is defined from the end of TEvent_DU until UE starts conditional cell switch execution to a target cell. 1> If SSB based L1-RSRP measurement is used in the event, Tmeasure shall be no larger than the maximum of L1-RSRP measurement periods of the serving cell and target cell. 2> serving cell measurement period TL1-RSRP_Measurement_Period_SSB is defined in clause 9.5.4.1 assuming TReport = 0, and 2> if the target cell is an intra-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TL1-RSRP_Measurement_Period_SSB_intra + TSSB_time_index_intra as defined in clause 9.2.5.1 and clause 9.14.5.1 assuming TReport = 0, otherwise. 2> if the target cell is an inter-frequency cell: 3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.3.4 and clause 9.15.5 or 9.15.6 assuming TReport = 0, if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell, or 3> target cell measurement period is TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TL1-RSRP_Measurement_Period_SSB_inter + TSSB_time_index_inter as defined clause 9.3.4 and in clause 9.15.5 or 9.15.6 assuming TReport = 0, otherwise.

The issue of whether to include known target cell case in SSB-based L1-triggered CLTM was discussed in RAN4#116bis. The following options were listed in the WF.

Issue 1-3-1: Whether to include unknown cell case on SSB triggered CLTM Recommended WF: Discuss the following: - For SSB based L1-RSRP measurement used with CLTM. Discuss and confirm which scenario is supported: - Option 1: Support only known case (specification change needed) - Option 2: Support both unknown and known case (specification change needed) - Option 3: Support only unknown case (no specification change).

The way Tmeasure requirements are currently defined results in an unnecessarily long triggering time for a known target cell. Hence, we believe that both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event. This distinguishment should be clarified in the requirements to be able to define useful L1-triggered CLTM test cases.

Both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.

Conclusion

The following proposal was made in this contribution:

Proposal 1: Both known and unknown target cell cases should be covered in the CLTM measurement time requirements (section 6.3.2.2.1) when an SSB-based L1-RSRP measurement is used in the event.