3GPP TSG RAN4 Meeting #117							R4-2521595

Dallas, Texas US, 17th – 21st November 2025

Source:	Nokia

Title:	UE and BS RF specification operation efficiency

Agenda item:	8.12.2.1

Document for:	Approval

Introduction

In the current study item [1], the following topics are listed under 6GR.

… - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, performance, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations …

In this contribution we discuss our considerations on the bold item listed above.

Discussion

Specification Format Modernization

RAN Plenary has agreed to study specification modernization [2]. The main study focuses on 3GPP procedures using DOCX files as the basis for standards and how they can be improved for specification drafting. The baseline is that e.g. MS Word’s WYSIWYG editing for DOCX files and integrated change-tracking have supported the Change Request (CR) workflow in 3GPP, scaling to thousands of CRs. However, the increasingly large specifications (in each RAN WG) have revealed technical limitations: latency and instability in large documents (e.g., repagination, sluggish search/navigation), fragmentation into multiple files that impedes global cross-referencing, inconsistent style definitions due to mixed editors, and high manual effort for CR integration.

With 6G Study Items starting/underway, the purpose of the study is to investigate whether it is possible to consider migration to a modern, structured authoring and version-control environment, potentially allowing text-based or semantic formats, to eliminate manual transcription errors, enable immediate post-plenary publication of new specifications. The progress of the SI is collected in the TR 21.802 [3].

As also expressed in the LS, [4] on FS_6GSpecs, RAN4 experts are encouraged to engage in this discussion.

RAN4 experts are encouraged to engage in the 6G specification modernization discussion under FS_6GSpecs.

At the RAN4#116bis meeting this was also captured in the WF [5]

## 1.2 On the conclusions of 6GSM SID Conclusions: - Postponed until March 2026 - Companies encouraged to be actively involved into SID discussion..

BS Specification Improvements

We identify the following general considerations to be take into account when drafting new BS specifications:

Specifications simplification:

  - Define requirements in such a way they do not need to be updated whenever new frequency band is defined

  - Requirements should be defined in generic way that no new channel bandwidths are needed to be introduced later

  - Optimize the specification structures to efficiently accommodate different BS types and classes. The specification clauses should be organized in logical order and paying attention to readability for audience outside of 3GPP organization.

Specification structure for BS RF:

  - As 6GR specification structure for BS RF will be decided in the work item phase, we would like to share our initial view on this issue. For NR approach with single BS core specification for conducted and radiated requirements (TS 38.104) and two separate specifications for BS conformance testing (TS 38.141-1 and TS 38.141-2) was established. Overall, it seems that split, which was done in Rel-15 for NR, worked fine up to now and can continue also for 6GR.

It is proposed to take into account above considerations for further work related to 6GR BS specifications structure and simplification.

UE Specification Improvements

How many UE RF specifications

For NR RAN4 has four specs for similar UE RF purposes depending on access type or frequency range. These are 38.101-1, -2, -3 and -5. These are for FR1, FR2, EN-DC and NTN purposes respectively. For 6GR RAN4 needs to discuss what kind of specifications are needed and what is the internal structure of those specifications.

For 6GR RAN4 needs to discuss what kind of specifications are needed and what is the internal structure of those specifications.

It is quite evident that -1 like specification is needed and it will have FR1 bands and also around 7 GHz bands. What needs to be discussed if NTN specification -5 is combined into the same specification when frequency ranges are same.

RAN4 needs to discuss if TN and NTN share same specification in 6GR for FR1 + around 7 GHz range

When thinking about -2 specification in NR i.e. FR2 a lot of effort has been put into developing the specification, but not much commercial benefits have been seen from that work. So the question that comes to mind is that should RAN4 port NR 38.101-2 pretty much as it is into 6GR and only update NR terminology to 6GR on band naming etc. Other aspects which surely needs attention are system parameter related.

RAN4 should consider maximal re-use of NR 38.101-2 when creating FR2 requirements for 6GR.

Then moving to -3 which has FR1-FR2 CA and EN-DC/NE-DC. There should not be a need for this specification in 6GR.

There should not be a need for 38.101-3 type of specification in 6GR.

How the UE RF specifications organized

NR and LTE used suffixes A, B, C etc to interleave different UE types/features into spec. Chronological order of spec is then such that each requirement is divided into sub-clauses covering all UE types/features under different suffixes. This way one can compare UE types/features impact to a certain requirement easily, but it also makes holistic view of requirements to one feature very difficult. NR 38.101-1 suffix-list in presented in Table 2.3.2.2-1

Table 2.3.2.2-1:NR 38.101-1 suffix-list

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |
| M | LP-WUS/WUR |

Alternative way of writing a specification compared to suffix-method should be discussed for 6GR.

Alternative way of writing a specification compared to suffix-method should be discussed for 6GR.

One obvious way is to contain all requirement for a UE types/features into same clause and use different clause numbers for other UE types/features. Like this

6 Tx requirements

6.1 eMMB UE single carrier

6.1.1 MOP

6.1.2 MPR

…..

6.2 eMMB UE carrier CA

6.2.1 CA MOP

6.2.2 CA MPR

…

7 Rx requirements

7.1 eMMB UE single carrier

6.1.1 REFSENS

6.1.2 ACS

…..

7.2 eMMB UE carrier CA

7.2.1 CA REFSENS

7.2.2 CA ACS

There may also be a linkage to RRM/demod specifications in 6GR that would be beneficial hence organization of specification structure should be done jointly with RRM/demod.

Organization of specification structure should be done jointly with UE RF/RRM/demod.

New tools for band combinations

RAN4 have since RAN#102 been investigating and developing a new tool/methodology for handling the specification of CA combinations in RAN4 with the latest RAN4 update presented in [6].

As a part of this work RAN4 has agreed that they intend to transition to fully use the database and remove the current DOCX table-based representation of supported band combinations in Rel-20.

RAN4 intends to transition to fully use the database and remove the current DOCX table-based representation of supported band combinations in Rel-20.

This means that some improvements are already implemented to the RAN4 NR specification before the release of the first 6G specification. It would be natural for RAN4 to adopt this improvement also for the 6G specifications.

RAN4 shall utilize the band combination database from the beginning of 6GR

As RAN4 in NR focus on inclusion of the CA configuration tables (Clause 5) to the database as a first step. Other aspects as the band combination specific requirements as delta values and MSD requirements is agreed to be considered at a later stage.

RAN4 is considering to also address band combination specific requirements such as delta values and MSD.

Since most of the band combination specific requirements are now defined by RAN4 following predefined methods as partly captured in the RAN4 PRD01 and the introductions of the LUT method for MSD it can be considered by RAN4 to completely automate the generation of these requirements.

RAN4 shall consider automated tools for generating supported band combinations and their related specific requirements as e.g. delta values and MSD in 6GR.

Conclusion

In this contribution we present our view on 6GR General RF and UE RF. The following observations have been made:

Observation 1: RAN4 experts are encouraged to engage in the 6G specification modernization discussion under FS_6GSpecs.

Proposal 1: It is proposed to take into account above considerations for further work related to 6GR BS specifications structure and simplification.

Proposal 2: For 6GR RAN4 needs to discuss what kind of specifications are needed and what is the internal structure of those specifications.

Proposal 3: RAN4 needs to discuss if TN and NTN share same specification in 6GR for FR1 + around 7 GHz range

Proposal 4: RAN4 should consider maximal re-use of NR 38.101-2 when creating FR2 requirements for 6GR.

Observation 2: There should not be a need for 38.101-3 type of specification in 6GR.

Proposal 5: Alternative way of writing a specification compared to suffix-method should be discussed for 6GR.

Observation 3: Organization of specification structure should be done jointly with UE RF/RRM/demod.

Observation 4: RAN4 intends to transition to fully use the database and remove the current DOCX table-based representation of supported band combinations in Rel-20.

Proposal 6: RAN4 shall utilize the band combination database from the beginning of 6GR

Observation 5: RAN4 is considering to also address band combination specific requirements such as delta values and MSD.

Proposal 7: RAN4 shall consider automated tools for generating supported band combinations and their related specific requirements as e.g. delta values and MSD in 6GR.

- References

RP-250858, New SID: Study on 6G Radio

SP-250802, Study on Modernization of Specification Format and Procedures for 6G, Nokia et. al.

TR 21.802, Study on modernization of specification format and procedures for 6G

R4-2513056, LS on Study on Modernization of Specification Format and Procedures for 6G, TSG SA

R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT

R4-2514216, Progress update and next steps for the Band Combination Database, Nokia, Ericsson