3GPP TSG-RAN WG4 Meeting #117                                   R4-2521668

# Dallas, TX, USA, November 17-21, 2025

Title:	Qualcomm views on 6G Spectrum

Source:	Qualcomm Incorporated

Agenda item:	8.5

Release:	Rel-20

SI: 	FS_6G_Radio

Document for:	Approval

# 1.	Introduction

In this paper we discuss and propose 6GR Spectrum issues in accordance with the chair guidance in [3]. We discuss and provide input to bands and band combinations work.

# 2.	Discussion

## 2.1	Band/Band combination definition

### 2.1.1 	Bands for 6GR

In previous meeting, prefixes “s” and “t” were discussed. Given there seems to be no ground-breaking difference between these two, we suggest to also consider “g” among the alternatives. “g” could refer to e.g. “Generation”.

Proposal 1: Decide if “s”, “t”, or “g” is used as prefix

There was some discussion on band numbering and numbering ranges in one of the Ad-Hoc meetings, but there was unfortunately not enough time to find common ground. Our view is that band numbering should be inherited from 5G. This has been the practice in 3GPP over different Generations. We would find it very difficult if the band numbering were changed, starting from very practical matters such as the way how engineers and leadership talk about that bands and so on.

Proposal 2: Use the same band numbering for re-farming 6G bands as in 5G

### 2.1.2 	Band combination definition and simplification for 6GR

Band combinations are one of the key topics for RAN4. With regards to that, word “simplification” is common in many contributions in association with band combinations. In our view there are two meanings for this “simplification” which must be clearly distinguished to avoid misunderstandings.

- Simplification of specification means ways how the specification can be made leaner and simpler to understand

- Simplification of the UE complexity means limiting the specified bands combinations

We are supporting of the former, i.e. simplification of specification to be leaner and simpler to understand. We are not supporting to limit the specified band combinations. Band grouping could be a good tool, pending further studies, to write the specifications in simpler way but band grouping cannot be used to limit the possibilities to aggregate spectrum. For instance, combinations n2/25-n66 and n1-n3 have been very common in UE’s for both 4G and 5G era, so it must be possible to aggregate those example combinations also in 6G.

RAN4 should study the process how band combinations specified in 5G are to be ported into 6G. There are several ways to do this process, ranging from starting 6G from clean slate into operator request per band combination into automatic porting of all combinations. It might not make sense to automatically copy them all, given it’s possible/likely that the requirements change as well and so on. On the other hand, while operator requests surely could work, they could cause issues if the process is not well defined.

One important aspect to account is also that currently there is no understanding how band combinations would be specified in detail, i.e. how would the band combination list look like. For instance, would the listing be like in 5G where we list all the configurations including suffixes for single CC, CA (“A”, “B”, C”, “2A”, 3A”, etc) for each band in band combinations, or could it be made in more coarse manner. This aspect should be discussed a bit later once the concept of CA in general is in better shape.

In all of this, RAN4 should be mindful and account the fact that in many cases only a subset of combinations are deployed.

Observation 1: Simplification in context of band combinations has two meanings which must be clearly distinguished:

- Simplification of specification means ways how the specification can be made leaner and simpler to understand

- Simplification of the UE complexity means limiting the specified bands combinations

Proposal 3: Simplification of specification and Simplification of UE complexity must be clearly distinguished as they have different meanings

Proposal 4: RAN4 should strive to find ways to simplify the band combination specification by making the specification leaner and easier to understand

Proposal 5: RAN4 to study if/how band grouping could be used in specifying requirements for band combinations

Proposal 6: Band grouping, if adopted, is not used to restrict which band combinations can be specified i.e. bands also within band group can be aggregated like in 5G

Proposal 7: RAN4 should study the process to port desired CA combinations specified in 5G into 6G. Process discussion should be only about the process itself and not on the criteria which kind of combinations can be ported.

Proposal 8: Postpone study on CA combination table format until the general concept of 6G CA is in better shape

# 3.	Conclusion

For 6GR Bands we made the following proposals:

Proposal 1: Decide if “s”, “t”, or “g” is used as prefix

Proposal 2: Use the same band numbering for re-farming 6G bands as in 5G

For Band Combination Definition and Simplification for 6GR we made the following observation and proposals:

Observation 1: Simplification in context of band combinations has two meanings which must be clearly distinguished:

- Simplification of specification means ways how the specification can be made leaner and simpler to understand

- Simplification of the UE complexity means limiting the specified bands combinations

Proposal 3: Simplification of specification and Simplification of UE complexity must be clearly distinguished as they have different meanings

Proposal 4: RAN4 should strive to find ways to simplify the band combination specification by making the specification leaner and easier to understand

Proposal 5: RAN4 to study if/how band grouping could be used in specifying requirements for band combinations

Proposal 6: Band grouping, if adopted, is not used to restrict which band combinations can be specified i.e. bands also within band group can be aggregated like in 5G

Proposal 7: RAN4 should study the process to port desired CA combinations specified in 5G into 6G. Process discussion should be only about the process itself and not on the criteria which kind of combinations can be ported.

Proposal 8: Postpone study on CA combination table format until the general concept of 6G CA is in better shape

# 4. 	References

[1]	RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025

[2]	TR 38.914, “Study on 6G Scenarios and Requirements”, v0.1.0, June 2025

[3]	R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple), 3GPP TSG-RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

[4]	R4-2513020, “Qualcomm Views on 6G general and UE RF”, Qualcomm, 3GPP TSG-RAN WG4 Meeting #116bis, Prague, Czech Republic, October 13-17, 2025