TSG-RAN Working Group 4 (Radio) meeting #117	R4-2521697

Dallas, US, November 17th – 21st 2025

Source:	Ericsson

Title:	On enhancement of BS co-location requirements

Agenda item:	7.2.3

Document for:	Approval

# Introduction

At RAN4#116 in Bangalore all findings from the work to investigate possible ways to improve co-location requirements as part of Rel-19 BS RF evolution Work Item (WI) were captured in a CR to TR 37.941. The CR [1] was formally approved at RAN#109 in Beijing.

At RAN#109 in Beijing a Work Item Description (WID) for a second phase of BS RF evolution was approved for Rel-20 in [2]. The new WI (NR_BS_RF_req_evo_Ph2-Core) have two separate objectives:

- Enhancement of co-location requirements

- SBFD BS to SBFD BS adjacent channel co-existence

At RAN4#116-bis in Prague the work related to NR BS RF requirement evolution phase 2 started, and the work plan was approved. The work plan for RAN4#117 is shown as below. In this contribution we present some further details on how to progress the work needed to improve the co-location requirements in BS RF specifications.

| Timeline | Work plan |
| --- | --- |
| RAN4#117 (0.5 TU) -Core part November, 2025 | Enhancement of co-location requirements - Continue the discussion on co-location scenario assumptions and the impacts on the OTA co-location requirements. - Continue the coupling loss analysis for missing cases from Rel-19 (e.g. band combination). - Continue the discussion on OTA co-location requirements improvement including: - CLRA concept (e.g. alternative testing antenna or other feasible testing method) - The possibility to define requirements using the existing OTA metrics SBFD BS to SBFD BS adjacent channel co-existence - Establish simulation assumptions and measurement assumptions - Initial simulation results for SBFD BS to SBFD BS adjacent channel coexistence |

# Discussion

  - Coupling loss analysis

To improve the co-location requirement concept and corresponding BS RF requirements RAN4 was tasked to study technical background and testability aspects in Rel-19. Based on Rel-19, it can be concluded that coupling loss evaluation for relevant situation is essential to improve co-location requirements. Therefore, in Rel-20 we need to continue to evaluate coupling loss for inter-band and intra band situations as indicated in Table 2.1-1.

Table 2.1-1: Requirement derivation assumptions

| Requirement | Coupling type | Frequency aspect | Description |
| --- | --- | --- | --- |
| Transmitter spurious emission | A-to-A | Inter-band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | A-to-SA | Inter-band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking is related to induvial receiver branches, A-to-SA is considered when the requirement level is derived. |
| Transmitter OFF power | A-to-A | Intra-band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on a neighbouring BS in the same network and not side-by-side at the same site. For this requirement the coupling loss between two neighbouring BS is different cells should be considered. The coupling loss for this requirement can be evaluated via network simulations using far-field assumptions. |
| Transmitter intermodulation | A-to-SA | Intra-band | This requirement is defined to guarantee that unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to individual transmitter branches, A-to-SA is considered when the requirement level is derived. |

As analysed in R4-2513942, the coupling loss evaluation from Rel-19 (as shown for intra-band coupling loss in Table 2.1-2 and for inter-band coupling loss in Table 2.1-3), which is captured in TR 37.941, clause 6.4.4.2, is not complete for all relevant situations. RAN4 need to further evaluate BS-to-BS coupling loss for different coupling situations relevant for considered requirements.

Table 2.1-2: Intra-band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| Vertical separation | SA-to-SA | M: 54 (R4-2504358) | M: 53 (R4-2504358) | M: 56 (R4-2504358) | FFS |
|  | A-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-A | FFS | FFS | FFS | FFS |
| Horizontal separation | SA-to-SA | M: 31 (R4-2504358) | M: 34 (R4-2504358) | M: 46 (R4-2504358) | FFS |
|  | A-to-SA | FFS | M: 35 (R4-2507633) S: 55±15 (R4-2506307) | M: 51 (R4-2507445) M: 69 (R4-2507511) | M: 55 (R4-2507633) S: 70±15 (R4-2506307) |
|  | A-to-A | FFS | FFS | FFS | S: 77±15 (R4-2506307) |

Table 2.1-3: Inter-band coupling loss summary

| Co-location scenario | Coupling loss type | Coupling loss (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 2.6 GHz to 3.5 GHz | 3.5 GHz to 4.9 GHz | 4.9 GHz to 7 GHz | 3.5 GHz to 7 GHz |
| Vertical separation | SA-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-A | FFS | FFS | FFS | FFS |
| Horizontal separation | SA-to-SA | FFS | FFS | FFS | FFS |
|  | A-to-SA | FFS | FFS | FFS | S: 57±10 (R4-2506307) |
|  | A-to-A | FFS | FFS | FFS | S: 54±10 (R4-2506307) |

Moreover, as expected, vertical separation results in higher coupling loss, hence horizontal separation is seen as the worst-case co-location scenario.

Regarding coupling loss type of interest for co-location requirement level derivation, it can be noticed that SA-to-SA is easy to measure, but not really relevant for requirement derivation. The cases to focus on is marked in bold green text in tables above.

Compared with vertical separation, horizontal separation is seen as the worst-case co-location scenario.

The evaluation of inter-band and intra-band coupling loss in Rel-20 should be focused on A-to-SA and A-to-A.

Regarding Rel-20 coupling loss analysis, RAN4 focuses on the worst-case co-location scenario, i.e., horizontal separation, and A-to-SA (Array-to-Subarray) and A-to-A (Array-to-Array).

For tower sharing, transmitter spurious emission and receiver out-of-band blocking are the two essential co-location requirements. Work to improve those should be prioritized, hence studies to evaluate inter-band coupling loss cases should get priority before intra-band cases.

For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised, i.e., the inter-band coupling loss evaluation should be prioritised.

To evaluate the inter-band coupling loss and to be able to determine relevant coupling loss values for requirement derivations, a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary. Even though it is possible to measure the SA-to-SA coupling loss, the measurement presented so far did not capture the beamforming aspects. If the beamforming aspects are to be added afterwards in simulation, the phase information will be needed to properly model the coupling loss in the near-field region. Hence, in our view, a near-field simulation campaign is necessary based on the near-field simulation methodology as shown in Section 2.2, and modelling assumptions needs to be agreed, e.g., deployment parameters, BS array configuration and out-of-carrier emission model based on correlation factor roll-off. The simulation assumption in TR 38.922, clause 7.3.2 in [3] can be used as baseline. The array antenna model can be used to produce the average radiation pattern outside the carrier based on a correlation parameter.  In addition, a model to generate instantaneous statistically distributed sample patterns is also needed. In a companion contribution we have a CR to TR 38.922 to add missing information on instantaneous patterns.

Based on EM-modelling (for defined co-location scenario) the element-to-element coupling matrix (including victim BS and aggressor BS elements) can be calculated as described in [4]. For the EM simulation RAN4 need to consider an approach using a simplified element antenna model that can be shared between companies. Here, simplified means to remove element details that have a minor impact on coupling but a large impact on simulation time and compute resources, and to hide specific implementation details.

Moreover, the choice of antenna element model also needs to capture out-of-band properties in a relevant way. In particular the antenna matching can be arbitrary and quite different for different antenna elements. Hence, it is probably a good idea to compensate for antenna mismatch in the simulated data and then use agreed values for antenna mismatch in a post-processing step.

Antenna mismatch for out-of-band response needs to be handled adequately.

To be able to determine relevant coupling loss values for requirement derivations a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary.

Measurement of the SA-to-SA coupling loss presented so far did not capture the beam-forming aspects. If the beamforming aspects are to be added afterwards in simulation, the phase information will be needed to properly model the coupling loss in the near-field region.

Simulation assumptions (ISD, Number of UEs, BS height, BS array configuration, band combinations, etc.) need to be defined to conduct the simulation campaign correctly and ensure RAN4 to agree on reasonable coupling loss values when concluding the Rel-20 co-location work.

Rel-20 co-location work should focus on the Urban Macro (UMa) scenario, as the high power and line-of-sight conditions in UMa make it the most challenging/worst-case deployment scenario.

The coupling loss between co-located AAS BSs depends strongly on the frequency, because wavelength, antenna aperture, element spacing and panel size all change with frequency. For Rel-20 co-location work, a single FR1-wide assumption would hide important trends: a 3.5 GHz panel and a 7 GHz panel behave very differently. To arrive at a reasonable number of combinations the FR1 range would need to be subdivided in sub-ranges. Each sub-range groups frequency bands with similar antenna sizes, array geometries and typical RF filtering, etc., which allows using a small set of representative coupling-loss values for each combination of sub-ranges, while still capturing how coupling varies across realistic situations. The cases to focus on (marked in bold green text in Table 2.1-2 and Table 2.1-3) is further merged into one table (as shown in Table 2.1-4).

Table 2.1-4: Coupling loss

| Band regions | 2.6 GHz | 3.5 GHz | 4.9 GHz | 7 GHz |
| --- | --- | --- | --- | --- |
| 2.6 GHz | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB |
| 3.5 GHz | -- | A-to-SA: M: 35 (R4-2507633) S: 55±15 (R4-2506307) A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB | A-to-SA: S: 57±10 (R4-2506307) A-to-A: S: 54±10 (R4-2506307) |
| 4.9 GHz | -- | -- | A-to-SA: M: 51 (R4-2507445) M: 69 (R4-2507511) A-to-A: FFS dB | A-to-SA: FFS dB A-to-A: FFS dB |
| 7 GHz | -- | -- | -- | A-to-SA: M: 55 (R4-2507633) S: 70±15 (R4-2506307) A-to-A: S: 77±15 (R4-2506307) |

The coupling loss assumptions should be captured for all band combinations picked from a division in sub-ranges.

Establish simulation methodology required for side-by-side co-location scenario considering near-field effects.

Based on the experience from previous simulation in Rel-19 it can be concluded that the coupling loss associated with 100% on the CDF also referred to as Minimum Coupling Loss (MCL) is very sensitive to the number of UEs considered. A more stable value can be found in the range 95% to 99% on the CDF.

For non-AAS BS the coupling loss was evaluated as MCL derived based on a link budget, since the BS radiation patterns was fixed. With AAS BS it would be more reasonable to use a point on the CDF in the range 95% to 99% due to the dynamic behaviour of the AAS BS beamforming due to UE location and channel condition.

When considering AAS BS coupling loss characteristics for deriving requirement levels, choosing the minimum coupling loss represents an outlier case which would risk driving design complexity and cost. A more suitable percentile would be in the range 95 to 99%.

  - Near-Field simulation methodology

Performing a co-location coupling analysis of two side-by-side large arrays is a formidable task that easily leads to memory overflow and long simulation times, especially if a complete coupling model is used in an Electromagnetic (EM) simulation tool. To overcome this, simplifications have been used such as using a simplified EM model of the antenna element and analytical circuit theory models of subarray networks. The overall idea is to choose these simplifications in a conservative way, i.e., to minimize the coupling loss. For example, there are no isolating structures on the sides of the arrays and no losses in the subarray models. Hence, in a real case the coupling loss should be larger compared to these simulations.

Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 2.2.4. Finally, array ports are obtained by applying beamforming weights to the subarray ports, see Section 2.2.6.

For the Near-Field simulation methodology presented in this contribution, we have split the simulations in two parts:

- In the first stage, the Element-to-Element coupling is represented by a scattering matrix simulated in an EM simulator, see Section 2.2.1. The victim and aggressor array antennas are placed according to decided co-location scenario side-by-side separated 0.1 m, as illustrated in Figure 2.2.2-1. The scattering matrix (S-matrix) includes all elements (both polarizations) of both arrays and contains element-to-element coupling loss and associated phase information. If needed, the S-matrix is calculated for several frequencies within the frequency range that is considered.

- In the second stage sub-arrays and array ports are created. Subarray beamforming networks are represented by S-matrices modelled analytically using circuit theory, see Section 2.2.4, and array ports are obtained by applying beamforming weights to the subarray ports (and positions), see Section 2.2.6. For this stage a general-purpose linear algebra computation tool, e.g., MathWorks MATLAB or Python can be used.

A block diagram of the simulation methodology is shown in Figure 2.2-1.

Figure 2.2-1: Overview of the simulation methodology

    - S-matrix simulation

The antenna element scattering matrix is simulated using a commercial 3D EM field simulation software, in this case CST Studio Suite. The S-matrix contains data for all elements and polarizations in both arrays.

As an example, in the case of 3.5 GHz arrays, each of the arrays has 192 ports (12 rows x 8 columns x 2 polarization). This results in a 384x384 scattering matrix for each frequency point in the defined frequency range. For more details on geometry and element model, see below.

    - Geometry

For simulating the coupling between two arrays, two antenna arrays are placed next to each other (see Figure 2.2.2-1).

Figure 2.2.2-1: Two antenna arrays in an edge-to-edge co-location scenario

Each array comprises 12x8 dual-polarized radiating elements and a metallic reflector. The vertical element separation is 0.7λ and the horizontal element separation 0.5λ at specific design frequency (e.g., 3.5 GHz).

The distance between the two antennas (edge-to-edge) has been increased from 10 cm to 16 cm to take account of the mechanical support structures, which are present in a real product, and of space for the enclosure, which is also not included in the simulation.

    - Antenna element model

A simplified antenna element was used to get reasonable simulation time for the Element-to-Element S-matrix. Mechanical details, not relevant for the EM simulation was removed to speed up the simulation stage. These removed details include, e.g., as corners, mechanical support structures, logotypes, etc. As example of such the antenna element model used for simulation is shown in Figure 2.2.3-1. It consists of a thin crossed half-wavelength dipole which is fed in the centre by a coaxial port for each polarization (+/- 45°) and a metallic reflector at quarter-wave distance below the dipoles. There is no connection between the reflector and the dipole.

Figure 2.2.3-1: Perspective, top and side view of a simplified antenna element model

    - Sub-array modelling

The antenna elements are grouped in 3-element single column sub-arrays, as shown in Figure 2.2.4-1. A sub-array RDN, having 3 element ports and one feed port, is modelled with lossless ideal transmission lines and T-splitters, see Figure 2.2.5-1. To achieve equal power on the three elements, one of the T-splitters, S1, has an uneven power split ratio of 2:1, and the other, S2, has an even power split. Impedances of the quarter wave transformers on the output ports are chosen so that the desired power split ratio and impedance matching is achieved. The lengths of the lines L12, L13, L22 and L23 are chosen to achieve the desired fixed subarray tilt and long enough for the physical element spacing. The propagation constant of the transmission lines is chosen to equal that of free space for simplicity.

# Figure 2.2.4-1: Array and subarray layout

    - Sub-array ports S-matrix (cascading)

The S-matrices of the components in the antenna RF model are combined to an S-matrix describing the entire antenna including RDNs using well known circuit simulation theory, described, e.g., in [5], as visualised in Figure 2.2.5-1.

Z0, L13

Z0, L12

Z0, L23

Z0, L22

S1

S2

Figure 2.1.5-1: A 1:3 RDN is simulated by standard RF circuit techniques using S-matrix cascading

    - Deployment and beamforming weights

In the simulation it is assumed that two AAS base stations are co-located at the same site according to the enclosure edge-to-edge scenario defined in Rel-15 for BS type 1-O co-location requirements in TS 38.104, Clause 4.9. In this scenario it is assumed that both the aggressor AAS base station and the victim AAS base station are mounted side-by-side separated 0.1 m. The aggressor network and the victim network are configured in a hexagonal layout with 0% grid shift as visualised in Figure 2.2.6-1.

# Figure 2.2.6-1: Network layout

The victim and aggressor networks are co-sited. Narrow beams (similar to layer 1 codebook generated beams) are formed from the aggressor AAS to the UEs of the aggressor network and the corresponding power is received on the subarray ports of the co-sited AAS of the victim network. Only one cell is considered in the analysis.

Details related to deployment and BS assumptions is listed in Table 2.2.6-1, and Table 2.2.6-2.

Table 2.2.6-1: Network assumptions

| Parameter | Assumption |
| --- | --- |
| Network layout | Hexagon |
| ISD | 500 m |
| Network grid shift | 0 % |
| BS height | 25 m |
| UE height | 1.5 m |
| Number of UEs per sector | 1500 |
| Minimum 2D distance between BS and UE | 35 m |

Table 2.2.6-2: BS assumptions

| Parameter | 3500 MHz | 7000 MHz |
| --- | --- | --- |
| Sub-array configuration | 3x1 | 3x1 |
| Array configuration | 4x8 | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees | 3 degrees |
| Mechanical down-tilt | 3 degrees | 3 degrees |
| Vertical element separation | 0.7l | 0.7l |
| Horizontal element separation | 0.5l | 0.5l |

For the evaluation in this contribution the physical size of the AAS BS antennas considered for 3500 MHz and 7000 MHz is equal. Considering other band combinations, such as lower bands, it is expected that the physical size will differ between aggressor and victim.

In the case of using randomly positioned UEs and A-to-A coupling loss, 1500 randomly distributed UEs per system are used. Then, at each time instance a pair of users are chosen, which are used to generate one sample of coupling loss, i.e., there will be a total of 1500 coupling loss samples. In Figure 2.2.6-2, an example of UE locations is visualised for the aggressor network and victim network.

In the case of A-to-SA coupling, only one system uses the above beamforming, and coupling is calculated between the array and the sub-array ports.

Figure 2.2.6-2: UE distribution

For an AAS BS array antenna operating outside the design frequency it is expected that the directivity will drop due to decorrelation effects due to out-of-band RF filter characteristics, transmission effects in the feeder network part of RDN and unwanted emission signal correlation.

For out-of-band beam weights a correlation model is used, corresponding to adding white noise to the antenna element weights. Here, 100% correlation (r equal to 1) corresponds to clean beam weights with no noise and 0% correlation (r equal to 0) corresponds to fully noisy beam weights. The effect of the beam pattern is that the directivity will decrease when the correlation decreases. In turn this leads to relatively more power being radiated sideways, which seems to minimize the coupling loss. Hence, 0% correlation could be used to derive conservative co-location requirements. The actual value of the correlation is unknown and is dependent on the specific implementation. Specifically, the digital predistortion will influence the out-of-band beam weights significantly.

Assuming a polynomial model, the beam weights for even harmonic frequencies equals the square of the beam weights of the carrier frequency, wc, while for odd harmonics the weights are equal to the beam weights of the carrier.

wnom=wcfor odd harmonicswc2/wc for even harmonics

where wnom, are the nominal beam weights.

All out-of-band beam weights, including the harmonics, are modelled as partly correlated with the correlation as a parameter. Partly correlated beam weights are modelled as:

w=ρ wnom+1-ρ wuncorr

where wuncorr are random weights:

wuncorr=x+jy2N

Here, x and y are normally distributed random variables with expected value 0 and standard deviation 1, and N is the number of subarray ports.

  - CLTA improvement

The CLTA concept provides a practical way of testing co-location performance. However, there are areas where the concept needs to be improved to ensure reliable, repeatable test results with low measurement uncertainty. Furthermore, the current concept does not work above 3 GHz which has been discussed in previous papers [4, 5, 6, 7, 8, 9, 10]. In addition, these improvements should also allow for alternative test antennas. A couple of these improvements are discussed below. Note that the following discussion does not exclude the use of a CLTA antenna.

    - Link CLTA test procedure to a physical OTA parameter

The current CLTA specification allows for arbitrary losses. There is no described procedure to quantify the part of the coupling loss due to the CLTA. If the test would be to measure a certain power density or field strength level the losses can be compensated for in the test setup by using a calibration procedure such as antenna substitution.

The CLTA specification lacks a link to a physical OTA parameter.

Work on a calibration method that can be used to remove CLTA variations from the test result.

The specification is based on chassis separation which opens up for arbitrary separations of the radiating elements.

If instead a physical field parameter is used, this ambiguity would be resolved.

    - Allow for rotation of the CLTA

The CLTA utilizes sidelobes or main lobe flanks which has disadvantages from a test perspective. The signal-to-noise ratio of the measured signal can be a problem. This has a direct impact on the needed high output power of the interferer source in the TxIM test case. Furthermore, positioning and alignment can be more challenging compared to measuring with the main lobe of the test antenna. One way to solve these issues would be to rotate the CLTA to point its main beam towards the EUT and adjust the required level of the measured or injected signal.

The current CLTA orientation leads to unnecessary low SNR and high MU.

Allow for CLTA rotation and study the needed adjustments in required signal levels.

    - Improve CLTA specification

Co-location is a near field, sideways, out-of-band phenomenon, whereas the CLTA is specified in terms of far field, main beam, and in-band parameters. Specifying physical dimensions together with beamwidths can be contradicting as the beamwidths are strongly linked to antenna dimensions. Moreover, the polarization matching makes little sense in sideways and out-of-band situations. Specifying height, width and requiring measurements in two polarizations should be enough.

The current CLTA specification includes unnecessary requirements on beam width and polarization match.

Remove, or make optional, beamwidths from CLTA specification.

Remove polarization matching from the test and add description to measure in two orthogonal polarizations and sum.

    - Improve CLTA to more modern scenario

The current CLTA concepts reflects AAS to non-AAS single column passive antenna co-location. This was a reasonable assumption in Rel. 15. Now RAN4 should reconsider this assumption.

  - Co-location scenario and reactive near field region

In the 10 cm co-location case both antennas are at risk of intersecting each other’s reactive nearfield regions. The depth of the reactive nearfield region varies with antenna design and the exact definition of “reactive near field” but typical values are 0.5λ to 2λ. In OTA testing it is well known that if the test antenna is put in the reactive near field region of the EUT there is a risk that the test antenna changes the performance of the EUT, as observed in TR 37.941 Clause 6.4.4 part of Rel-.19 outcome. Hence, there is a potential risk with using the CLTA or any test antenna too close to the EUT. Moreover, to ensure unperturbed operation of AASs avoidance of the reactive near field region should typically be covered in installation guidelines. Therefore, instead of having 10 cm as the worst-case scenario, pick the maximum of 2λ, FFS, and 10 cm. This would ensure repeatable test results both with the current CLTA approach and any other test setup.

10 cm separation colocation scenario may lead to inaccurate test results and disturbed installed performance.

Use the maximum of 10 cm and 2λ (FFS) as smallest co-location scenario.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Compared with vertical separation, horizontal separation is seen as the worst-case co-location scenario.

Observation 2	The evaluation of inter-band and intra-band coupling loss in Rel-20 should be focused on A-to-SA and A-to-A.

Observation 3	Antenna mismatch for out-of-band response needs to be handled adequately.

Observation 4	To be able to determine relevant coupling loss values for requirement derivations a simulation campaign including near-field modelling of victim BS and aggressor BS is necessary.

Observation 5	Measurement of the SA-to-SA coupling loss presented so far did not capture the beam-forming aspects. If the beamforming aspects are to be added afterwards in simulation, the phase information will be needed to properly model the coupling loss in the near-field region.

Observation 6	Simulation assumptions (ISD, Number of UEs, BS height, BS array configuration, band combinations, etc.) need to be defined to conduct the simulation campaign correctly and ensure RAN4 to agree on reasonable coupling loss values when concluding the Rel-20 co-location work.

Observation 7	Rel-20 co-location work should focus on the Urban Macro (UMa) scenario, as the high power and line-of-sight conditions in UMa make it the most challenging/worst-case deployment scenario.

Observation 8	The coupling loss assumptions should be captured for all band combinations picked from a division in sub-ranges.

Observation 9	When considering AAS BS coupling loss characteristics for deriving requirement levels, choosing the minimum coupling loss represents an outlier case which would risk driving design complexity and cost. A more suitable percentile would be in the range 95 to 99%.

Observation 10	The CLTA specification lacks a link to a physical OTA parameter.

Observation 11	The current CLTA orientation leads to unnecessary low SNR and high MU.

Observation 12	The current CLTA specification includes unnecessary requirements on beam width and polarization match.

Observation 13	10 cm separation colocation scenario may lead to inaccurate test results and disturbed installed performance.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Regarding Rel-20 coupling loss analysis, RAN4 focuses on the worst-case co-location scenario, i.e., horizontal separation, and A-to-SA (Array-to-Subarray) and A-to-A (Array-to-Array).

Proposal 2	For tower sharing, transmitter spurious emission and receiver out-of-band blocking requirements are essential and should be prioritised, i.e., the inter-band coupling loss evaluation should be prioritised.

Proposal 3	Establish simulation methodology required for side-by-side co-location scenario considering near-field effects.

Proposal 4	Work on a calibration method that can be used to remove CLTA variations from the test result.

Proposal 5	Allow for CLTA rotation and study the needed adjustments in required signal levels.

Proposal 6	Remove, or make optional, beamwidths from CLTA specification.

Proposal 7	Remove polarization matching from the test and add description to measure in two orthogonal polarizations and sum.

Proposal 8	Use the maximum of 10 cm and 2λ (FFS) as smallest co-location scenario.

# References

[1]	RP-252586, “CR to TR 37.941: Rel-19 CLTA-related agreements”, Huawei, HiSilicon, ZTE, Nokia, Ericsson

[2]	RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, Moderator (Huawei)

[3]	TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”, 3GPP

[4]	R4-2506307, “Overview of BS-to-BS coupling loss simulation results”, Ericsson

[5]	R4-2412912, “On the topic of BS RF evolution related to co-location requirements”, Ericsson

[6]	R4-2415971, “On the topic of aspects related to BS-to-BS co-location”, Ericsson

[7]	R4-2408086, “Evolution of BS co-location requirements”, Ericsson

[8]	R4-2510233, “On alternatives to current CLTA concept”, Ericsson

[9]	R4-2510229, “Discussion on BS RF evolution general aspects”, Ericsson

[10]	R4-2510227, “On improvements of co-location requirements”, Ericsson